| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0086-v1 |
| --- | --- |

| Title: | EE2-2.9: EIP with bias and clipping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Pekka Astola | Tel: Email: | - first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports results of EE2-2.9 on EIP (extrapolation-based intra prediction) with bias and clipping. It is proposed to substitute one of the sample inputs of the EIP filters with a bias term (Test 2.9a) and to clip the filter output to the value range of the local reference samples (Test 2.9b). Test 2.9c reports the results of enabling both bias and clipping. It is asserted that inclusion of these aspects has positive impact on the EIP performance.

It is reported the proposed modifications achieve the following coding gains for AI configuration when implemented in ECM 12.0 {for Y, U, V}:

AI - {-0.02%, 0.00%, 0.00%} EE2-2.9a with a bias term
AI - {-0.01%, 0.02%, 0.04%} EE2-2.9b with clipping
AI - {-0.04%, -0.02%, 0.01%} EE2-2.9c with both bias term and clipping

The reported impact for RA is as follows for the three tests {for Y, U, V}:

RA - {-0.01%, 0.00%, -0.08%} EE2-2.9a with a bias term
RA - {-0.01%, -0.02%, -0.03%} EE2-2.9b with clipping
RA - {-0.02%, -0.02%, 0.01%} EE2-2.9c with both bias term and clipping

It is further asserted the ECM runtimes and peak memory consumption are virtually unchanged as the filter size remains unchanged and there is already clipping to the full sample range in the original EIP.

# Introduction

The EIP (extrapolation-based intra prediction) selects one of the filter shapes illustrated in Figure 1 to perform intra prediction for a block.

Figure 1: EIP filter shapes as depicted in JVET-AG0058

The filter selected for use is generated using a template area around the prediction block and used for sample prediction in a recursive fashion as shown in Figure 2.

Figure 2: Recursive filtering using earlier prediction samples as input as illustrated in JVET-AG0058

The filters used by EIP are of form:

predx,y=i=014(ci×tx-offsetXi, y-offsetYi)

Where the spatial input samples are multiplied by the filter coefficients derived from block’s neighborhood. This form may have challenges when it comes to representing certain kinds of relatively usual textures. For example, if there is a constant gradient in the reference area with sample values on average increasing or decreasing in the direction of the output sample, the prediction could take forms like:

predx,y=g∙averagetx-offsetXi, y-offsetYi,  i=0…14; or

predx,y=g∙tx-1, y-1

However, when applying this kind of a filter to the prediction area the gradient will no longer be constant, but the filter will amplify the prediction samples by a factor of g every step in the recursion. Thus, instead of a constantly increasing surface we get an exponential one with gain of gN after N steps.

# Proposed modifications

## EE2-2.9a (EIP with bias)

It is proposed to include a bias term to the EIP filter models to make the recursive filter operation more likely to represent textures in the prediction block’s neighborhood. This is done by substituting the last spatial input to the filter (the furthest away sample from the output sample location) by a bias term given as 2B-1, where B is the bitdepth of the channel. That is, the filters are modified to the following form:

predx,y=i=013(ci×tx-offsetXi, y-offsetYi)+c14×2B-1

## EE2-2.9b (EIP with clipping)

Is proposed to clip the predicted sample values to the range of the reference samples instead of the full sample value range. Reference sample area used for determining the range is the same that is used when generating the filter coefficients.

## EE2-2.9c (EIP with bias and clipping)

Test 2.9c enables both Test 2.9a and 2.9b at the same time.

# Coding efficiency results

Summary of the coding efficiency impact of the three tests is provided in the tables below. Full results are available as separate Excel spreadsheets.

## EE2-2.9a (EIP with bias)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03 % | -0.02 % | 0.01 % | 99.8% | 98.6% | 100.0% | 100.0% |
| Class A2 | -0.05 % | -0.03 % | 0.01 % | 99.4% | 99.2% | 100.0% | 100.0% |
| Class B | -0.04 % | 0.03 % | 0.09 % | 99.8% | 99.4% | 97.9% | 100.0% |
| Class C | -0.02 % | -0.01 % | -0.04 % | 100.0% | 101.2% | 100.0% | 100.0% |
| Class E | 0.02 % | -0.01 % | -0.08 % | 100.1% | 100.6% | 100.0% | 100.0% |
| Overall | -0.02 % | 0.00 % | 0.00 % | 99.8% | 99.8% | 99.4% | 100.0% |
| Class D | -0.03 % | 0.20 % | -0.09 % | 99.8% | 102.2% | 100.0% | 100.0% |
| Class F | 0.02 % | 0.13 % | -0.04 % | 100.0% | 99.9% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05 % | -0.03 % | -0.07 % | 99.8% | 99.7% | 100.0% | 100.0% |
| Class A2 | 0.03 % | -0.01 % | -0.03 % | 99.6% | 99.7% | 100.3% | 100.6% |
| Class B | -0.03 % | -0.04 % | -0.12 % | 100.5% | 99.9% | 99.9% | 99.9% |
| Class C | 0.02 % | 0.07 % | -0.06 % | 100.1% | 100.5% | 100.8% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01 % | 0.00 % | -0.08 % | 100.1% | 100.0% | 100.3% | 100.1% |
| Class D | 0.00 % | -0.08 % | 0.23 % | 100.6% | 99.8% | 99.8% | 100.0% |
| Class F | 0.01 % | -0.02 % | 0.00 % | 100.0% | 100.5% | 98.9% | 100.4% |

## EE2-2.9b (EIP with clipping)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00 % | 0.01 % | 0.07 % | 100.0% | 99.6% | 100.0% | 100.0% |
| Class A2 | -0.03 % | 0.04 % | 0.03 % | 99.7% | 99.5% | 100.0% | 100.1% |
| Class B | -0.02 % | 0.07 % | 0.07 % | 100.3% | 99.1% | 100.0% | 100.0% |
| Class C | -0.01 % | -0.01 % | 0.15 % | 100.1% | 98.2% | 99.7% | 100.0% |
| Class E | -0.02 % | -0.02 % | -0.15 % | 100.2% | 99.5% | 100.0% | 100.0% |
| Overall | -0.01 % | 0.02 % | 0.04 % | 100.1% | 99.1% | 99.9% | 100.0% |
| Class D | -0.03 % | 0.17 % | -0.06 % | 100.5% | 97.4% | 100.0% | 100.0% |
| Class F | 0.04 % | 0.23 % | 0.10 % | 100.5% | 100.2% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03 % | -0.05 % | -0.11 % | 99.6% | 99.3% | 99.8% | 100.0% |
| Class A2 | 0.03 % | 0.09 % | -0.02 % | 99.3% | 99.4% | 100.0% | 100.5% |
| Class B | -0.01 % | -0.12 % | -0.06 % | 100.3% | 98.8% | 99.6% | 100.0% |
| Class C | -0.02 % | 0.06 % | 0.04 % | 100.4% | 99.8% | 101.2% | 99.7% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01 % | -0.02 % | -0.03 % | 100.0% | 99.3% | 100.1% | 100.0% |
| Class D | -0.02 % | 0.00 % | -0.08 % | 100.6% | 99.5% | 99.6% | 100.0% |
| Class F | -0.03 % | 0.01 % | 0.11 % | 100.9% | 99.3% | 99.2% | 100.4% |

## EE2-2.9c (EIP with bias and clipping)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04 % | 0.01 % | -0.05 % | 99.8% | 98.8% | 100.1% | 100.0% |
| Class A2 | -0.07 % | 0.06 % | -0.02 % | 99.4% | 98.6% | 100.6% | 100.1% |
| Class B | -0.05 % | -0.01 % | 0.02 % | 99.9% | 99.2% | 99.7% | 100.0% |
| Class C | -0.02 % | -0.06 % | 0.05 % | 100.4% | 101.6% | 100.0% | 100.0% |
| Class E | -0.02 % | -0.06 % | 0.04 % | 99.9% | 101.3% | 100.0% | 100.0% |
| Overall | -0.04 % | -0.02 % | 0.01 % | 99.9% | 99.9% | 100.0% | 100.0% |
| Class D | -0.02 % | 0.00 % | -0.10 % | 100.0% | 102.1% | 100.2% | 100.0% |
| Class F | 0.00 % | 0.06 % | -0.20 % | 100.1% | 100.6% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04 % | -0.13 % | -0.09 % | 100.0% | 99.7% | 100.1% | 99.9% |
| Class A2 | 0.00 % | 0.09 % | 0.16 % | 99.8% | 99.4% | 100.0% | 100.3% |
| Class B | -0.03 % | 0.08 % | -0.06 % | 100.0% | 99.3% | 100.0% | 100.0% |
| Class C | 0.00 % | -0.13 % | 0.05 % | 100.4% | 100.1% | 99.6% | 99.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02 % | -0.02 % | 0.01 % | 100.1% | 99.6% | 99.9% | 99.8% |
| Class D | 0.01 % | -0.18 % | 0.33 % | 100.1% | 99.4% | 99.8% | 100.0% |
| Class F | 0.00 % | -0.04 % | -0.03 % | 100.2% | 100.0% | 99.3% | 100.4% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).