| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0089-v2 |
| --- | --- |

| Title: | AhG12 On entropy models training |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, F. Lo Bianco C. Salmon-Legagneur | Tel: Email: | franck.galpin@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

In test EE2-5.1 of JVET-AG0196, a full retraining of the CABAC models parameters of ECM-1..0 was proposed and accepted for integration in ECM-12.0. The training of the parameters was done using all CTC sequences (including F and TGM) for all configurations (RA/AI/LDB/LDP) and 4 QPs (22/27/32/37). In this contribution, we show the difference in performance of using a difference training set to compute the parameters of the models.

# Introduction

In ECM-11.0, CABAC has the following parameters: probability states, short and long window sizes, weights, and offsets for short and long windows. In total, 3*3+2=11 parameters are required per context for the 3 slices type B, P and I.

The training of the parameters is done using all CTC sequences (including F and TGM) for all configurations (RA/AI/LDB/LDP) and 4 QPs (22/27/32/37). In total 30 sequences for a total of 420 bitstreams are used to train the model. The results of the training can be found in JVET-AG0196 test 5.1.

In this contribution we show the impact of using a different dataset, namely the BVI dataset. The dataset specification is defined in JVET-AF2016 and details can be found at jvet-ahg-nnvc / nnvc-ctc · GitLab (fraunhofer.de) . The dataset is the one used by AhG11 and is composed of 800 sequences of 64+1 frames for a total of 12800 bitstreams.

# Simulations

## CTC results (from JVET-AG0196)

The results are the ones of JVET-AG0196 test 5.1.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | 0.04% | 0.07% | 99.1% | 97.2% | 99.9% | 99.9% |
| Class A2 | 0.01% | 0.03% | -0.04% | 98.9% | 98.7% | 99.9% | 100.2% |
| Class B | -0.02% | 0.10% | -0.01% | 98.5% | 98.2% | 98.2% | 99.9% |
| Class C | -0.06% | 0.01% | 0.12% | 98.1% | 98.2% | 99.5% | 100.0% |
| Class E | -0.08% | 0.05% | -0.12% | 97.8% | 98.6% | 98.6% | 100.0% |
| Overall | -0.04% | 0.05% | 0.01% | 98.4% | 98.2% | 99.2% | 100.0% |
| Class D | -0.11% | 0.05% | 0.16% | 98.7% | 97.6% | 99.6% | 100.0% |
| Class F | -0.07% | -0.21% | -0.14% | 101.3% | 102.0% | 99.8% | 100.2% |
| Class TGM | -0.16% | -0.13% | -0.23% | 99.1% | 105.8% | 99.9% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.13% | 0.45% | 0.26% | 99.5% | 100.5% | 101.0% | 100.2% |
| Class A2 | -0.06% | 0.18% | 0.05% | 99.2% | 101.2% | 99.9% | 100.5% |
| Class B | -0.13% | 0.36% | 0.14% | 99.1% | 102.0% | 99.4% | 100.0% |
| Class C | -0.21% | 0.10% | 0.17% | 96.7% | 96.9% | 99.8% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.14% | 0.28% | 0.15% | 98.5% | 100.1% | 99.9% | 100.1% |
| Class D | -0.34% | 0.69% | 0.40% | 97.8% | 98.2% | 99.3% | 100.1% |
| Class F | -0.26% | -0.15% | -0.07% | 96.4% | 96.5% | 99.5% | 101.5% |
| Class TGM | -0.31% | -0.37% | -0.28% | 108.9% | 111.6% | 99.9% | 104.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | 1.22% | 1.13% | 103.4% | 102.6% | 100.0% | 104.9% |
| Class C | 0.02% | 0.57% | 0.17% | 101.8% | 103.7% | 99.9% | 102.6% |
| Class E | -0.09% | 0.27% | 0.14% | 102.1% | 104.4% | 99.3% | 102.9% |
| Overall | -0.06% | 0.77% | 0.56% | 102.5% | 103.4% | 99.8% | 103.6% |
| Class D | -0.10% | 0.49% | -0.94% | 101.0% | 102.4% | 97.7% | 100.8% |
| Class F | -0.21% | 0.38% | 0.17% | 103.9% | 105.5% | 98.2% | 103.4% |
| Class TGM | -0.53% | -0.59% | -0.51% | 104.7% | 102.1% | 99.4% | 107.7% |

## BVI dataset

The results compare ECM-11.0 trained with CTC dataset (as anchor) vs ECM-11.0 trained with the BVI dataset.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.01% | 0.06% |  |  |  |  |
| Class A2 | -0.02% | 0.08% | 0.13% |  |  |  |  |
| Class B | -0.01% | -0.07% | -0.02% |  |  |  |  |
| Class C | 0.00% | -0.09% | 0.04% |  |  |  |  |
| Class E | 0.06% | -0.12% | 0.27% |  |  |  |  |
| Overall | 0.01% | -0.05% | 0.08% |  |  |  |  |
| Class D | 0.03% | -0.26% | -0.10% |  |  |  |  |
| Class F | 0.59% | 0.63% | 0.77% |  |  |  |  |
| Class TGM | 0.67% | 0.59% | 0.65% |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.03% | 0.35% | 0.05% |  |  |  |  |
| Class A2 | 0.00% | 0.06% | 0.06% |  |  |  |  |
| Class B | 0.02% | -0.09% | 0.26% |  |  |  |  |
| Class C | 0.05% | 0.17% | 0.06% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.03% | 0.10% | 0.12% |  |  |  |  |
| Class D | -0.01% | 0.14% | -0.06% |  |  |  |  |
| Class F | 0.53% | 0.57% | 0.95% |  |  |  |  |
| Class TGM | 0.46% | 0.54% | 0.51% |  |  |  |  |
|  | Y | U | V |  |  |  |  |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.01% | -0.24% | 0.05% |  |  |  |  |
| Class C | 0.04% | -0.61% | -0.44% |  |  |  |  |
| Class E | -0.07% | 1.54% | 0.07% |  |  |  |  |
| Overall | 0.00% | 0.08% | -0.11% |  |  |  |  |
| Class D | -0.17% | 0.46% | 0.67% |  |  |  |  |
| Class F | 0.37% | 0.47% | 0.84% |  |  |  |  |
| Class TGM | 0.34% | 0.33% | 0.21% |  |  |  |  |

The results show that the difference in performance is small except for class F and TGM. It can be explained since the BVI dataset does not contain any screen content sequences, hence the bins specific to the tools of screen content are let ”as-is”.

## BVI dataset + screen content

## In this test, screen content UVG sequences are added to the BVI dataset for training the cabac model parameters. It is observed that the relative loss compared to training on the CTC is reduced for classes F and TGM: in AI the loss is reduced from 0.63% to 0.24%, in RA from 0.5% to 0.33% and LDB from 0.35% to xxx%.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | -0.08% | 0.03% | 100.6% | 103.7% | 99.8% | 101.2% |
| Class C | 0.01% | -0.03% | -0.10% | 101.3% | 100.8% | 101.2% | 100.4% |
| Class E | 0.05% | -0.17% | 0.40% | 96.0% | 100.0% | 104.8% | 100.6% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.03% | -0.21% | -0.01% | 101.2% | 94.6% | 100.3% | 100.3% |
| Class F | 0.16% | 0.41% | 0.45% | 106.0% | 105.7% | 96.7% | 100.5% |
| Class TGM | 0.31% | 0.31% | 0.40% | 107.9% | 106.8% | 101.3% | 100.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.16% | 0.36% | 96.1% | 101.1% | 100.0% | 100.1% |
| Class C | 0.04% | 0.12% | 0.08% | 99.6% | 102.2% | 100.1% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.02% | -0.06% | -0.29% | 98.3% | 95.3% | 100.1% | 100.3% |
| Class F | 0.20% | 0.25% | 0.13% | 90.1% | 96.8% | 100.1% | 99.5% |
| Class TGM | 0.46% | 0.63% | 0.62% | 93.1% | 104.7% | 100.0% | 97.4% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E | -0.04% | -0.16% | -0.04% | 87.2% | 107.0% | 103.1% | 99.9% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.14% | 0.80% | 0.15% | 100.0% | 102.5% | 100.1% | 100.2% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Conclusion

This contribution shows that current entropy model parameters dataset training using the CTC sequences does not have a large impact on the performance. It is proposed to continue to use the current process during the exploration phase. For “final” training, a larger dataset is preferable to cover all use cases of the final codec.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).