| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0090-v2 |
| --- | --- |

| Title: | EE2-2.4: DIMD Merge List |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Médéric Blestel, Pierre Andrivon, Kirill Suverov | Email: | mblestel@ofinno.com pandrivon@ofinno.com |
| Source: | Ofinno |  |  |

_____________________________

# Abstract

The contribution reports the results of the EE2 Test 2.4. The method, initially described in JVET-AG0084, proposes to extend the Decoder-side Intra Mode Derivation (DIMD) process by introducing the generation of a DIMD merge list. The DIMD merge list is built reusing available DIMD information and merged DIMD information (JVET-AF0120) from spatial adjacent and non-adjacent neighboring blocks (JVET-AF0106). The maximal number of candidates in the DIMD merge list is adapted to the width and height of the CU.

On top of ECM-12.0, the reported PSNR-Y, Cb, Cr, BD-rate and {EncT, DecT, EncVmPeak, DecVmPeak} results are as follows:

- AI: -0,09%,	0,01%,	0,02%, {100,7%, 100,2%, 101,2%, 100,1%}

- RA: -0,04%	, -0,01%, -0,07%, { 100,0%,	 100,1%, 100,4%, 100,1%}

# DIMD Merge List

## Proposed approach

The method proposes to create a DIMD merge list from neighboring blocks’ DIMD information.
It includes:

- DIMD information from spatial neighbors (as in Figure 1),

- DIMD information from non-adjacent neighbors (as in Figure 2),

- DIMD information derived from the MHoG (as described in [3]).

Figure 1: neighboring blocks

Figure 2: non-adjacent spatial neighboring candidates for DIMD merge.

Two redundancy checks (pruning stage) are applied: one comparing with existing DIMD merge candidates, and one comparing with DIMD information derived from the current block. Thus, a DIMD merge candidate is added to the DIMD merge list when the associated DIMD information is different from every information from DIMD merge candidates already present in the list and the current block DIMD information.

Two additional flags are added conditioned to the DIMD flag, i.e., the DIMD Merge is considered a sub-mode of DIMD.

The two flags are:

- The DIMD merge mode flag.

- The DIMD merge mode index representative of the DIMD merge candidate.

The CABAC contexts have been reviewed for these two new flags.

JVET-AG146[2], adopted in the ECM-12.0, proposes to adaptively select between planar or block-vector based prediction obtained from IntraTMP or IBC mode of neighboring blocks. This prediction is then blended with angular intra predictions to generate the final DIMD prediction.

The DIMD merge prediction has been designed to use the same mechanism as described in JVET-AG146. Then, for each DIMD merge candidates, like the DIMD, the planar mode can be replaced by a block-vector based prediction.

## DIMD merge candidates’ evaluation

DIMD merge is only available as an option if the current block has at least one neighbor coded with DIMD or DIMD merge modes using the same method as proposed in [2]. Besides, the neighborhood is extended to include part of the non-adjacent candidates (initially proposed in [4]).

When the DIMD merge is available, the DIMD merge list is derived both at the encoder and decoder sides. On the encoder side, each candidate is evaluated using a Hadamard pass, then in the RDO.

To reduce the complexity at the encoder side, several optimizations have been made:

- For a given CU, DIMD and DIMD merge intra-predictions are stored and shared between both modes, to avoid re-generating the intra-predictions during the Hadamard and RDO passes.

- During RDO passes, a DIMD merge candidate may be discarded if the associated cost is not competitive compared to other modes.

- For a given CU, the maximum number of DIMD merge candidates depends on the CU’s width and height. For small CUs, only one candidate is allowed, reducing the signaling and the complexity. The maximum number of candidates increases with the CU’s width and height.

# DIMD statistics

DIMD usage was analyzed in the JVET-AH0090 compared to ECM-12.0, in terms of the number of CU blocks using a DIMD mode, and in terms of pixels.

The DIMD mode usage is then increased by 141% (146% per pixel) in this contribution when compared to its usage in the ECM-12.0. Adding a sub-mode makes DIMD mode more competitive and then more selected, either as a regular DIMD mode or as a DIMD Merge mode.

|  | DIMD |  |
| --- | --- | --- |
|  | Nb CU | Nb pixels |
| ECM-12,0 | 6082432 | 1366378288 |
| JVET-AH0090 | 8566472 | 1998288480 |
| delta | 141% | 146% |
| Max | 177% | 185% |

Table 1: DIMD usage in JVET-AH0090

In JVET-AH0090, the DIMD Merge mode represents about 35% of the DIMD CU block (41% in pixel), and up to 57% in specific content. Note that the DIMD Merge mode is enabled only on conditions, i.e., it is not available for several CUs.

| JVET-AH0090 | DIMD |  |
| --- | --- | --- |
|  | Nb CU | Nb pixels |
| DIMD | 8566472 | 1998288480 |
| DIMDMM | 3025180 | 819875184 |
| delta | 35% | 41% |
| Max | 57% | 62% |

Table 2: DIMD Merge Mode (DIMDMM) usage

# Experimental results

The tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 12.0 using the JVET common test conditions.

Table 3: Performances of the AI test EE2-2.4 over ECM-12.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,13% | 0,07% | -0,05% | 100,1% | 100,1% | 100,4% | 100,3% |
| Class A2 | -0,14% | 0,00% | 0,02% | 100,9% | 100,3% | 100,4% | 100,4% |
| Class B | -0,09% | 0,02% | 0,04% | 100,9% | 100,1% | 100,6% | 99,8% |
| Class C | -0,06% | -0,05% | 0,05% | 100,7% | 100,1% | 101,0% | 100,1% |
| Class E | -0,04% | 0,04% | -0,02% | 100,6% | 100,6% | 103,9% | 100,1% |
| Overall | -0,09% | 0,01% | 0,02% | 100,7% | 100,2% | 101,2% | 100,1% |
| Class D | -0,07% | 0,16% | -0,05% | 100,5% | 99,9% | 100,8% | 100,0% |
| Class F | -0,06% | 0,04% | 0,03% | 100,7% | 100,4% | 100,0% | 100,1% |
| Class TGM | -0,01% | 0,08% | -0,04% | 100,5% | 100,5% | 97,3% | 100,0% |

Table 4: Performances of the RA test EE2-2.4 over ECM-12.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,10% | -0,06% | -0,17% | 99,7% | 99,6% | 100,1% | 99,9% |
| Class A2 | -0,04% | -0,05% | -0,03% | 99,9% | 100,2% | 99,8% | 100,2% |
| Class B | -0,05% | 0,00% | -0,14% | 100,4% | 100,2% | 100,3% | 100,1% |
| Class C | 0,02% | 0,04% | 0,07% | 100,1% | 100,3% | 101,1% | 100,1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,04% | -0,01% | -0,07% | 100,0% | 100,1% | 100,4% | 100,1% |
| Class D | 0,05% | -0,23% | 0,30% | 100,5% | 100,2% | 101,1% | 100,0% |
| Class F | -0,03% | 0,21% | -0,06% | 99,8% | 99,9% | 100,4% | 100,2% |

Several Qps are missing (classA1 and A2), and anchors results were used instead.

# Conclusion

The proposed method enriches the DIMD process by building a list of DIMD information that can be used to predict CU blocks. The DIMD merge list of candidates is built from available DIMD information available in the CU neighborhood. The method provides additional coding gain on top of the ECM-12.0 at a reasonable cost. It is proposed to include this method in the next version of the ECM reference software.

# References

- M. Blestel, P. Andrivon, “AHG12: DIMD Merge List”, JVET-AG0084, January 2024.

- K. Naser and al, “EE2-1.4: IntraTMP extension to DIMD”, JVET-AG0146, January 2024.

- S. Blasi, I. Zupancic, J. Lainema, “AHG12 - Decoder-side Intra Mode Derivation Merge,” JVET-AE0071, July 2023.

- S. Blasi, I. Zupancic, J. Lainema, “EE2-2.1 DIMD merge,” JVET-AF0120, October 2023.

- J. Huo, J. Fan, Z. Zhang, Y. Ma, F. Yang, M. Li, “EE2-related: Non-adjacent spatial candidates for DIMD merge,” JVET-AF0106, October 2023.

# Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).