| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0092-v2 |
| --- | --- |

| Title: | EE2-2.3: Intra-prediction using Merged Histogram of Gradients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saverio Blasi Ivan Zupancic Pekka Astola Jani Lainema | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document reports on experiment EE2-2.3 on Intra-prediction using Merged Histogram of Gradients. The impact on coding efficiency and runtimes of the EE2 tests over ECM-12.0 are reportedly {for Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}:

Test 2.3a:

AI {-0.08%, 0.05%, 0.04%, 100.5%, 99.9%, 99.7%, 100.0%}

RA {-0.04%, -0.07%, -0.12%, 100.2%, 99.9%, 100.1%, 99.9%}

Test 2.3b:

AI {-0.11%, 0.00%, 0.03%, 103.4%, 100.4%, 100.0%, 99.8%}

RA {-0.06%, 0.01%, -0.13%, 100.3%, 100.6%, 100.0%, 100.0%}

# Proposed Method

In test EE2-2.3, Intra-prediction using Merged Histogram of Gradients [1] is tested. When using this method, a new intra prediction mode, referred to as Merged Intra Mode Derivation (MIMD), is used based on the computation of a Merged Histogram of Gradients (MHoG). The MHoG is computed based on information extracted from neighbouring blocks, including both DIMD and non-DIMD neighbours.

In Test 2.3a some restrictions to the usage of MIMD are used to limit the impact on complexity. In Test 2.3b, the unrestricted method is tested.

# Experimental results

The proposed methods are implemented on top of ECM-12.0 [2] and tested based on the common test conditions [3]. The simulation results are summarized in the following Tables.

Table 1: Coding performance of EE2-2.3a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.12% | 0.06% | -0.03% | 100.2% | 100.0% | 100.1% | 99.9% |
| Class A2 | -0.13% | 0.00% | -0.05% | 100.0% | 99.4% | 100.1% | 100.0% |
| Class B | -0.10% | 0.09% | 0.12% | 100.6% | 99.2% | 99.1% | 100.1% |
| Class C | -0.03% | -0.01% | 0.07% | 100.6% | 100.2% | 99.8% | 100.0% |
| Class E | -0.04% | 0.09% | 0.01% | 101.1% | 101.2% | 100.1% | 100.0% |
| Overall | -0.08% | 0.05% | 0.04% | 100.5% | 99.9% | 99.7% | 100.0% |
| Class D | -0.06% | 0.10% | 0.15% | 101.7% | 103.3% | 100.4% | 100.0% |
| Class F | -0.06% | 0.00% | -0.16% | 100.8% | 100.6% | 100.1% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | -0.16% | -0.28% | 100.0% | 100.7% | 100.0% | 99.7% |
| Class A2 | -0.05% | 0.03% | 0.10% | 99.8% | 100.5% | 99.7% | 99.9% |
| Class B | -0.04% | -0.08% | -0.24% | 100.5% | 99.6% | 100.1% | 100.0% |
| Class C | -0.01% | -0.06% | 0.00% | 100.5% | 99.0% | 100.3% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.07% | -0.12% | 100.2% | 99.9% | 100.1% | 99.9% |
| Class D | 0.04% | -0.34% | 0.03% | 100.7% | 104.1% | 100.0% | 100.0% |
| Class F | -0.07% | -0.19% | -0.11% | 100.5% | 99.9% | 99.9% | 100.2% |

Table 2: Coding performance of EE2-2.3b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.13% | -0.04% | 0.04% | 103.6% | 99.6% | 100.0% | 99.9% |
| Class A2 | -0.14% | 0.02% | -0.04% | 102.9% | 99.4% | 100.0% | 99.9% |
| Class B | -0.12% | 0.02% | 0.06% | 103.5% | 101.2% | 99.6% | 99.8% |
| Class C | -0.10% | 0.05% | 0.14% | 103.1% | 100.5% | 100.3% | 99.6% |
| Class E | -0.05% | -0.09% | -0.12% | 103.7% | 100.8% | 100.0% | 99.6% |
| Overall | -0.11% | 0.00% | 0.03% | 103.4% | 100.4% | 100.0% | 99.8% |
| Class D | -0.09% | 0.01% | -0.14% | 104.7% | 104.2% | 100.0% | 99.4% |
| Class F | -0.08% | 0.08% | -0.02% | 102.6% | 100.2% | 100.1% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | -0.12% | -0.14% | 99.9% | 100.7% | 99.9% | 99.7% |
| Class A2 | -0.06% | 0.15% | 0.08% | 99.5% | 100.6% | 99.6% | 100.1% |
| Class B | -0.07% | 0.07% | -0.22% | 100.6% | 100.1% | 100.2% | 100.1% |
| Class C | -0.04% | -0.08% | -0.15% | 100.9% | 101.1% | 99.9% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.06% | 0.01% | -0.13% | 100.3% | 100.6% | 100.0% | 100.0% |
| Class D | -0.04% | 0.15% | 0.30% | 100.7% | 101.6% | 100.2% | 100.1% |
| Class F | -0.07% | -0.15% | -0.08% | 100.7% | 100.9% | 100.0% | 100.3% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- S. Blasi, J. Lainema, “AHG12: Intra-prediction using Merged Histogram of Gradients” JVET-AG0078, Jan. 2024.

- ECM12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct, 2023.