| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0095-v1 |
| --- | --- |

| Title: | AHG12: Regularized EIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongdong Qin Jacek Konieczny Keqin Ding Zhuowei Xu | Email | {hongdong.qin, jacek.k, keqin.ding, zhuowei.xu}@tcl.com |
| Source: | TCL |  |  |

_____________________________

# Abstract

It is proposed to introduce regularized least squares to EIP.

The experimental results over ECM-12.0 are summarized as follows:

AI:		x.xx% (Y), x.xx % (U), x.xx % (V), 100 % (EncT), 100 % (DecT)

RA:	x.xx% (Y), x.xx % (U), x.xx % (V), 100 % (EncT), 100 % (DecT)

# Introduction

ECM uses ordinary least squares methods for several coding modes, including CCCM [1], EIP [2], etc. The ordinary least squares regression uses a ‘solver’ to obtain the coefficients of CCCM/EIP model. It is proposed to introduce regularized least squares regression to EIP.

# Proposed methods

## Regularized EIP (ReEIP)

In EIP, the model coefficients are computed as solving a system of linear equations by Gaussian elimination:

A⊤y=A⊤Aβ

With the solution to the model coefficients β being

β=A⊤A-1A⊤y

It is proposed to introduce L2-regularization to EIP. The model coefficients are computed as:

β=A⊤A+λI-1A⊤y

Where λ is the regularization parameters. L2-regularization is achieved by a diagonal matrix being added to the ‘ATA’ matrix.

The rest parts of EIP solver and prediction are kept unchanged.

## Implementation details

Test 1: ReEIP is always used, with λ=15×128=1920, as 15 is the number of filter taps in EIP.

#define LINEAR_MODEL_L2_PARAM_EIP 128 for (int coli0 = 0; coli0 < numParams; coli0++) { ATA[coli0][coli0] += LINEAR_MODEL_L2_PARAM_EIP * numParams; }

Test 2: ReEIP is always used, with a derived regularization parameter

λ=15×128×m8

m=Clip30, 8,maxcoeffs-mincoeffs+1≪13≫14

m is the dynamic range of EIP coefficients by OLS (maximum value minus minimum value). The 14-bit right-shift comes from the fact that the fractional coefficients are represented as integers with 14-bit decimal accuracy. Compared to Test 1, λ is scaled by a factor of 0, 1/8, 2/8, ..., 7/8 or 1.

Test 3: ReEIP is always used, with a derived regularization parameter

λ=15×128×m32

m=Clip30, 32,c=014ci+1≪13≫14

m is the sum of absolute values of EIP coefficients by OLS. The 14-bit right-shift comes from the fact that the fractional coefficients are represented as integers with 14-bit decimal accuracy. Compared to Test 1, λ is scaled by a factor of 0, 1/32, 2/32, ..., 31/32 or 1.

## Signalling

There is no signalling change as ReEIP is always used and the regularization parameter is fixed or decoder-derived.

# Experimental results

The proposed method was tested on top of ECM-12.0 [3] under the common test conditions [4]. The table below shows the performance results.

Test 1: ReEIP with λ=15*128.

Table 1. Performance results

|  | AI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.05% | 0.01% | 0.03% |  |  |  |  |
| Class C | 0.00% | -0.06% | 0.02% | 100.1% | 100.9% | 99.5% | 100.0% |
| Class E | -0.04% | -0.06% | -0.20% | 100.1% | 100.4% | 103.0% | 99.9% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.02% | 0.18% | 0.01% | 99.3% | 100.0% | 100.0% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Test 2: ReEIP with λ=15*128*m/8. m is the dynamic range of EIP coefficients by OLS.

Table 2. Performance results

|  | AI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.01% | 0.00% | 0.03% | 100.2% | 101.4% | 100.5% | 100.0% |
| Class E | -0.08% | -0.06% | -0.07% | 100.0% | 100.2% | 101.8% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.05% | 0.21% | -0.05% | 99.5% | 100.4% | 100.0% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Test 3: ReEIP with λ=15*128*m/32. m is the sum of absolute values of EIP coefficients by OLS.

Table 3. Performance results

|  | AI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.00% | -0.02% | 0.01% | 99.4% | 101.2% | 100.0% | 100.0% |
| Class E | -0.07% | -0.06% | -0.12% | 99.6% | 100.2% | 100.0% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.04% | 0.11% | -0.07% | 98.8% | 99.1% | 100.0% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Conclusion

In this contribution, regularized least squares is proposed as a replacement of ordinary least squares for computing EIP coefficients. The proposed methods offer coding gains with good tradeoff against encoding time. It is recommended that the proposed methods be studied in the next round of EE2.

# References

- P. Astola, J. Lainema, R. G. Youvalari, A. Aminlou, K. Panusopone, “EE2-1.1a: Convolutional cross-component intra prediction model”, document JVET-AA0057, Jul. 2022.

- L. Xu, Y. Yu, H. Yu, J. Gan, D. Wang, “EE2-1.14: An extrapolation filter-based intra prediction mode”, document JVET-AG0058, Jan. 2024.

- ECM-12.0 reference software, https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, document JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

TCL may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).