| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0095-v3 |
| --- | --- |

| Title: | AHG12: Regularized EIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongdong Qin Jacek Konieczny Keqin Ding Zhuowei Xu | Email | {hongdong.qin, jacek.k, keqin.ding, zhuowei.xu}@tcl.com |
| Source: | TCL |  |  |

_____________________________

# Abstract

It is proposed to introduce regularized least squares to EIP.

The experimental results over ECM-12.0 are summarized as follows:

AI:		-0.04% (Y), -0.02% (U), -0.01% (V), 100.1 % (EncT), 100.5 % (DecT)

# Introduction

ECM uses ordinary least squares methods for several coding modes, including CCCM [1], EIP [2], etc. The ordinary least squares regression uses a ‘solver’ to obtain the coefficients of CCCM/EIP model. It is proposed to introduce regularized least squares regression to EIP.

# Proposed methods

## Regularized EIP (ReEIP)

In EIP, the model coefficients are computed as solving a system of linear equations by Gaussian elimination:

A⊤y=A⊤Aβ

With the solution to the model coefficients β being

β=A⊤A-1A⊤y

It is proposed to introduce L2-regularization to EIP. The model coefficients are computed as:

β=A⊤A+λI-1A⊤y

Where λ is the regularization parameters. L2-regularization is achieved by a diagonal matrix being added to the ‘ATA’ matrix.

The rest parts of EIP solver and prediction are kept unchanged.

## Regularized CCCM (ReCCCM)

L2-regularization can also be applied on CCCM.

## Tests

Test 1: ReEIP is always used, with λ=pEIP×128, where pEIP=15 is the number of filter taps in EIP.

#define LINEAR_MODEL_L2_PARAM_EIP 128 const int regularizationParam = LINEAR_MODEL_L2_PARAM_EIP * numParams; for (int coli0 = 0; coli0 < numParams; coli0++) { ATA[coli0][coli0] += regularizationParam; }

Test 2: In addition to ReEIP, ReCCCM is also always used, with λ=np×32, where np is the number of parameters in the model to be computed. For ReCCCM specifically, the first (co-located luma) and the last (bias) terms are unregularized by setting the top-left and bottom-right entries of the ridge identity matrix to zero.

#define LINEAR_MODEL_L2_PARAM_CCCM 32 #define RECCCM_LEAVE_FIRST_LUMA_OUT 1 #define RECCCM_LEAVE_BIAS_OUT 1 const int regularizationParam = LINEAR_MODEL_L2_PARAM_CCCM * numParams; for( int i = RECCCM_LEAVE_FIRST_LUMA_OUT; i < numParams - RECCCM_LEAVE_BIAS_OUT; i++ ) { ATA[i][i] += regularizationParam; }

## Signalling

There is no signalling change as ReEIP and ReCCCM are always used and the regularization parameters are fixed or decoder-derived.

# Experimental results

The proposed method was tested on top of ECM-12.0 [3] under the common test conditions [4]. The table below shows the performance results.

Test 1: ReEIP with λ=15*128.

|  | AI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | -0.03% | -0.01% | 100.4% | 100.3% | 102.0% | 100.1% |
| Class A2 | -0.03% | 0.06% | -0.01% | 100.1% | 100.2% | 98.9% | 99.8% |
| Class B | -0.05% | 0.01% | 0.03% | 99.8% | 100.3% | 98.7% | 100.0% |
| Class C | 0.00% | -0.06% | 0.02% | 100.1% | 100.9% | 99.5% | 100.0% |
| Class E | -0.04% | -0.06% | -0.20% | 100.1% | 100.4% | 103.0% | 99.9% |
| Overall | -0.04% | -0.01% | -0.02% | 100.1% | 100.5% | 100.2% | 99.9% |
| Class D | -0.02% | 0.18% | 0.01% | 99.3% | 100.0% | 100.0% | 100.0% |
| Class F | 0.03% | -0.07% | 0.13% | 99.2% | 99.9% | 100.0% | 100.0% |
| Class TGM | -0.01% | 0.06% | -0.02% | 100.4% | 99.9% | 101.3% | 100.1% |

*Class A2* is missing DaylightRoad2 QP 22. The line is filled with Anchor.

Test 1x: ReEIP with λ=15*128 and ReCCCM with λ=*32.

|  | AI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.01% | -0.09% | 0.06% |  |  |  |  |
| Class E | -0.04% | -0.20% | -0.08% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.04% | -0.03% | -0.11% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Conclusion

In this contribution, regularized least squares is proposed as a replacement of ordinary least squares for computing EIP coefficients. The proposed methods offer coding gains with good tradeoff against encoding time. It is recommended that the proposed methods be studied in the next round of EE2.

# References

- P. Astola, J. Lainema, R. G. Youvalari, A. Aminlou, K. Panusopone, “EE2-1.1a: Convolutional cross-component intra prediction model”, document JVET-AA0057, Jul. 2022.

- L. Xu, Y. Yu, H. Yu, J. Gan, D. Wang, “EE2-1.14: An extrapolation filter-based intra prediction mode”, document JVET-AG0058, Jan. 2024.

- ECM-12.0 reference software, https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, document JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

TCL may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).