| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0096-v1 |
| --- | --- |

| Title: | EE1-1.4: Content-adaptive loop-filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Maria Santamaria Ruiying Yang Francesco Cricri Miska M. Hannuksela Döne Bugdayci Sansli Antti Hallapuro Hamed R. Tavakoli Jani Lainema Honglei Zhang Nokia Technologies, Tampere, Finland | Tel: Email: | {maria.santamaria_gomez, ruiying.yang, francesco.cricri, miska.hannuksela, done.bugdayci_sansli, antti.hallapuro, hamed.rezazadegan_tavakoli, jani.lainema, honglei.1.zhang}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the results for the EE1-1.4 test which studies the loop filter in JVET-AG0111 with content adaptation per random-access segment and application to the LOP2 loop-filter. The content-adaptive parameters are signalled via a specifically-designed Adaption Parameter Set (APS) called Neural Network Filter Update (NNFU). It is reported that the average coding gains of this approach in the RA configuration are 1.18% (Y), 2.18% (Cb) and 2.62% (Cr) with respect to NNVC-8.0 with NN intra tool ON and LOP2 tool ON.

# Introduction

The content-adaptive approach for neural network loop-filter, as detailed in JVET-AG0111 [1], is achieved by means of overfitting the LOP2 filter in NNVC-8.0 independently for each random-access segment. The generated weight-updates are coded with the MPEG-NNR standard [2, 3] and signalled within a NNFU APS. Both the training code and inference code are available in [4].

# Content adaptation via NN overfitting

The content-adaptive LOP filter is obtained by overfitting multiplier parameters that are applied after a convolutional block, like:

σW*x+b∙m,
where W is the kernel, * is the convolution operator, x is the input, b is the bias, m is the multiplier and σ is the activation function.

The network architecture is based on JVET-AF0043 [5] (Figure 1) and the difference is that the multiplier layers are added after each convolutional block in the luma and chroma backbone.

Figure 1. LOP2 filter architecture from JVET-AF0043.

The content adaptation is done per RA segment independently, and the weight-update is signaled within the first B slice of such segment. In order to lower the overhead of the weight-update that is coded with MPEG-NNR, only a subset of multiplier layers is overfitted. The multiplier layers to overfit are selected by L1-norm. Specifically, first, all multiplier layers are overfitted for 10 iterations; then, assuming that K is the number of trainable multiplier layers, the top 0.8⋅K  and 0.2⋅K   multiplier layers with highest L1-norm in luma and chroma backbones, respectively, will be overfitted.  K is determined by bitrate overhead percentage, as the following steps show:

- In an offline stage, an optimal class-wise ratio rC is determined, which indicates an acceptable bitrate increase.

- Let bm be an empirically determined average bitrate of a NNR bitstream representing one multiplier layer overfitted on one segment. Let Vi be the bitrate of the i-th anchor-coded video sequence. The number of multiplier layers, K, to overfit in each segment is given by

K=rc*Vibm

Thus, the number Kof trainable multiplier layers is dependent on the specific video sequence.

Table 1 shows the details of NN information during the overfitting stage.

Table 1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Overfitting Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU NVIDIA A100-SXM-80GB |
|  | Framework: | PyTorch 1.12.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 100 epochs |
|  | Batch size: | 64 |
|  | Loss function: | Weighted-MSE |
|  | Training time (for 1 model – 1 segment): | A1 & A2: from 30 min to 5h B: from 20 min to 1.5 h C: from 3 min to 10 min D: from 2 min to 6 min F: from 6 min to 1.5 h |
|  | Training data information: | JVET CTC RA mandatory sequences |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-3 |
|  | Learning rate update strategy | Constant learning rate |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Up-sample 72x72 UV signal to 144x144 blocks Normalize to 0~1 |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

# Quantization

The floating-point overfitted models, obtained after decoding the NNR weight updates, were quantized to int16 using the same quantizers as the base models (for convolutional kernels and bias terms). Additionally, the multiplier layers use a fixed quantizer of 13.

# Neural network update signalling

The MPEG-NNR bitstream is carried within a specifically-designed NN filter update APS NAL unit, which is described in this section.

## Proposed specification text

### Adaptation parameter set

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| aps_params_type | u(3) |
| aps_adaptation_parameter_set_id | u(5) |
| aps_chroma_present_flag | u(1) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| else if( aps_params_type = = NN_UPDATE_APS ) |  |
| nn_filter_update_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

aps_params_type specifies the type of APS parameters carried in the APS as specified in Table 2. The value of aps_params_type shall be in the range of 0 to 2, inclusive, in bitstreams conforming to this version of this Specification. Other values of aps_params_type are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore APS NAL units with reserved values of aps_params_type.

Table 2. APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3 | NN_UPDATE_APS | Neural-network filter update |

All APS NAL units with a particular value of aps_params_type, regardless of the nuh_layer_id values and whether they are prefix or suffix APS NAL units, share the same value space for aps_adaptation_parameter_set_id. APS NAL units with different values of aps_params_type use separate values spaces for aps_adaptation_parameter_set_id.

aps_adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements.

When aps_params_type is equal to ALF_APS or SCALING_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive.

When aps_params_type is equal to LMCS_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

When aps_params_type is equal to NN_UPDATE_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

### Neural network filter update data syntax

| nn_filter_update_data( ) { | Descriptor |
| --- | --- |
| nnfu_bitstream_size_minus1 | u(16) |
| for( i = 0; i <= nnfu_bitstream_size_minus1; i++ ) |  |
| nnfu_payload_byte[ i ] | u(8) |
| } |  |

nnfu_bitstream_size_minus1 plus 1 specifies the size in bytes of the nnfu_payload.

nnfu_payload_byte[ i ] contains the i-th byte of a bitstream conforming to ISO/IEC 15938-17. The byte sequence nnfu_payload_byte[ i ] for all present values of i shall be a complete bitstream that conforms to ISO/IEC 15938-17.

# Inference

Table 3 describes the loop-filter information at inference stage.

Table 3. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task |  |
|  | Number of Parameters (Each Model) | 52263 |
|  | Total Number of Parameters (All Models) | 52263 |
|  | Parameter Precision (Bits) | Int16 |
|  | Memory Parameter (MB) | 0.1 |
|  | Multiply Accumulate (kMAC/pixel) | 17.6 |
|  | Calculation Method | on a block basis |
| Optional |  |  |
|  | Total Conv. Layers | 106 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 128x128 / 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

## Encoder

Each RA segment is overfitted independently and once the overfitted model is ready to be deployed, the encoding of the RA segment is also done independently from other segments. Adjacent RA segments have overlapping I-frames and these are filtered using the original LOP2 filter.

After a RA segment of a sequence-QP pair has been encoded using the respective overfitted LOP2 filter, a decision is performed to determine whether this segment or the one encoded with the anchor configuration (i.e., with the base model) is better in terms of delta PSNR. Therefore, a final merged bitstream may include some RA segments filtered with the original (base) LOP2 filter and some RA segments filtered with the overfitted LOP2 filter.

## Decoder

When the decoder parses an NNFU APS, the reconstruction of the overfitted LOP2 filter is triggered. The process relies on the original (base) LOP2 filter and the decoded NNR bitstream carried within the NNFU APS.

# Experimental results

Table 4 shows the coding performance of the proposed approach (content-adaptive LOP2 and NN-intra ON) in the RA configuration over NNVC-8.0 with LOP2 and NN-intra ON.

Table 4. Coding performance over NNVC-8.0 with LOP2 and NN intra ON

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0rc3 |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |  | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -2,34 % | -1,13 % | -5,17 % | -3,72 % | -1,10 % | -3,90 % |  | 199 % | 104 % | 100 % |
| Class A2 | -0,68 % | -1,50 % | -1,21 % | 0,18 % | -0,94 % | -1,63 % |  | 202 % | 98 % | 100 % |
| Class B | -1,41 % | -4,52 % | -3,34 % | 0,65 % | -3,92 % | -1,40 % |  | 202 % | 108 % | 99 % |
| Class C | -0,40 % | -0,56 % | -0,89 % | 0,25 % | -0,71 % | -0,45 % |  | 201 % | 101 % | 100 % |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1,18 % | -2,18 % | -2,62 % | -0,43 % | -1,90 % | -1,70 % |  | 201 % | 103 % | 100 % |
| Class D | -0,02 % | -0,17 % | -0,11 % | -0,01 % | -0,11 % | 0,06 % |  | 199 % | 100 % | 100 % |
| Class F | -0,35 % | -0,61 % | -0,86 % | 0,04 % | -0,78 % | -1,29 % |  | 202 % | 130 % | 100 % |
| Class H |  |  |  |  |  |  |  |  |  |  |

# Recommendation

Adopt the content-adaptive LOP filter described in this document into the next version of NNVC software.

# References

- R. Yang, M. Santamaria, F. Cricri, M. M. Hannuksela, H. Zhang, J. Lainema, A. Hallapuro, D. B. Sansli. EE1-2.2: Content-adaptive LOP filter. JVET-AG0111. January 2024.

- ISO/IEC FDIS 15938-17. Information technology – Multimedia content description interface – Part 17: Compression of neural networks for multimedia content description and analysis.

- Test Model of Incremental Compression of Neural Networks for Multimedia Content Description and Analysis (INCTM), N0179. February 2022.

- https://vcgit.hhi.fraunhofer.de/jvet-ag-ee1/VVCSoftware_VTM/-/tree/EE1-1.4

- D. Rusanovskyy, Y.Li, M. Karczewicz, T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang, R. Chang, L. Wang, X. Xu, S. Liu. AHG11: Status of the joint EE1-0 (LOP.2) unified filter training. JVET-AF0043. October 2023.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).