| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0097_v1 |
| --- | --- |

| Title: | EE2-2.14: Bilateral Filtering for Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Yang Wang1, Zhipin Deng1, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In this contribution, the results of EE2-2.14 are reported. In EE2-2.14, Bilateral Filtering (BF) is applied to intra prediction samples. Intra prediction samples are filtered before they are used in the later process. The proposed method is not applied to planar mode in test 2.14a and applied to planar mode in test 2.14b.

On top of ECM-12.0, simulation results of test 2.14a and test 2.14b are reported as below:

EE2-2.14a:

AI:  -0.07%, -0.07%, -0.04%, 100.9%, 100.4%.

RA: %, %, %, %, %.

EE2-2.14b:

AI:  -0.08%, 0.01%, -0.01%, 101.3%, 100.3%.

RA: %, %, %, %, %.

Introduction

Intra prediction samples are generated from intra reference samples. In general, the generated intra prediction samples may contain coding noises. The bilateral filter is suitable to reduce coding noises inside prediction since it can achieve edge-protected denoising.

Proposed Method

In EE2-2.14, intra prediction samples are further filtered by bilateral filtering. The bilateral filter used in EE2-2.14 follows the design of the bilateral filter used in the loop-filtering stage. The filter length is kept as 2. The proposed method is not applied to planar mode in 2.14a and applied to planar mode in 2.14b.

Simulation Results

The EE2-2.14 was implemented on top of the ECM-12.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The results compared to ECM-12.0 are shown in Table 1 and Table 2 for test 2.14a and test 2.14b respectively.

Table 1: Performance of EE2-2.14a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | 0.01% | 0.07% | 102.0% | 100.6% | 101.1% | 100.2% |
| Class A2 | -0.08% | -0.07% | -0.05% | 101.2% | 99.5% | 99.6% | 99.9% |
| Class B | -0.06% | -0.08% | -0.02% | 101.1% | 99.7% | 99.2% | 100.1% |
| Class C | -0.11% | -0.13% | -0.09% | 99.9% | 101.7% | 99.0% | 100.0% |
| Class E | -0.05% | -0.04% | -0.13% | 100.4% | 100.5% | 100.0% | 100.0% |
| Overall | -0.07% | -0.07% | -0.04% | 100.9% | 100.4% | 99.7% | 100.0% |
| Class D | -0.11% | -0.03% | -0.07% | 99.9% | 100.2% | 99.9% | 100.0% |
| Class F | -0.07% | -0.10% | -0.07% | 100.0% | 100.9% | 101.6% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.08% | -0.09% | 100.4% | 100.6% | 100.1% | 99.8% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | 0.12% | -0.14% | 99.1% | 99.4% | 99.7% | 99.8% |
| Class C | 0.02% | -0.01% | 0.00% | 99.8% | 99.7% | 99.6% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.04% | -0.12% | 0.37% | 99.5% | 99.3% | 99.9% | 100.0% |
| Class F | -0.03% | -0.04% | 0.07% | 100.1% | 99.6% | 100.1% | 99.9% |

Table 2: Performance of EE2-2.14b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | 0.13% | 0.04% | 99.1% | 99.8% | 101.0% | 99.9% |
| Class A2 | -0.10% | 0.02% | -0.03% | 101.0% | 99.7% | 98.7% | 100.0% |
| Class B | -0.07% | -0.02% | 0.01% | 103.7% | 100.2% | 101.1% | 100.0% |
| Class C | -0.12% | -0.14% | -0.09% | 100.0% | 100.5% | 100.5% | 100.0% |
| Class E | -0.05% | 0.10% | 0.04% | 101.8% | 101.0% | 102.1% | 100.0% |
| Overall | -0.08% | 0.01% | -0.01% | 101.3% | 100.3% | 100.7% | 100.0% |
| Class D | -0.13% | 0.02% | -0.10% | 99.1% | 100.9% | 100.0% | 100.0% |
| Class F | -0.09% | -0.08% | -0.20% | 98.6% | 100.6% | 100.8% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.16% | -0.02% | 101.1% | 101.6% | 100.0% | 99.6% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.03% | 0.06% | -0.19% | 99.6% | 99.1% | 100.0% | 99.8% |
| Class C | 0.00% | -0.17% | -0.04% | 100.3% | 99.4% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.05% | -0.43% | 0.09% | 99.1% | 99.3% | 100.0% | 100.0% |
| Class F | -0.02% | -0.01% | 0.00% | 100.6% | 100.0% | 100.1% | 99.9% |

Conclusions

In this contribution, results of EE2-2.14 are reported. It is observed that EE2-2.14 can achieve promising coding performance with negligible encoding and decoding time increase. It is recommended to adopt EE2-2.14 into the next version of ECM.

Reference

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AF2017, DE, October 2023.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).