| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET- AH0099-v1 |
| --- | --- |

| Title: | EE1-4.1: Adaptive resolution coding for NNVC-8.0 with RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin, Yue Li, Junru Li, Kai Zhang, Li Zhang (Bytedance Inc.) Zhuoyi Lv, Chuan Zhou (vivo) | Email: | linchaoyi.cy@bytedance.com {zhuoyi.lv, chuan.zhou}@ vivo.com |
| Source: | Bytedance Inc. vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This document reports the results of EE1-4.1: Adaptive resolution coding for NNVC-8.0 with RPR using scaling factor s=1.5 2.

The overall BD rate savings are：

- s=2.0: {-0.39%, 2.09%, 2.66%} and {-0.59%, 2.31%, 2.52%} for AI and RA configurations, respectively.

- s=1.5: {-0.65%, 1.20%, 1.53%} and {-0.78%, 1.39%, 1.48%} for AI and RA configurations, respectively.

Comparison to NNVC-8.0 anchor

The NNVC-8.0 anchor with NN-Intra and LOP-2 filter enabled. The resampling strategy is disabled for QP < 32.

Table 1 s=2

|  | AI Main10 |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.48% | 3.38% | 4.33% | -3.06% | 1.25% | 2.77% |
| Class A2 | -0.85% | 9.15% | 11.64% | 0.09% | 10.28% | 9.82% |
| Overall | -0.39% | 2.09% | 2.66% | -0.59% | 2.31% | 2.52% |

Table 2 s=1.5

|  | AI Main10 |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.27% | 0.19% | 1.28% | -2.72% | 0.02% | 0.74% |
| Class A2 | -2.65% | 7.00% | 7.92% | -1.19% | 6.95% | 6.65% |
| Overall | -0.65% | 1.20% | 1.53% | -0.78% | 1.39% | 1.48% |

Conclusion

In this document, the results of EE1-4.1: Adaptive resolution coding for NNVC-8.0 with RPR using scaling factor s=1.5 2 are reported.

References

- E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AF2016

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).