| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0101-v1 |
| --- | --- |

| Title: | EE2-4.2: Fix on LFNST/NSPT index signalling for inter coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Jie Zhao Seung-Hwan Kim 10680 Treena St. St# 550 San Diego, CA 92131, USA | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com jaehyun.lim@lge.com jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this contribution, test results of EE2-4.2 (Fix on LFNST/NSPT index signalling for inter coding) are reported, of which proposal was originally shown in JVET-AG0237. In the last JVET meeting, it was adopted into ECM utilizing LFNST/NSPT for inter coding (called inter-LFNST/NSPT). Since index signalling for inter-LFNST/NSPT is aligned with intra case, an LFNST/NSPT index is signalled only when so-called “DC condition” is met, also for inter coding. Here, meeting “DC condition” for a component indicates that last non-zero coefficient of the component should not be at DC position. Since inter-LFNST/NSPT is applied to only Luma component, index signalling for inter-LFNST/NSPT should be performed only if DC condition for Luma is met. However, in ECM (ECM-12.0), an LFNST/NSPT index can be signalled even when DC condition for Luma is not met, because current DC condition for inter-LFNST/NSPT is affected by both Luma and Chroma. In this contribution, a simple and straightforward fix for this problem was tested on top of ECM-12.0. The BD-rate and complexity changes for RA with the proposed fix are -0.00%/-0.02%/-0.05% for Y/U/V with 100% EncT and 100% DecT.

Problem Statement

Since LFNST/NSPT for inter coding is applied to only Luma component and an LFNST/NSPT index is signalled only when a set of DC condition (i.e., last non-zero coefficient of each of relevant components is not at DC position) is met, it makes sense that the Luma component should satisfy its own DC condition when inter-LFNST/NSPT is performed. However, in the current ECM, DC condition checking process considers both Luma and Chroma for inter-LFNST/NSPT. Furthermore, it is effectively hard coded that DC condition for each Chroma component is always true when non-zero coefficient exists in the Chroma component, which forces the entire DC condition as true irrespective of Luma. As a result, an LFNST/NSPT index, which is set as 0, can be signalled unnecessarily, even when last non-zero coefficient of Luma component lies in DC position. The problematic code fragment is shown in Figure 1.

void CABACWriter::residual_coding( …… ) { ...... if (cuCtx && tu.mtsIdx[compID] != MTS_SKIP && tu.blocks[compID].height >= 4 && tu.blocks[compID].width >= 4) { const int lfnstLastScanPosTh = CU::isIntra(*(tu.cu)) ? (isLuma(compID) ? LFNST_LAST_SIG_LUMA : LFNST_LAST_SIG_CHROMA) : (isLuma(compID) ? LFNST_LAST_SIG_LUMA_INTER : LFNST_LAST_SIG_CHROMA_INTER); // LFNST_LAST_SIG_LUMA_INTER = 1, LFNST_LAST_SIG_CHROMA_INTER = 0 cuCtx->lfnstLastScanPos ¦= cctx.scanPosLast() >= lfnstLastScanPosTh; } ...... }

Figure 1  The problematic code snippet related to inter-LFNST/NSPT index signalling in ECM-12.0

More precisely, through the code shown in Figure 1, the DC condition (DC_cond) is determined by the following logical operations:

DC_cond = ( Y_DC_cond && eff_Y_flag ) || ( Cb_DC_cond && eff_Cb_flag ) || ( Cr_DC_cond && eff_Cr_flag )

eff_Y_flag = Y_CBF && LN_Y_size_cond

eff_Cb_flag = Cb_CBF && LN_Cb_size_cond

eff_Cr_flag = Cr_CBF && LN_Cr_size_cond

In the above operations, Y_DC_cond, Cr_DC_cond, and Cr_DC_cond mean DC conditions for Y, Cb, and Cr components, respectively. Y_CBF, Cb_CBF, and Cr_CBF indicate Coded Block Flags (CBF) for Y, Cb, and Cr components, respectively, indicating whether or not non-zero coefficients exist for each component. LN_Y_size_cond, LN_Cb_size_cond, and LN_Cr_size_cond are set as true, when both width and height are greater than or equal to 4 for Y, Cb, and Cr components, respectively. Specifically, for 4xN/Nx4 block (N ≥ 4), only LN_Y_size_cond is set as true.

According to Figure 1, Cb_DC_cond and Cr_DC_cond are always true, because LFNST_LAST_SIG_CHROMA_INTER is currently set as 0. Therefore, if one of eff_Cb_flag and eff_Cr_flag is true, DC_cond becomes true, irrespective of whether Y_DC_cond is true or not, which can lead the unnecessary signalling even when Y_DC_cond is false.

Proposed fix

A simple and straightforward fix for the problem pointed out in Section 1 is proposed in the following code (Figure 2):

if (cuCtx && tu.mtsIdx[compID] != MTS_SKIP && tu.blocks[compID].height >= 4 && tu.blocks[compID].width >= 4) { const int lfnstLastScanPosTh = isLuma( compID ) ? ( CU::isIntra(*(tu.cu)) ? LFNST_LAST_SIG_LUMA : LFNST_LAST_SIG_LUMA_INTER ) : LFNST_LAST_SIG_CHROMA; cuCtx->lfnstLastScanPos ¦= ( CU::isIntra( *( tu.cu ) ) ¦¦ isLuma( compID ) ) ? ( cctx.scanPosLast() >= lfnstLastScanPosTh ) : false; }

Figure 2  The proposed fix for the problem pointed out in Section 1

In Figure 2, you can find that “cuCtx->lfnstLastCanPos” is not updated by Chroma components when inter-LFNST/NSPT is applied, because checking DC condition for the Chroma components is bypassed by setting them as false. Following the same notion as in Section 1, with the proposed fix shown in Figure 2, the DC condition is decided in the following manner:

DC_cond = ( Y_DC_cond && eff_Y_flag )

eff_Y_flag = Y_CBF && LN_Y_size_cond

With the above operations that depend only on Luma, LFNST/NSPT index signalling for inter coding becomes more consistent with current LFNST/NSPT design, since inter-LFNST/NSPT is applied to only Luma component.

Experimental results

The proposed fix has been implemented based on ECM-12.0 reference software according to common test conditions defined in JVET-AF2017 [2].

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.13% | -0.15% | 101% | 101% |
| Class A2 | 0.01% | -0.01% | -0.05% | 101% | 101% |
| Class B | 0.01% | 0.06% | -0.02% | 100% | 100% |
| Class C | 0.00% | -0.04% | -0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.002% | -0.02% | -0.05% | 100% | 100% |
| Class D | 0.02% | 0.15% | 0.03% | 100% | 100% |
| Class F | -0.04% | -0.02% | -0.18% | 100% | 100% |

Conclusion

In this contribution, test results EE2-4.2 (Fix on LFNST/NSPT index signalling for inter coding) were provided. In the current ECM, unnecessary LFNST/NSPT index signalling can occur in inter coding even when DC condition for Luma component is not met, which is completely resolved by the proposed simple and straightforward fix that also makes inter-LFNST/NSPT index signalling more consistent with current LFNST/NSPT design. It is recommended to adopt the proposed fix in next ECM.

References

[1] M. Koo, J. Zhao, J. Lim, and S. Kim, “EE2-3.3 related: Fix on LFNST/NSPT index signalling”, JVET-AG0237, Jan. 2024.

[2] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).