| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0103-v1 |
| --- | --- |

| Title: | Non-EE2: Low-delay configurations for LFNST/NSPT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Yongjo Ahn Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Jie Zhao Seung-Hwan Kim 10680 Treena St. St# 550 San Diego, CA 92131, USA | Tel: Email: | +82-2-6912-6561 moonmo.koo@lge.com yongjo.ahn@lge.com junghak.nam@lge.com jaehyun.lim@lge.com jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In current ECM common test condition, LFNST/NSPT is disabled under low-delay configurations without evaluating its potential coding gain and complexity reduction. In this contribution, two low-delay configurations with LFNST/NSPT enabled are investigated with fast encoder implementation based on adaptive early termination of LFNST/NSPT. In Test A, intra-LFNST/NSPT is enabled for I-slice and inter-LFNST/NSPT is always enabled. In Test B, only inter-LFNST/NSPT is enabled. More details on the two configurations are as follows;

|  | Intra-LFNST/NSPT* |  | Inter-LFNST/NSPT* with fast encoder |
| --- | --- | --- | --- |
|  | Enabled in I-slice | Enabled in P/B slice |  |
| Test A | O | X | O |
| Test B | X | X | O |

* Intra(Inter)-LFNST/NSPT: LFNST/NSPT for intra (inter) coding

The BD-rate and complexity changes of Test A relative to ECM-12.0 are:

- LDB:  -1.15%/-0.29%/-0.58% for Y/U/V with 101.7% EncT and 101.6% DecT

- LDP:  -1.24%/-0.31%/-0.32% for Y/U/V with 101.9% EncT and 101.5% DecT

The BD-rate and complexity changes of Test B relative to ECM-12.0 are:

- LDB:  -0.77%/0.17%/0.04% for Y/U/V with 101.2% EncT and 101.6% DecT

- LDP:  -0.95%/0.14%/0.25% for Y/U/V with 101.6% EncT and 101.7% DecT

Problem Statement

In the 33th JVET meeting, it was adopted into ECM to utilize LFNST/NSPT for inter coding [1]. Due to this adoption, coding gain that can be realized by LFNST/NSPT became higher for low-delay configuration, but required run time was still undesirable (according to the internal test, -1.56%/-0.23%/-0.76% for Y/U/V with 113% EncT and 102% DecT relative to ECM-11.0, when LFNST/NSPT was enabled for both intra and inter coding). Since encoding time change under LDB due to inter-LFNST/NSPT is reported as 103.8% in [1], encoding complexity for both intra- and inter- LFNST/NSPT should be reduced in order to enable LFNST/NSPT for low-delay configurations.

Proposed configuration

For decreasing encoding run time, 1) intra-LFNST/NSPT is enabled for only I-slice (disabled for P/B-slice) or turned off, and 2) a faster encoding option is applied to inter-LFNST/NSPT.

The faster encoding option consists of a) lowering a threshold value for comparing with cost ratio, which is used for early termination by not trying inter-LFNST/NSPT, and b) disabling adjustment of the threshold value adaptive to block size.

In this contribution, the following two configurations are tested:

- Test A:  intra-LFNST/NSPT is enabled for only I-slice, and inter-LFNST/NSPT is enabled with the faster encoding option.

- Test B:  intra-LFNST/NSPT is disabled, and inter-LFNST/NSPT is enabled with the faster encoding option.

In order to facilitate Test A/B in ECM, three SPS (Sequence Parameter Set) syntax elements are employed (i.e. sps_intra_lfnst_intra_slice_enabled_flag, sps_intra_lfnst_inter_slice_enabled_flag, sps_inter_lfnst_enabled_flag), and the following encoding options are added to .cfg files (e.g. encoder_lowdelay_ecm.cfg) for ECM:

| # Test A IntraLFNSTISlice : 1 IntraLFNSTPBSlice : 0 InterLFNST : 1 FastInterLFNST : 1 | # Test B IntraLFNSTISlice : 0 IntraLFNSTPBSlice : 0 InterLFNST : 1 FastInterLFNST : 1 |
| --- | --- |

Experimental results

The proposed configurations have been implemented based on ECM-12.0 reference software according to common test conditions defined in JVET-AF2017 [2].

Test A

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.85% | -0.32% | -0.43% | 101.2% | 100.9% |
| Class C | -1.54% | -0.54% | -0.17% | 102.0% | 102.1% |
| Class E | -1.14% | 0.08% | -1.37% | 102.0% | 102.1% |
| Overall | -1.15% | -0.29% | -0.58% | 101.7% | 101.6% |
| Class D | -0.91% | 0.41% | -0.20% | 102.2% | 102.6% |
| Class F | -1.25% | 0.11% | -0.33% | 101.5% | 102.9% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.97% | -0.29% | -0.35% | 101.7% | 100.8% |
| Class C | -1.50% | -0.36% | 0.06% | 101.9% | 102.0% |
| Class E | -1.34% | -0.28% | -0.78% | 102.3% | 101.9% |
| Overall | -1.24% | -0.31% | -0.32% | 101.9% | 101.5% |
| Class D | -1.16% | -1.03% | -0.06% | 102.0% | 101.1% |
| Class F | -1.28% | 0.21% | 0.17% | 101.6% | 100.1% |

Test B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.74% | 0.28% | 0.41% | 100.9% | 100.7% |
| Class C | -0.84% | 0.07% | 0.23% | 101.5% | 102.2% |
| Class E | -0.72% | 0.14% | -0.81% | 101.4% | 102.1% |
| Overall | -0.77% | 0.17% | 0.04% | 101.2% | 101.6% |
| Class D | -0.78% | 0.66% | 0.16% | 101.3% | 102.8% |
| Class F | -0.32% | -0.07% | -1.13% | 101.1% | 100.7% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.83% | 0.20% | 0.46% | 101.6% | 101.0% |
| Class C | -1.02% | 0.17% | 0.21% | 101.5% | 101.7% |
| Class E | -1.08% | 0.00% | -0.02% | 101.7% | 102.7% |
| Overall | -0.95% | 0.14% | 0.25% | 101.6% | 101.7% |
| Class D | -0.96% | -0.48% | 0.56% | 101.7% | 101.4% |
| Class F | -0.55% | 0.13% | 0.60% | 101.1% | 100.5% |

Conclusion

In this contribution, low-delay configurations for LFNST/NSPT are proposed, where intra-LFNST/NSPT is disabled or enabled only for I-slice, together with faster encoder option for inter-LFNST/NSPT. With the proposed configurations, significant coding gains, i.e. 1.15% and 0.77% under LDB for Test A/B respectively, were achieved under low-delay configurations with reasonable encoding complexity (i.e. less than 2%). It is recommended to adopt the proposed configurations into next ECM.

References

[1] F. Wang, J. Gan, Y. Yu, H. Yu, and D. Wang, “EE2-3.3: Utilizing LFNST/NSPT for inter coding”, JVET-AG0061, Jan. 2024.

[2] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).