| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0104-v1 |
| --- | --- |

| Title: | EE1-4.2: Adaptive resolution coding for NNVC-8 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuan Zhou Zhuoyi Lv | Email: | chuan.zhou@vivo.com zhuoyi.lv@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of EE1-4.2, a unified CNN-based super-resolution method that replaces the low-resolution reconstruction with the high-resolution reconstruction after RPR and includes scaling ratio as an input, to support multiple scaling factors.

The average BD rate savings on top of NNVC-8.0 anchor are reported as follows:

Test 4.2.1 (s=2)

AI { -0.77%, 1.19%, 1.65% }  RA { -1.02%, 1.37%, 1.40% }

Test 4.2.2 (s=1.5)

AI { -0.74%, 0.75%, 1.11% }  RA { -0.95%, 0.83%, 0.87% }

Additional results compared to NNVC-8.0 RPR are reported as follows:

Test 4.2.1 (s=2)

AI { -0.39%, -0.90%, -0.93% }  RA { -0.44%, -1.06%, -1.18% }

Test 4.2.2 (s=1.5)

AI { -0.09%, -0.46%, -0.40% }  RA { -0.18%, -0.59%, -0.60% }

Introduction

In many CNN-based super-resolution proposals, the low-resolution reconstruction is used as the input of the network, while the high-resolution reconstruction after RPR is used as the input of shortcut connection layer. Moreover, a network structure can only handle one scaling factor.

JVET-AG0129 replaces the low-resolution reconstruction with the high-resolution reconstruction after RPR and introduces scaling ratio as an input of the network, allowing a single network to handle multiple scaling factors.

Proposed method

The proposed unified architecture is illustrated in Figure 1. The high-resolution reconstruction after RPR, scaling ratio, slice QP and base QP are fed into the proposed network same as proposed in JVET-AG0129.

Figure 2 network architecture

Training details

Table 1 Network information for models in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | A100 |
|  | Framework: | PyTorch v2.1.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 60 |
|  | Batch size: | 128 |
|  | Training time: | ~10 days |
|  | Training data information: | DIV2K, BVI_DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Turn on NNLF-LOP2 and NNIntra |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128 |
|  | Learning rate: | 4e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Inference

The network information in the inference stage is provided in Table 2.

Table 2 Network information for the proposed models in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | Framework: | SADL |
|  | Number of GPUs per Task |  |
|  | Number of Parameters (Each Model) | 0.05M |
|  | Total Parameter Number | 0.05M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 0.2MB |
|  | Multiply Accumulate (MAC) | 13kMAC/pixel |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | 256 |

Experimental results

The proposed unified NNSR is tested on top of NNVC 8.0 anchor, according to the common test conditions.

Table 3 Results of 4.2.1 (s=2)

|  | AI Main10 |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.80% | 2.95% | 3.54% | -3.74% | 0.32% | 0.68% |
| Class A2 | -2.81% | 4.17% | 6.34% | -1.37% | 6.55% | 6.31% |
| Overall | -0.77% | 1.19% | 1.65% | -1.02% | 1.37% | 1.40% |

Table 4 Results of 4.2.2 (s=1.5)

|  | AI Main10 |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.42% | 0.03% | 1.04% | -3.03% | -0.34% | -0.04% |
| Class A2 | -3.02% | 4.49% | 5.62% | -1.73% | 4.47% | 4.41% |
| Overall | -0.74% | 0.75% | 1.11% | -0.95% | 0.83% | 0.87% |

The proposed unified NNSR is tested on top of NNVC 8.0 PRR.

Table 3 Results of 4.2.1 (s=2)

|  | AI Main10 |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.32% | -0.43% | -0.77% | -0.69% | -0.97% | -2.15% |
| Class A2 | -2.00% | -4.98% | -4.82% | -1.49% | -4.35% | -3.77% |
| Overall | -0.39% | -0.90% | -0.93% | -0.44% | -1.06% | -1.18% |

Table 4 Results of 4.2.2 (s=1.5)

|  | AI Main10 |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.16% | -0.18% | -0.23% | -0.33% | -0.36% | -0.78% |
| Class A2 | -0.39% | -2.57% | -2.20% | -0.57% | -2.59% | -2.20% |
| Overall | -0.09% | -0.46% | -0.40% | -0.18% | -0.59% | -0.60% |

References

- E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AF2016

Conclusion

This contribution reports the test results of EE1-4.2, which demonstrate a good trade-off between complexity and coding performance. It is recommended to adopt this method into NNVC software.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).