| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0105-v2 |
| --- | --- |

| Title: | AHG12: Multiple Reference Area Decoder-side Intra Mode Derive |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiawei Fan, Junyan Huo, Zhenyao Zhang, Yanzhuo Ma, Fuzheng Yang, Ming Li | Tel: Email: | jw.Fan@stu.xidian.edu.cn jyhuo@mail.xidian.edu.cn zhyzhang@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn myron.li@oppo.com |
| Source: | Xidian, OPPO |  |  |

_____________________________

- Abstract

This contribution proposes a new intra prediction method based on the computation of multiple Histogram of Gradients by using different reference area candidates. One Histogram of Gradients is used to derive a set of intra modes and weights that are then combined to compute the intra prediction for current block. The simulation results on top of ECM-12.0 are as follows {for Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}:

AI {-0.10%	0.03%	0.06%	110.8%	98.5%},

RA {%, %, %, %, %, %, %}

Introduction

In ECM-12.0, when using DIMD, a Histogram of Gradients (HoG) is computed using a three-sample wide/high L-shaped template formed from already reconstructed samples. The HoG is constructed by accumulating gradient magnitudes along various directions, determined through the application of the Sobel filter. And then up to five DIMD modes are extracted based on the highest amplitudes observed within the HoG. The final prediction is generated by blending up to five predictors and the planar mode predictor or block vector-based predictor with the weights derived from the histogram of gradients.

Proposal

This contribution proposes to add a new intra prediction mode as a sub-mode of DIMD, referred to as Multiple Reference Area Decoder-side Intra Mode Derivation (MR-DIMD), which uses multiple three-sample wide/high L-shaped reference area candidates to construct Candidate Histogram of Gradients (CHoG) list. The maximum length of the CHoG list is 6. Reference area candidates are formed of already reconstructed samples. The distance between reference area candidates list and current block is {0, 3, 6, 9, 12, 15} as shown in Fig1, N=0,…,5. And then, similar to DIMD, up to five MR-DIMD modes are derived from each CHoG and are then blended together. The derivation of the modes and blending weights follows the same process to derive DIMD modes.

Fig1. The proposed scheme of reference area candidates

Bitstream Signalling

Usage of the MR-DIMD mode is signaled with a CU level Index after DIMD flag.

The “index” has various meanings:

- It indicates the intra prediction mode of the current block. When index is equal to 0, it uses DIMD mode, otherwise is MR-DIMD mode.

- It denotes the reference index of the candidate reference area.

Moreover, the MR-DIMD mode accounts for one additional RD check at the encoder side.

Experimental results

The proposed method was implemented on top of ECM-12.0 [1], tests were performed following ECM CTC [2]. The test results of the proposed method are summarized in the following Table1. Note that the encoding time and the decoding time are unreliable.

Table 1: Test result of the proposed scheme on top of ECM-12.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | 0.00% | 0.06% | 110.4% | 99.9% |
| Class A2 | -0.12% | 0.09% | 0.02% | 110.0% | 100.2% |
| Class B | -0.12% | 0.02% | 0.08% | 111.4% | 98.9% |
| Class C | -0.06% | 0.01% |  | 0.15% |  | 111.3% |  | 97.7% |
| Class E | -0.04% | 0.05% |  | -0.08% |  | 110.5% |  | 96.1% |
| Overall | -0.10% | 0.03% | 0.06% | 110.8% | 98.5% |
| Class D | -0.09% | 0.16% | 0.07% | 112.6% | 100.9% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusion

The contribution proposes a new intra prediction method termed multiple reference-area decoder-side intra mode derivation (MR-DIMD) that improves the intra prediction performance of ECM-12.0. The proposed method extends multiple reference area candidates to derive the intra modes of current block. Simulation results show the effectiveness of the proposed method. It is recommended to study the proposed method in the next round of EE.

- References

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct, 2023.

Patent rights declaration(s)

Xidian Univ. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).