| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0107-v1 |
| --- | --- |

| Title: | EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijie Bao, Nianxiang Fu, Xin Chen, Jianghao Jia, Zhenzhong Chen Wuhan University Zizheng Liu, Xiaozhong Xu, Shan Liu Tencent | Email: | {baoweijie, nianxiangfu, cinched, jiajh2021, zzchen} @whu.edu.cn {zizhengliu, xiaozhongxu, shanl} @tencent.com |
| Source: | Wuhan University, Tencent |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of EE1-3.1 based on JVET-AG0122. The focus of the EE1-3.1 test lies on the training dataset and network architecture. Specifically, the training dataset is substituted with Vimeo, compressed using VTM-11.0_NNVC-7.1 under NNVC-AI configuration (with LOP filter and NN-intra enabled). Building upon the low operation point deep reference frame generation (LDRF) network from JVET-AG0122, this contribution proposes a high operation point deep reference frame generation (HDRF) network, in which optical flows are calculated without scaling. Furthermore, this contribution retrains the DRF network from JVET-AF0208 using the new training dataset, to compare with HDRF.

Implemented on the VTM-11.0_NNVC-8.0, the experimental results under the NNVC-RA configuration (with LOP filter and NN-intra enabled) are reported as follows:

LDRF network (2900K, 69kMAC/pixel): -1.66%, -1.58%, -1.28%, EncT 108%, DecT 370%

HDRF network (2900K, 182kMAC/pixel): -2.63%, -1.83%, -1.46%, EncT 124%, DecT 763%

DRF network from JVET-AF0208 (3800K, 504kMAC/pixel): -2.94%, -2.93%, -2.72%, EncT 147%, DecT 1411%

Introduction

JVET-AG0122 [1] proposes a deep reference frame generation (DRF) method to enhance VVC inter prediction. During the encoding and decoding process, the network receives two reconstructed frames from the decoded picture buffer as inputs and generates a new frame. The generated frame will be put into the reference picture lists as an additional reference for the current encoding picture, which enhances the inter prediction.

In this contribution, the training dataset is replaced and the network architecture is further explored. The main modifications are summarized as follows:

- The training dataset of this contribution is Vimeo, which is compressed with VTM-11.0_NNVC-7.1 under the NNVC-AI configuration.

- Building upon the LDRF network from JVET-AG0122, this contribution provides the HDRF network by removing the inputs and outputs scaling of the small IFRNet module.

Proposed Method

Framework

Fig. 1: The framework of the proposed DRF method

The framework of the proposed DRF method is depicted in Figure 1. In this approach, two reconstructed frames chosen from the DPB are inputted into the deep reference frame generation network to synthesize a new frame. This synthesized frame is then added to both reference picture list0 and list1, using the same POC as the current encoding frame. It serves as an additional reference frame for subsequent encoding processes.

Network architecture

Fig. 2: The architecture of the LDRF and HDRF networks

Figure 2 illustrates the architecture of the LDRF (2900K, 69kMAC/pixel) and HDRF (2900K, 182kMAC/pixel) networks. For the LDRF network from JVET-AG0122, the inputs of the small IFRNet module are downsampled and then the resulting optical-flows are upsampled, to reduce the complexity of optical-flow calculations. This contribution removes the inputs and outputs scaling of the small IFRNet module, to provide an HDRF network. The precise computation of optical-flows contributes to the enhanced performance of the HDRF network. Compared with the DRF network (3800K, 504kMAC/pixel) from JVET-AF0208 [2], HDRF exhibits minor performance loss but offers significant complexity savings.

Network information in the training stage

Table 1: Network information for the NN-based video coding tool testing in the training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GTX 3090 |
|  | Framework | PyTorch v1.13.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch | 200 |
|  | Batch size | 48 |
|  | Loss function | L1 |
|  | Training time (for 1 model) | 51h (LDRF Network), 59h (HDRF Network) |
|  | Training data information | Vimeo |
|  | Training configurations for generating compressed training data (if different to VTM CTC) | VTM-11.0_NNVC-7.1 under NNVC-AI configuration (with LOP filter and NN-intra enabled), QP{22, 27, 32, 37, 42} |
| Optional | Number of iterations |  |
|  | Patch size | 256 × 256 |
|  | Learning rate | 3.5e-4 (3.5e-4 for IFRNet) |
|  | Learning rate update strategy | ReduceLROnPlateau |
|  | Optimizer | ADAM |
|  | Preprocessing |  |
|  | Mini-batch selection process |  |
|  | Training data update strategy |  |
|  | Other information |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  |  | LDRF Network | HDRF Network |
| Mandatory | HW environment |  |  |
|  | GPU Type | CPU only |  |
|  | Framework | SADL |  |
|  | Number of GPUs per Task | 0 |  |
|  | Number of Parameters (Each Model) | 2900K | 2900K |
|  | Total Number of Parameters (All Models) | 5800K |  |
|  | Parameter Precision (Bits) | 32 (floating) |  |
|  | Multiply Accumulate (kMAC/pixel) | 69 | 182 |
| Optional | Total Conv. Layers |  |  |
|  | Total FC Layers |  |  |
|  | Total Memory (MB) |  |  |
|  | Batch size | 1 |  |
|  | Patch size | 256x256 (with 16 boundary padding) |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage (Total) |  |  |
|  | Peak Memory Usage (per Model) |  |  |
|  | Border handling |  |  |
|  | Other information |  |  |

Results

The method has been implemented on the VTM-11.0_NNVC-8.0, and the experimental results of the LDRF and HDRF networks under NNVC-RA configuration are presented in Table 3 and Table 4, respectively. Furthermore, this contribution also provides the experimental results of the DRF network from JVET-AF0208 in Table 5, which has the same training and inference settings as the HDRF network.

As depicted in Tables 4 and 5, the HDRF network, in contrast to the DRF network from JVET-AF0208, demonstrates significantly higher speed with minimal performance degradation.

Table 3: Performance of the LDRF network (NNVC-RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.71% | -2.02% | -2.33% | -2.15% | -2.49% | -3.15% | 111% | 362% |
| Class A2 | -2.36% | -2.54% | -1.00% | -2.88% | -3.34% | -2.88% | 108% | 366% |
| Class B | -1.32% | -0.99% | -1.11% | -2.03% | -2.36% | -2.85% | 110% | 387% |
| Class C | -1.54% | -1.25% | -0.93% | -2.86% | -2.97% | -2.85% | 105% | 360% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1.66% | -1.58% | -1.28% | -2.44% | -2.75% | -2.92% | 108% | 370% |
| Class D | -1.05% | 0.14% | 0.77% | -2.14% | -1.89% | -1.61% | 104% | 349% |
| Class F | -0.57% | 0.07% | 0.05% | -1.19% | 0.03% | -0.94% | 120% | 969% |

Table 4: Performance of the HDRF network (NNVC-RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.86% | -2.28% | -2.59% | -2.19% | -3.32% | -3.27% | 129% | 744% |
| Class A2 | -3.11% | -2.89% | -1.30% | -3.56% | -4.17% | -3.25% | 123% | 751% |
| Class B | -2.30% | -1.26% | -1.31% | -2.68% | -3.12% | -3.74% | 127% | 805% |
| Class C | -3.26% | -1.41% | -0.93% | -4.33% | -3.89% | -3.87% | 116% | 737% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -2.63% | -1.83% | -1.46% | -3.20% | -3.58% | -3.58% | 124% | 763% |
| Class D | -3.41% | 0.60% | 1.33% | -4.25% | -3.42% | -2.80% | 114% | 709% |
| Class F | -1.04% | 0.04% | 0.14% | -1.53% | 0.26% | -1.23% | 147% | 2154% |

Table 5: Performance of the DRF network from JVET-AF0208 (NNVC-RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -2.03% | -3.75% | -3.72% | -2.26% | -4.82% | -4.51% | 155% | 1372% |
| Class A2 | -3.40% | -3.75% | -2.34% | -3.75% | -5.01% | -4.05% | 147% | 1382% |
| Class B | -2.51% | -2.31% | -2.28% | -2.85% | -4.33% | -4.56% | 154% | 1496% |
| Class C | -3.81% | -2.48% | -2.81% | -4.89% | -5.19% | -5.89% | 135% | 1361% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -2.94% | -2.93% | -2.72% | -3.46% | -4.80% | -4.80% | 147% | 1411% |
| Class D | -4.53% | -0.85% | -0.74% | -4.85% | -5.17% | -4.95% | 132% | 1305% |
| Class F | -1.17% | -0.68% | -0.24% | -1.72% | -1.60% | -1.67% | 188% | 4114% |

Conclusions

This contribution presents a deep reference frame generation method for VVC inter prediction enhancement. Implemented on the VTM-11.0_NNVC-8.0, the LDRF network (69kMAC/pixel) can bring -1.66%-1.58%-1.28% coding efficiency improvements under NNVC-RA configuration. Meanwhile, the HDRF network (182kMAC/pixel) can bring -2.63%-1.83%-1.46% coding efficiency improvements under NNVC-RA configuration.

The proposed DRF method shows a trade-off in terms of coding performance and complexity. It is suggested to adopt the proposed LDRF and HDRF into NNVC common software for further study.

Reference

- X. Chen, W. Bao, J. Jia, Z. Chen, Z. Liu, X. Xu, and S. Liu. “EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement”. JVET-AG0122, 33rd Meeting, by teleconference, 17–26 January 2024.

- W. Bao, X. Chen, J. Jia, Y. Zhang, Z. Chen, Z. Liu, X. Xu, and S. Liu. “EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement”. JVET-AF0208, 32nd Meeting, Hannover, DE, 13–20 October 2023.

Patent rights declaration(s)

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).