| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0108-v1 |
| --- | --- |

| Title: | AHG9: Editorial improvements of the encoder optimization information SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Jaehyun Lim Seung hwan Kim | Email: | chulkeun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

This contribution proposes some changes to the Encoder Optimization Information SEI and they are asserted to be editorial / bug fixes / clarification to the intent. The proposed changes are:

- Change the coding of syntax element eoi_object_based_idc from ue(v) to u(8).

- Clarify the semantics of eoi_privacy_protected_info_type when its value is equal to 0

- Modify the semantics of value 0 for eoi_privacy_protection_type_idc from "unknown" to "determined by the application"

Changing the coding of eoi_object_based_idc from ue(v) to u(8)

Currently, syntax element for eoi_object_based_idc in Encoder Optimization Information (EOI) SEI message is coded as ue(v). In the semantics, the value of eoi_object_based_idc is used as bit masking which means that each of its bits represent a certain meaning as shown below:

eoi_object_based_idc, when present, indicates the type of object-based optimization as specified in Table x2, where ( eoi_object_based_idc & bitMask ) not equal to 0 indicates that the object-based optimization type associated with the bitMask value in Table x2 has been applied. When eoi_object_based_idc is greater than 0 and ( eoi_object_based_idc & bitMask ) is equal to 0, the object-based optimization type associated with the bitMask value has not been applied. When eoi_object_based_idc is equal to 0, an application-defined type of object-based optimization has been applied. The value of eoi_object_based_idc shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 65 535, inclusive, for eoi_object_based_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_object_based_idc is in the range of 8 to 65 535, inclusive, decoders conforming to this version of this Specification shall ignore eoi_object_based_idc.

Table x2 – Definition of eoi_object_based_idc

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Areas outside the detected objects have been blurred prior to encoding. |
| 0x02 | Areas outside the detected objects have been encoded with coarser transform-domain quantization than the quantization used for the detected objects. |
| 0x04 | Areas outside the detected objects have been overwritten. For example, an encoding system can overwrite areas outside the detected objects with a constant sample value. |

To make the bit masking works, in our assertion, eoi_object_based_idc cannot be coded as ue(v), instead, it should be coded using a fixed-length coding.

We propose to change the coding of eoi_object_based_idc from ue(v) to u(8).

- Proposed specification text (relative to JVET-AG2034[1])

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| eoi_object_based_idc | u(8) |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| } |  |
| if( EoiPrivacyProtectionFlag ) { |  |
| eoi_privacy_protection_type_idc | u(4) |
| eoi_privacy_protected_info_type | u(8) |
| } |  |
| } |  |
| } |  |

eoi_object_based_idc, when present, indicates the type of object-based optimization as specified in Table x2, where ( eoi_object_based_idc & bitMask ) not equal to 0 indicates that the object-based optimization type associated with the bitMask value in Table x2 has been applied. When eoi_object_based_idc is greater than 0 and ( eoi_object_based_idc & bitMask ) is equal to 0, the object-based optimization type associated with the bitMask value has not been applied. When eoi_object_based_idc is equal to 0, an application-defined type of object-based optimization has been applied. The value of eoi_object_based_idc shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 255, inclusive, for eoi_object_based_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_object_based_idc is in the range of 8 to 255, inclusive, decoders conforming to this version of this Specification shall ignore eoi_object_based_idc.

Clarifying the semantics of eoi_privacy_protected_info_type.

The current semantics of eoi_privacy_protected_info_type does not include the description about the value of eoi_privacy_protected_info_type equal to 0. It only includes when eoi_privacy_protected_info_type & bitMask is not equal to 0. To avoid confusion and to align with semantics of similar syntax elements in the EOI SEI message, the meaning of value 0 should be clarified.

eoi_privacy_protected_info_type, when present, indicates the types of protected information as specified in Table x4 where ( eoi_privacy_protected_info_type & bitMask ) not equal to 0 indicates that the information type with the bitMask value in Table x4 has been protected. The value of eoi_privacy_protection_info_type shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 255, inclusive, for eoi_privacy_protected_info_type are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_privacy_protected_info_type is in the range of 8 to 255, inclusive, decoders conforming to this version of this Specification shall ignore eoi_privacy_protected_info_type.

Table x4 – Definition of eoi_privacy_protection_info_type

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Information that identifies a person is protected. For example, the face of the person. |
| 0x02 | Information that can identify vehicles is protected. For example, the license plate of the vehicle. |
| 0x04 | Information that can infer locations is protected. For example text or images on signs. |

We propose to clarify that when eoi_privacy_protected_info_type is equal to 0, it specifies that information that is protected is application-defined.

- Proposed specification text (relative to JVET-AG2034)

eoi_privacy_protected_info_type, when present, indicates the types of protected information as specified in Table x4 where eoi_privacy_protected_info_type is greater than 0 and ( eoi_privacy_protected_info_type & bitMask ) not equal to 0 indicates that the information type with the bitMask value in Table x4 has been protected. When eoi_privacy_protected_info_type is equal to 0, an application-defined type of information has been protected. The value of eoi_privacy_protection_info_type shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 255, inclusive, for eoi_privacy_protected_info_type are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_privacy_protected_info_type is in the range of 8 to 255, inclusive, decoders conforming to this version of this Specification shall ignore eoi_privacy_protected_info_type.

Modifying of the description for eoi_privacy_protection_type_idc.

In the current semantics of eoi_privacy_protection_type_idc, when its value is 0, it is represented as "unknown". We propose to change "unknown" to "determined by the application". In our assertion, such change would allow a better use of such optimization when a protection method that might not be standardized / widely use but is known and agreed to be used by encoder and decoder for the bitstream.

- Proposed specification text (relative to JVET-AG2034)

Table x3 – Definition of eoi_privacy_protection_type_idc

| eoi_privacy_protection_type_idc | Interpretation |
| --- | --- |
| 0 | Unknown. Determined by the application. |
| 1 | Blurring; personal information is blurred to make it unidentifiable. |
| 2 | Replacing; personal information is replaced with something different from the original to make it unidentifiable. |
| 3 | Masking; personal information is masked so that it cannot be identified |
| 4…15 | Reserved for future use. |

References

- S. McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullivan, Y.-K. Wang, " SEI messages for VSEI version 4 (Draft 1)", JVET-AG2034.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).