| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0111-v1 |
| --- | --- |

| Title: | AHG9: Proposed update on the encoder optimization information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Jaehyun Lim Seung hwan Kim | Email: | chulkeun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

This contribution proposes the following modifications to Encoder Optimization Information (EOI) SEI message:

- Add a constraint on the values of eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc to avoid signalling of values that do not make sense in term of optimization purpose:

- Option 1: at least eoi_for_human_viewing_idc or eoi_for_machine_analysis_idc shall be greater than 1.

- Option 2: eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc shall not be both equal to 1.

- Option 3: eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc shall not be both equal to 1 or both equal to 0.

- When the optimization type of EOI SEI message includes spatial resampling, signal a flag to identify whether the spatial resampling is an up-sampling or subsampling.

Constraint on the values of human viewing and machine analysis indications

Introduction

At the 33rd JVET meeting, it was agreed to modify the signalling of whether encoder optimization types of an Encoder Optimization Information (EOI) SEI message is targetted for human viewing and / or for machine analysis tasks from flags to indications. In other words, it was agreed to change syntax element eoi_for_human_viewing_flag to eoi_for_human_viewing_idc and syntax element eoi_for_machine_analysis_flag to eoi_for_machine_analysis_idc.

The signalling of the updated syntax elements are 2-bits indications with the meaning as summarized below:

- Value 3: the purposes for the applied optimization include human viewing / machine analysis.

- Value 2: the purpose for the applied optimization is not specifically for human viewing / machine analsys but it is suitable for the task.

- Value 1: the purpose for the applied optimization does not include (and also is not suitable) for human viewing / machine analysis.

- Value 0: it is unknown if the purpose of the optimization includes / is suitable for human viewing / machine analysis.

With the above modification, the two syntax elements become more flexible, capable of describing finer range of combinations whether the optimizations are for human viewing and / or for machine analysis tasks. However, it was also noted that some combinations may not make sense or be useful. Snippet below shows meeting notes about this aspect:

-------------snippet from JVET meeting notes starts-------------

It was asked which benefit the value 0 (unknown) would have? In combination of both syntax elements, a decoder might still draw useful conclusions. Also the combination 0/0 (both unknown) might be useful, drawing conclusions from optimization type. Combination 1/1 would not be useful.

Generally, the finer granularity is agreed to be beneficial. The proposed modification 1. was agreed for inclusion in TuC. It should be investigated if both syntax elements could be combined into one, avoiding unreasonable combinations (not with the goal of saving bits). Further study on this aspect.

-------------snippet from JVET meeting notes ends-------------

We did the study on combinations for the values of these two syntax elements to find the best way to constraint the values of these two syntax elements.

Discussion

Our guideline for constraining the values of the two indications is based on how they provide information that can be useful for decoder or any post-processing entity that uses the SEI message. Based on this, we have the following considerations:

- At the very least, both syntax elements should not be both equal to 1 (i.e., the purpose does not include and also not suitable for human viewing and for machine analsys tasks) because in such situation, no information at all is provided for decoder to make decision what to do / what can be done.

- On top of the above point, it may also be desirable that both syntax element should not be both equal to 0 (i.e., it is unknown whether the purpose of the optimization is for human vieweing and / or for machine analysis tasks). When both syntax elements have value 0, it is difficult for decoder to make decision, although, it may decision speculatively.

- Finally, it may be argued that, if the two syntax elements are designed to provide information that help decoder to make decision on what to do / what can be done, at least one of them must have value either 3 or 2.

Proposal

Based on the above considerations, we propose that one of the options below is used to constraint the values of syntax element eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc.

- Option 1

Proposed constraint: At least eoi_for_human_viewing_idc or eoi_for_machine_analysis_idc shall be greater than 1.

The text implementation of this constraint is as follows:

It is a requirement of bitstream conformance that at least eoi_for_human_viewing_idc or eoi_for_machine_analysis_idc shall have value greater than 1.

- Option 2

Proposed constraint: eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc shall not be both equal to 1.

The text implementation of this constraint is as follows:

It is a requirement of bitstream conformance that the value of eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc shall not be both equal to 1.

- Option 3

Proposed constraint: eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc shall not be both equal to 1 or both equal to 0.

The text implementation of this constraint is as follows:

It is a requirement of bitstream conformance that the value of eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc shall not be both equal to 1 or both equal to 0.

Indicating the type of spatial resampling

Problem statement & proposal

The table below represents eoi_type, with bitmask 0x04 indicating spatial resampling optimization [1]. Spatial resampling can either be subsampling or up-sampling, but currently, there is no information in the EOI SEI message to distinguish between the two.

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| 0x02 | Temporal resampling optimization |
| 0x04 | Spatial resampling optimization |
| 0x08 | Temporal quality optimization in a manner that quality fluctuates temporally |
| 0x10 | Spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |
| 0x20 | Privacy protection optimization; the pictures for which this SEI message persists have been pre-processed or encoded to protect personal information. (e.g. removal or replacing of personal identifiable information, pseudonymization, anonymization) |

It is proposed to signal flag to identify whether the spatial resampling is an up-sampling or subsampling.

Proposed specification text (relative to JVET-AG2034)

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| } |  |
| if(EoiSpatialResamplingFlag) |  |
| eoi_spatial_resampling_type_flag | u(1) |
| if( EoiPrivacyProtectionFlag ) { |  |
| eoi_privacy_protection_type_idc | u(4) |
| eoi_privacy_protected_info_type | u(8) |
| } |  |
| } |  |
| } |  |

eoi_spatial_resampling_type_flag equal to 0 specifies that the spatial resampling optimization is a subsampling operation. eoi_spatial_resampling_type_flag equal to 1 specifies that the spatial resampling optimization is an up-sampling operation.

References

- S. McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullivan, Y.-K. Wang, " SEI messages for VSEI version 4 (Draft 1)", JVET-AG2034.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).