| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0113-v1 |
| --- | --- |

| Title: | AHG16: Lightweight CFTE with Multi-Resolution support |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Zou Bolin Chen Ru-Ling Liao Jie Chen Yan Ye | Email: | {zourenjie.zrj, chenbolin.chenboli, ruling.lrl, jiechen.cj, yan.ye }@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

The contribution proposes to expand the capability of current AHG16 GFVC software to support higher sequence resolutions. A new model, developed on top of the lightweight CFTE method, is capable of processing video sequences at three different sequence resolutions including 256x256, 512x512, and 640x640. Compared with VTM-22.2, the overall coding performance impact of the proposed model for {DISTS, LPIPS, RGB444_PSNR, RGB444_SSIM, YUV420_PSNR, YUV420_SSIM} is {-50.69%, -46.92%, 0.25%, -0.7%, 1.0%, -0.2%} at resolution 256x256, {-63.67%, -57.11%, 0.00%, 1.0%, 0.0%, -0.9%} at resolution 512x512, {-63.80%, -51.91%, 0.00%, -1.1%, 0.0%, -1.0%} at resolution 640x640.

Introduction

During the 32nd JVET meeting, the newly established Ad hoc Group on Generative Face Video Compression (GFVC) took on the mandate to investigate various aspects such as software implementation, testing conditions, coordinated experiments, and interoperability studies. A unified software [2], accommodating different GFVC methods through various face video representations and enabling coding with the VVC Main 10 profile, was proposed. The results showed that GFVC could achieve significantly better reconstruction quality than the existing VVC standard at ultra-low bitrate ranges. However, this software only supports coding video sequences with a resolution of 256x256. This limitation poses a significant challenge in catering to the growing demand for higher resolution content. Expanding the software's capability to support higher resolutions like 512x512, 640x640, and beyond is imperative to meeting current and future demands.

In addressing this limitation, it is crucial to devise a solution that is not merely incremental but scalable, ensuring that the software remains compatible with evolving technological standards and user expectations. We make an improvement to the lightweight CFTE proposed in JVET-AH0109 [4] in this meeting to support higher resolutions such as 512x512 and 640x640. In JVET-AH0109, the lightweight CFTE in AHG16 software is further simplified by removing the analysis model from the decoder and selectively replacing 3x3 convolution layers with depthwise separable convolution in generative model. Figure 1 depicts the block diagram of the lightweight CFTE proposed in JVET-AH0109. For video sequence with a resolution 256x256, the base picture is encoded using VTM software, while the subsequent pictures are processed through the analysis model, yielding subsequent features with a dimensionality of [1, 4, 4]. On the decoder side, the base picture is firstly decoded using VTM software and the feature of the base picture with dimensionality of [1, 4, 4] is decoded. Subsequently, the dense motion model uses the base picture features and the decoded features of the subsequent pictures as input to generate motion information. Finally, the generative model utilizes this motion information along with the reconstructed base picture to generate the reconstructed subsequent picture with dimensions of [3, 256, 256].

Figure 1. Illustration of block diagram of the lightweight CFTE method proposed in JVET-AH0109.

Proposed method

This contribution proposes a multi-resolution lightweight CFTE method to accommodate multi-resolution processing simultaneously for 256x256, 512x512, and 640x640 resolutions. Figure 2 illustrates the block diagram of the proposed method. To achieve this, we have adapted the analysis model, dense motion model, and generative model by incorporating adaptive interpolation (a down-sampling operator consists of a gaussian blur filter followed by a nearest-neighbor down-sampling, and an up-sampling operator using bilinear interpolation). Specifically, within the analysis and dense motion models, the down-sampling operator is employed to down-sample picture to a unified size intermediate output across varying resolutions, using an adaptive down-sampling factor (i.e., 0.25 for 256x256, 0.125 for 512x512, and 0.1 for 640x640), ensuring that subsequent operations remain consistent. Consequently, through adaptive interpolation, the analysis model consistently produces features with a fixed dimensionality of [1, 4, 4], regardless of the original picture resolution. Similarly, the dense motion model is configured to produce motion information of a fixed size. In the generative model, the up-sampling operator is applied to upscale the fixed-size motion information back to the respective target resolution.

Figure 2. Illustration of block diagram of proposed multi-resolution lightweight CFTE method.

Simulation results

The proposed method is implemented on top of the AHG16 GFVC software, and are evaluated using the test conditions and evaluation procedures specified in JVET-AG2035[5]. It is important to point out that for resolutions of 512x512 and 640x640, the same test sequences are employed, with bilinear interpolation applied to obtain the test sets tailored to the respective resolutions. Table 1, Table 2, and Table 3 show the simulation results of the proposed multi-resolution lightweight CFTE method over VTM-22.2 at resolution 256x256, 512x512, 640x640, respectively. It can be seen that overall, the proposed method achieves significant performance improvements in DISTS and LPIPS over VTM-22.2 among three resolutions.

Table 1. Simulation results of the proposed method over VTM-22.2 at resolution 256x256

|  | Over VTM-22.2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -54.83% | -51.75% | 0.00% | 0.0% | 0.0% | 0.0% |
| Class B | -46.54% | -42.10% | 0.49% | -1.4% | 2.0% | -0.4% |
| Overall | -50.69% | -46.92% | 0.25% | -0.7% | 1.0% | -0.2% |

Table 2. Simulation results of the proposed method over VTM-22.2 at resolution 512x512

|  | Over VTM-22.2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -69.34% | -66.80% | 0.00% | 0.0% | 0.0% | 0.0% |
| Class B | -57.99% | -47.42% | 0.00% | 2.1% | 0.0% | -1.9% |
| Overall | -63.67% | -57.11% | 0.00% | 1.0% | 0.0% | -0.9% |

Table 3. Simulation results of the proposed method over VTM-22.2 at resolution 640x640

|  | Over VTM-22.2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -70.29% | -61.60% | 0.00% | 0.0% | 0.0% | 0.0% |
| Class B | -57.31% | -42.22% | 0.00% | -2.2% | 0.0% | -2.0% |
| Overall | -63.80% | -51.91% | 0.00% | -1.1% | 0.0% | -1.0% |

Conclusions

The contribution extends the capabilities of the lightweight CFTE method by incorporating support for higher resolution video sequences up to 512x512 and 640x640, in addition to the existing 256x256 format. The proposed multi-resolution lightweight CFTE method demonstrates the ability to efficiently process video sequences at multiple resolutions simultaneously. It is recommended to adopt this contribution to the GFVC software.

References

- B. Chen, et al., “Beyond keypoint coding: Temporal evolution inference with compact feature representation for talking face video compression,” Proc. Data Compression Conference (DCC), Mar. 2022.

- B. Chen, et al., “AHG16: Proposed Common Software Tools and Testing Conditions for Generative Face Video Compression,” JVET-AG0042, Jan. 2024.

- F. Chollet, “Xception: Deep learning with depthwise separable convolutions,” IEEE Conference on Computer Vision and Pattern Recognition (CVPR), Nov. 2017.

- R. Zou, et al., “AHG16: Removal of the analysis model on the decoder side and lightweight design,” JVET-AH0109, Apr. 2024.

- S. McCarthy and B. Chen, “Test conditions and evaluation procedures for generative face video coding,” JVET-AG2035, Jan. 2024.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).