| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0114-v1 |
| --- | --- |

| Title: | AHG 16: Updated Common Software Tools for Generative Face Video Compression |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bolin Chen, Yan Ye Goluck Konuko, Giuseppe Valenzise Shanzhi Yin, Shiqi Wang | Email: | chenbolin.chenboli@alibaba-inc.com, yan.ye@alibaba-inc.com goluck.konuko@centralesupelec.fr, giuseppe.valenzise@l2s.centralesupelec.fr shanzhyin3-c@my.cityu.edu.hk, shiqwang@cityu.edu.hk |
| Source: | Alibaba, CNRS/CentraleSupelec/Université Paris-Saclay, City University of Hong Kong |  |  |

_____________________________

- Abstract

At the 33th JVET meeting, a unified software package as well as test conditions for generative face video coding (GFVC) were established for JVET AHG16 experimentation. The AHG16 software incorporates various GFVC methods and allows coding to be performed using VVC Main 10 profile. However, it has been observed that the current GFVC software can only achieve promising face video compression in ultra-low bitrate ranges, and cannot provide a more comprehensive coverage for mid-to-high bitrate ranges. In this contribution,

- We newly incorporate two other GFVC algorithms into the AHG16 software: i) the first algorithm, Deep Animation Codec (DAC), can achieve more promising rate-distoration performance in ultra-low bitrate ranges compared with the three existing GFVC algorithms in the AHG16 software, ii) the second algorithm, Hybrid Deep Animation Codec (HDAC), can further achieve performance gains at higher bitrate ranges (10~30 kbps) compared with VVC.

- We further improve the AHG16 GFVC software code, including improving codebase readability and function format uniformity, removing CUDA dependency, and so on. The updated software package is available at https://github.com/xyzysz/GFVC_Software_v2/tree/master.

Introduction

Generative face video compression (GFVC) [2]-[7] can achieve promising rate-distortion performance especially at ultra-low bitrate ranges thanks to its ability to use compact features to achieve high-quality image reconstruction via strong inference capabilities of deep generative models [1]. Inspired by this, a series of technical proposals [8]- [25] on GFVC have been submitted to the JVET regarding carrying GFVC parameters using SEI messages, GFVC software implementation, test conditions, coordinated experimentation, interoperability study and so on. More specifically, JVET-AG0042 [17] proposed a unified software tool for experimentation with different GFVC methods and JVET-AG2035 [25] specified test conditions and software reference configurations for GFVC testing. At the 33rd meeting, it was agreed to establish GFVC software and test conditions as part of AHG16 activities.

The current AHG16 GFVC software can only achieve promising rate-distortion performance at ultra-low bitrate ranges. In this contribution, we propose to add two new GFVC algorithms[2] [7] to achieve performance gains with a more comprehensive coverage of bitrate ranges. In addition, we further optimize the AHG16 GFVC software code: i) we made the codebase more readable by separating the folder structure and formatting GFVC functions; ii) we made the code forward compatible with the latest versions of pytorch and scikit-learn; iii) we removed all hardcoded dependencies on CUDA so that the GFVC inference can be executed on CPU. iv) the current AHG16 software needs to store GFVC parameters for all frames before coding them; this is fixed in the proposed software such that GFVC parameters for each frame can be coded “on the fly”.

Proposed Algorithms

Overview

The updated software package integrates the following five representative GFVC algorithms in Table 1, including AHG16 GFVC software [17] (i.e., FOMM [3], FV2V [4] and CFTE [5]) and the two newly proposed GFVC algorithms (i.e., DAC [2] and HDAC [7]). Table 1 summarizes the implementation url that includes the training/test code and pretrained model.

Table 1 - Summary of facial representations for GFVC algorithms

| Facial Representation | Representative Method | Implementation URL |
| --- | --- | --- |
| 2D keypoints+affine transformation matrix | FOMM [3] | https://github.com/AliaksandrSiarohin/first-order-model |
| 3D keypoints+head rotation/translation matrix | FV2V [4] | https://github.com/zhanglonghao1992/One-Shot_Free-View_Neural_Talking_Head_Synthesis |
| compact feature martix | CFTE [5] | https://github.com/Berlin0610/CFTE_DCC2022 |
| 2D keypoints | DAC [2] | https://github.com/Goluck-Konuko/animation-based-codecs |
| 2D keypoints + VVC-coded frame | HDAC [7] |  |

Deep Animation Codec (DAC)

Figure 1 – Illustration of encoding and decoding processes for DAC algorithm.

As shown in Figure 1, the DAC algorithm [2] is similar to FOMM [3], but it only uses a group of 10 keypoints as compact representation without any Jacobian matrics, in order to reduce bit consumption for coding the GFVC paprameters without compromising reconstruction quality. As illustrated in Figure 1, the inputs to DAC at the encoder are the input base picture F0 and inter picture Ft (t≥1). First, if not already in YUV420, the input base picture F0 needs to go through color format conversion YUV 420 before being fed into the VVC encoder (i.e., VVC) that is compliant with VVC Main 10 Profile. In addition, the input inter picture Ft is fed into the analysis model for extracting “Keypoints” represention Kt. Subsequently, the extracted Kt is inter-predicted with the reconstructed “Keypoints” represention Kt-1 from the previous picture Ft-1 to obtain the “Keypoints” residual. Finally, quantization and entropy coding (i.e., EC) are applied to the inter-predicted “Keypoints” residual.

When the decoder receives the base-picture bitstream and the “Keypoint” representation bitstream, the face video can be reconstructed. First, the base-picture bitstream is decoded by the VVC decoder (i.e., VVC-1) that is compliant with VVC Main 10 Profile. Then, color format conversion is applied to convert it from YUV 420 to its original color space, e.g. RGB 444, to obtain the reconstructed base picture F0. Afterwards, the “Keypoint” representation Kt of inter picture can be obtained via context-based entropy decoding (i.e., EC-1) from the “Keypoint” bitstream and feature compensation with Kt-1. Finally, the decoded base picture F0 and “Keypoint” parameter Kt  of inter picture are jointly fed into the synthesis model to reconstruct face pictures Ft.

Hybrid Deep Animation Codec (HDAC)

Figure 2 – Illustration of encoding and decoding processes for HDAC algorithm.

As depicted in Figure 2, the HDAC algorithm [7] is an extension of DAC, aiming at overcoming the GFVC limiations in limited bitrate coverage and unstable reconstruction quality (e.g., long-term dependencies and background motion). As illustrated in Figure 2, HDAC augments the DAC bitstream with an auxiliary stream consisting of a very low bitrate version of the video, obtained through a conventional video codec (e.g., HEVC or VVC). Afterwards, the DAC output and auxiliary decoded video are fed into a fusion module such that the high-quality face video can be reconstructed, where the auxiliary video can provide enough information to regularize the animation and compensate for the motion estimation errors in the DAC output. This is similar to the driving picture fusion function in the GFV SEI message in TuC.

Software Optimization and Usages

In this section, we provide the details of software improvements made to the current AHG16 software.

Software improvements

The AHG16 GFVC software is improved as follows:

- We made the codebase more readable by separating the folder structure and formatting GFVC functions.

- We made the code forward compatible with the latest versions of pytorch and scikit-learn.

- We removed all hardcoded dependencies on CUDA so that the GFVC experimetation can be executed on "CPU".

- We reconfigured the coding processes to remove the need to store GFVC parameters for all frames before they are coded; this way, we can achieve on-the-fly coding of GFVC parameters for each frame.

Platform Details

As shown in Figure 3, the folder structure of AHG16 software has been updated as follows,

The GFVC source codes are relocated to “source/” path, the source codes and encoding/decoding functions of DAC and HDAC algorithms are added:

- The pretrained analysis/synthesis models and codes of the three representative GFVC algorithms are encapsulated in the “source/GFVC/{model}” folder.

- The corresponding interfaced functions regarding the encoder and decoder are defined in “source/GFVC/{model}_{encoder/decoder}.py.

- The “source/GFVC/arthmetic” and “source/GFVC/anchor” folders include the packaged tools regarding the context-adaptive arithmetic coder for feature parameter encoding and the latest VVC software VTM 22.2 for base picture encoding.

- The shell file (i.e., “source/GFVC run.sh” ) and batch execution code (i.e., “source/GFVC run.py” ) are provided to complete the encoding and decoding processes.

The configurations for GFVC coding are now encapsuled by yaml files and placed under “/config/” fold, and the configuration files of FOMM, CFTE, FV2V, DAC, HDAC are formatted accordingly:

- The configueration file are encapsulated in “config/{model}.yaml” and coding parameters can be specified in corresponding files.

Figure 3 – Illustration of folder structure for the software implementation of GFVC.

Hyper Parameters

Instead of using command line to set hyper parameters during GFVC experimentation, we encapsule all hyperparameters settings for each GFVC algorithm in corresponding yaml files. As such, these newly-defined configuration files can be better used for each test case. The entries in yaml file are specified below:

- “sequence_dir”: the path to the input test sequence

- “seq_list”: the list of sequence to be encoded

- “dataset”: the name of dataset to be encoded

- “num_frames”: the number of frames to be encoded

- “height”: height of the video to be encoded

- “width”: width of the video to be encoded

- “iframe_format”: color format for key frame encoding

- “inference_mode”: encode or decode the video

- “qp_list”: list of QPs for key frame encoding

- “quantization_factor”: number of bits for key point encoding

- “metrics”: type of metrics to be evaluated during evaluation

For HDAC model, it has additional configurations for base layer:

- “base_layer_params”:

      - “use_base_layer”: distinguish whether to augment new bitstream to enhance DAC

      - “base_codec”: determine to choose HEVC or VVC to augment new bitstream

- “qp”: base layer coding QP

- “scale_factor”: set to fraction to downscale this layer

Compile-time settings

The AHG16 software tools [17] needs to run on CUDA with Python. And in the updated software, all hardcoded dependencies on CUDA have been removed so that the GFVC inference can be executed on CPU.

Experiment

In this section, we provide the experimentation and performance analysis using the proposed DAC and HDAC algorithms.

Testing Conditions

Herein, we attempt to follow the test conditions and software reference configurations for GFVC testing specified in JVET-AG2035 [25]. For low bitrate range, test settings for VVC and for DACare as follows:

- For VVC, VTM 22.2 is used and the test is conducted under low-delay B (LDB) configuration with quantization parameters (QP) setting to 37, 42, 47 and 52.

- For DAC, its base picture is compressed as an I picture using the VTM reference software 22.2, and other pictures are further represented with a series of facial parameters and compressed via a context-adaptive arithmetic coder. The QPs of the base picture are set to 22, 32, 42 and 52.

In addition, the newly-added HDAC algorithm is designed for middle bitrate coverage, therefore the HDAC test settings are modified as follows,

- For VVC, VTM 22.2 is used and the test is conducted under low-delay B (LDB) configuration with quantization parameters (QP) setting to 32, 37, 42 and 47

- For HDAC, its base picture is compressed as an I picture using the VTM reference software 22.2 where the QP of the base picture is set to 22, and other pictures are further represented with a series of facial parameters and compressed via a context-adaptive arithmetic coder. In addition, other pictures are also compressed into auxiliary video stream using the VTM reference software 22.2 low-delay P (LDP) configuration with the QPs of 37,42,47,52.

Rate-distortion Performance

Figure 4 shows the rate distortion performance (i.e., Rate-DISTS and Rate-LPIPS) of five different generative methods compared to that of the latest VVC. We can see that the proposed DAC algorithm show better or comparable performances compared to FOMM, CFTE and FV2V algorithms in terms of perceptual-level quality measures DISTS and LPIPS.

Moreover, it is clear that HDAC can extend the current bitrate ranges of GFVC from ultra-low (<10kbps) to relatively middle (10~30kbps) with promising RD performance compared with VVC.

(a) Overall – DISTS                            (b) Overall -LPIPS

(c) Class A – DISTS                            (d) Class A -LPIPS

(e) Class B – DISTS                            (f) Class B -LPIPS

Figure 4 – Rate distortion performance of GFVC and VVC measured by two perceptual quality metrics DISTS and LPIPS in terms of overall dataset, Class A and Class B.

Figure 5 shows the rate-PSNR and rate-SSIM performance of five different GFVC methods compared to that of VVC in both RGB444 domain and YUV 420 domain. It is clear that, DAC shows better perfromance than other GFVC methods. As for HDAC, it can improve the fidelity of signal reconstruction compared with other GFVC algorithms, while the overall RD performance is still not as good as VVC.

(a) Overall – PSNR (RGB444)                         (b) Overall -SSIM (RGB444)

5

(a) Overall – PSNR (YUV420)                         (b) Overall -SSIM (YUV420)

Figure 5 – Rate distortion performance of GFVC and VVC measured by two pixel-level quality metrics PSNR and SSIM in terms of overall dataset in RGB444 and YUV420 domain.

BD-Rate Savings

As shown in Table 1 and Table 2, in comparison with the latest VVC codec, DAC can achieve 67.60% average bit-rate savings in terms of Rate-DISTS and 68.42% bit-rate savings in terms of Rate-LPIPS. Besides, HDAC can achieve 27.22% average bit-rate savings in terms of Rate-DISTS and 22.51% bit-rate savings in terms of Rate-LPIPS.

Table 1 - Average bit-rate savings of 30 face video sequences for DAC algorithm compared with the traditional VVC codec evaluated in the RGB 444 and YUV 420 domain

| Sequences |  | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A (VoxCeleb) | Seq001 | -77.08% | -77.32% | 0.00% | -67.90% | 0.00% | -63.23% |
|  | Seq002 | -74.50% | -78.04% | -60.08% | -67.83% | -52.42% | -66.77% |
|  | Seq003 | -72.23% | -74.53% | -61.25% | -68.38% | -58.55% | -65.52% |
|  | Seq004 | -69.44% | -72.90% | -48.24% | -57.32% | -38.32% | -54.88% |
|  | Seq005 | -72.05% | -73.14% | -48.65% | -60.79% | -40.21% | -59.16% |
|  | Seq006 | -71.57% | -72.57% | 0.00% | -60.84% | 0.00% | 0.00% |
|  | Seq007 | -70.93% | -69.91% | 0.00% | -53.27% | 0.00% | -53.13% |
|  | Seq008 | -70.69% | -68.44% | 0.00% | -57.77% | 0.00% | -54.48% |
|  | Seq009 | -68.82% | -67.82% | 0.00% | -46.44% | 0.00% | -46.65% |
|  | Seq010 | -76.14% | -78.33% | -50.87% | -63.71% | 0.00% | -59.77% |
|  | Seq011 | -71.43% | -75.24% | -57.78% | -65.79% | -54.09% | -64.07% |
|  | Seq012 | -74.75% | -73.79% | 0.00% | -50.14% | 0.00% | -37.49% |
|  | Seq013 | -71.62% | -74.70% | -52.77% | -65.84% | -28.76% | -62.92% |
|  | Seq014 | -68.27% | -68.77% | -50.54% | -59.60% | 0.00% | -57.36% |
|  | Seq015 | -70.50% | -71.60% | -47.21% | -59.04% | 0.00% | -58.52% |
| Class B (CFVQA) | Seq001 | -58.45% | -51.76% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq002 | -60.80% | -58.23% | 0.00% | -48.94% | 0.00% | -46.37% |
|  | Seq003 | -64.26% | -64.62% | -30.36% | -54.01% | 0.00% | -51.73% |
|  | Seq004 | -66.31% | -66.89% | 0.00% | -32.46% | 0.00% | -46.07% |
|  | Seq005 | -61.07% | -62.58% | -48.65% | -53.02% | -45.23% | -50.86% |
|  | Seq006 | -63.29% | -62.91% | 0.00% | -44.59% | 0.00% | -36.89% |
|  | Seq007 | -64.48% | -66.03% | -40.11% | -54.61% | 0.00% | -51.26% |
|  | Seq008 | -59.55% | -59.96% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq009 | -58.81% | -59.94% | -43.02% | -55.31% | 0.00% | -51.05% |
|  | Seq010 | -59.59% | -62.89% | 91.01% | -52.41% | 0.00% | -49.19% |
|  | Seq011 | -66.00% | -68.81% | 0.00% | -43.54% | 0.00% | -49.63% |
|  | Seq012 | -62.98% | -62.34% | 0.00% | -50.71% | 0.00% | 0.00% |
|  | Seq013 | -68.49% | -70.70% | -49.48% | -59.35% | -43.07% | -55.59% |
|  | Seq014 | -65.57% | -66.08% | -48.52% | -56.33% | -47.53% | -53.65% |
|  | Seq015 | -60.92% | -62.07% | -35.01% | -43.75% | -26.34% | -45.36% |
| Average | Class A | -70.01% | -70.80% | 26.13% | -57.38% | 0.00% | -61.13% |
|  | Class B | -63.36% | -64.04% | 0.00% | -46.57% | 0.00% | -43.70% |
|  | Class A+B | -66.87% | -67.71% | 116.38% | -52.29% | 0.00% | -48.91% |
| Class A |  | -72.00% | -73.14% | -31.83% | -60.31% | -18.16% | -53.60% |
| Class B |  | -62.71% | -63.05% | -13.61% | -43.27% | -10.81% | -39.18% |
| Overall |  | -67.35% | -68.10% | -22.72% | -51.79% | -14.48% | -46.39% |

Table 2 - Average bit-rate savings of 30 face video sequences for HDAC algorithm compared with the traditional VVC codec evaluated in the RGB 444 and YUV 420 domain

| Sequences |  | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A (VoxCeleb) | Seq001 | -60.95% | -56.16% | 122.17% | -0.14% | 147.45% | 21.30% |
|  | Seq002 | -62.56% | -55.79% | 82.28% | -7.06% | 111.55% | 9.61% |
|  | Seq003 | -44.34% | -40.60% | 81.34% | 55.68% | 93.78% | 48.13% |
|  | Seq004 | -55.43% | -51.80% | 140.46% | 12.66% | 177.27% | 36.94% |
|  | Seq005 | -54.63% | -45.32% | 0.00% | 10.26% | 0.00% | 20.83% |
|  | Seq006 | -40.10% | -27.79% | 146.16% | 29.01% | 174.59% | 60.34% |
|  | Seq007 | -28.85% | -18.99% | 89.53% | 31.53% | 112.15% | 46.14% |
|  | Seq008 | -31.25% | -21.69% | 173.30% | 31.80% | 0.00% | 39.04% |
|  | Seq009 | -49.04% | -38.87% | 78.27% | 23.17% | 107.32% | 33.77% |
|  | Seq010 | -61.64% | -51.52% | 97.93% | 13.92% | 116.44% | 27.56% |
|  | Seq011 | -41.83% | -50.22% | 82.00% | 13.57% | 97.95% | 33.83% |
|  | Seq012 | -67.71% | -53.80% | 300.58% | 25.30% | 0.00% | 37.55% |
|  | Seq013 | -34.30% | -43.20% | 91.32% | 31.09% | 112.21% | 51.54% |
|  | Seq014 | -9.88% | -2.48% | 203.76% | 34.51% | 0.00% | 45.02% |
|  | Seq015 | -33.33% | -34.54% | 0.00% | 31.09% | 0.00% | 35.28% |
| Class B (CFVQA) | Seq001 | -11.36% | 7.89% | 179.29% | 112.94% | 206.97% | 113.81% |
|  | Seq002 | -51.03% | -23.49% | 0.00% | 51.67% | 0.00% | 67.52% |
|  | Seq003 | -10.40% | 2.70% | 197.90% | 85.53% | 250.84% | 88.86% |
|  | Seq004 | -40.48% | -23.09% | 174.89% | 119.26% | 203.72% | 101.82% |
|  | Seq005 | 28.54% | 13.03% | 324.71% | 199.71% | 382.39% | 196.21% |
|  | Seq006 | -28.65% | -16.61% | 0.00% | 62.44% | 0.00% | 73.57% |
|  | Seq007 | -16.90% | -14.36% | 212.70% | 66.93% | 259.28% | 81.61% |
|  | Seq008 | -3.51% | 1.92% | 119.86% | 108.64% | 154.69% | 82.86% |
|  | Seq009 | 10.46% | 21.77% | 0.00% | 96.46% | 0.00% | 100.92% |
|  | Seq010 | 0.00% | 0.00% | 0.00% | 92.27% | 0.00% | 105.47% |
|  | Seq011 | -14.17% | -9.64% | 136.37% | 99.61% | 155.67% | 97.62% |
|  | Seq012 | -3.14% | -6.31% | 165.38% | 32.66% | 215.95% | 83.50% |
|  | Seq013 | 0.00% | -36.37% | 118.89% | 60.44% | 145.93% | 121.01% |
|  | Seq014 | 0.00% | 0.00% | 0.00% | 114.00% | 0.00% | 119.02% |
|  | Seq015 | 0.00% | 0.00% | 209.78% | 94.98% | 229.18% | 95.24% |
| Average | Class A | -49.30% | -42.87% | 130.20% | 21.11% | 163.81% | -33.76% |
|  | Class B | -26.12% | -12.72% | 210.26% | 93.46% | 255.09% | 100.98% |
|  | Class A+B | -41.32% | -31.37% | 167.68% | 52.93% | 207.50% | 64.83% |
| Class A |  | -45.06% | -39.52% | 112.61% | 22.43% | 83.38% | 36.46% |
| Class B |  | -9.38% | -5.50% | 122.65% | 93.17% | 146.97% | 101.93% |
| Overall |  | -27.22% | -22.51% | 117.63% | 57.80% | 115.18% | 69.20% |

Subjective Quailty Comparisons

Reconstructed face images are shown in Figure 6 to provide comparison of reconstruction quality using VVC and DAC algorithm at similar bitrates. It can be seen that at ultra-low bitrates, VVC reconstructed face images are unrecognizable, whereas DAC methods can still reconstruct vivid faces.

Figure 6 – Comparison of reconstruction quality using VVC vs. DAC methods at similar coded bitrates.

Figure 7 shows the reconstructed faces of HDAC algorithm compared to VVC. It can be seen that, HDAC can produce comparable reconstruction quality at middle-range bitrate.

Figure 7 – Comparison of reconstruction quality using VVC vs. HDAC methods at similar coded bitrates.

Conclusion

This contribution proposes to add two new GFVC algorithms (i.e., DAC and HDAC) in the AHG16 software for bitrate range extension and performance improvement. This contribution also makes improvements to the AHG16 software code.. It is suggested to adopt both elements of this proposal and use the proposed package as the new version of AHG16 software.

References

- Goodfellow I, Pouget-Abadie J, Mirza M, et al. “Generative adversarial nets,” Advances in neural information processing systems, vol. 27, 2014.

- Konuko G, Valenzise G, Lathuilière S. Ultra-low bitrate video conferencing using deep image animation[C]//ICASSP 2021-2021 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP). IEEE, 2021: 4210-4214.

- Siarohin A, Lathuilière S, Tulyakov S, et al. First order motion model for image animation[J]. Advances in Neural Information Processing Systems, 2019, 32.

- Wang T C, Mallya A, Liu M Y. One-shot free-view neural talking-head synthesis for video conferencing[C]//Proceedings of the IEEE/CVF conference on computer vision and pattern recognition. 2021: 10039-10049.

- Chen B, Wang Z, Li B, et al. Beyond keypoint coding: Temporal evolution inference with compact feature representation for talking face video compression[C]//2022 Data Compression Conference (DCC). IEEE, 2022: 13-22.

- Chen B, Wang Z, Li B, et al. Interactive Face Video Coding: A Generative Compression Framework[J]. arXiv preprint arXiv:2302.09919, 2023.

- Konuko G, Lathuilière S. and Valenzise G., "A Hybrid Deep Animation Codec for Low-Bitrate Video Conferencing," 2022 IEEE International Conference on Image Processing (ICIP), Bordeaux, France, 2022, pp. 1-5.

- Chen B, Chen J, et al. “AHG9: Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AC0088, January 2023.

- Chen B, Chen J, et al. “AHG9: Common SEI Message for Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AD0051, April 2023.

- McCarthy S, et al. “AHG9: Generative Face Video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0080, July 2023.

- Chen B, Chen J, et al. “AHG9: Common SEI Message of Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0083, July 2023.

- Chen B, Chen J, et al. “AHG9: Common Text for Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0280, July 2023.

- Han Boon Teo, Jing Yuan Thong, Karlekar Jayashree, et al. “AHG9: On Face Motion Information for Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AF0146, October 2023.

- Chen B, Yin S, Chen J, et al. “A Study on Decoder Interoperability of Generative Face Video Compression,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AF0048, October 2023.

- Chen B, Chen J, et al. “AHG9: Common text for proposed generative face video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AF0234, October 2023.

- Ye Y et al., “On VVC-assisted ultra-low rate generative face video coding,” MPEG ISO/IEC JTC 1/SC 29/WG 2 doc. no. m64987, October 2023.

- Chen B, Chen J, et al. “AHG16: Proposed Common Software Tools and Testing Conditions for Generative Face Video Compression,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0042, January 2024.

- Yin S, Chen B, et al. “AHG16: Interoperability Study on Parameter Translator of Generative Face Video Coding,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0048, January 2024.

- Yin S, Chen B, et al. “AHG16: Interoperability Study on Parameter Translator of Generative Face Video Coding,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0048, January 2024.

- Hannuksela M. M., Cricri F, Zhang H. “AHG9: On the generative face video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0087, January 2024.

- Hannuksela M. M., Cricri F, Zhang H. “AHG9: Usage of the neural-network post-filter characteristics SEI message to define the generator NN of the generative face video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0088, January 2024.

- Zou R, Liao R-L, et al. “AHG16: Depthwise separable convolution for generative face video compression,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0139, January 2024.

- McCarthy S, Yin P et al. “AHG16: Study text for common test conditions and evaluation procedures for generative face video coding (draft 1) ,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0187, January 2024.

- Chen J, Chen B et al. “AHG9/AHG16: Common text for proposed generative face video SEI message  ,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0203, January 2024.

- McCarthy S, Chen B et al. “Test conditions and evaluation procedures for generative face video coding ,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG2035, January 2024.

Patent rights declaration(s)

Alibaba, CentraleSupelec and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).