| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0115-v1 |
| --- | --- |

| Title: | AHG 9: Bit-depth truncation as optimization type in EOI SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Jaehyun Lim Seung hwan Kim Ding Ding Roman Chernyak Gilles Teniou Stephan Wenger Shan Liu | Email: | chulkeun.kim@lge.com ddding@global.tencent.com |
| Source: | LG Electronics and Tencent |  |  |

_____________________________

- Abstract

In the last two JVET meetings, bit-depth truncation technique that can be applied at encoder side has been demonstrated to be able to reduce bit-rate of the coded bitstream without significant effect to the performance of processes that may be applied to the reconstructed pictures of the bitstream such as machine analysis tasks.

This contribution proposes to include the bit-depth truncation as an optimization type that encoder may do into the current Encoder Optimization Information SEI message.

Introduction

In the 32nd JVET meeting, JVET-AF0138 described that bit-depth truncation applied at encoder can reduce bit-rate of coded bitstream without significantly affecting the performance of machine analysis tasks that may be applied to the bitstream. A new SEI message was proposed to convey information about bit-depth truncation that may be applied to the bitstream that contain the SEI message. It was asserted that the inverse operation to the bit-depth truncation can be applied, if necessary, before the bit-depth truncated pictures are used as input for machine analysis task / process.

During the discussion of JVET-AF0138, the following note was recorded:

“It appears that the benefit of this proposal would be mainly for VCM. It was negotiated in joint meetings with WG 4 that JVET would not define metadata that are specifically useful only for machine vision applications. This would be a typical case of an SEI message that should be managed within WG 4.”

After further study on this, in authors’ assertion, bit-truncation of one or more color component of the picture may be invoked at encoder for reasons more than for VCM, although currently it has shown benefit in the VCM application. To this point, designing a new SEI message for conveying bit-depth truncation only for VCM application purpose may not be the best idea. Such process can be considered as a type of encoder optimization which can be conveyed within the existing Encoder Optimization Information (EOI) SEI message.

Proposal

To include signalling of bit-depth truncation in an EOI SEI message, we propose the following:

- Assign one bit from the eoi_type syntax element of EOI SEI message to indicate bit-depth optimization.

- Further signalling number of arithmetic right shift that is applied to each color component.

Text implementation

The text implementation of the above proposed points are as shown below:

Modification to syntax table:

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| } |  |
| if( EoiPrivacyProtectionFlag ) { |  |
| eoi_privacy_protection_type_idc | u(4) |
| eoi_privacy_protected_info_type | u(8) |
| } |  |
| if( EoiBitDepthTruncationFlag ) { |  |
| eoi_num_bit_shift_luma | u(3) |
| eoi_num_bit_shift_cb | u(3) |
| eoi_num_bit_shift_cr | u(3) |
| } |  |
| } |  |
| } |  |

Modification to semantics table:

…

eoi_type indicates the types of optimization method as specified in Table x1 where ( eoi_type & bitMask ) not equal to 0 indicates that the optimization type with the bitMask value in Table x1 has been applied. When eoi_type is greater than 0 and ( eoi_type & bitMask ) is equal to 0, the optimization type with the bitMask value has not been applied. When eoi_type is equal to 0, optimization as determined by the application has been used.

Table x1 – Definition of eoi_type

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| 0x02 | Temporal resampling optimization |
| 0x04 | Spatial resampling optimization |
| 0x08 | Temporal quality optimization in a manner that quality fluctuates temporally |
| 0x10 | Spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |
| 0x20 | Privacy protection optimization; the pictures for which this SEI message persists have been pre-processed or encoded to protect personal information. (e.g. removal or replacing of personal identifiable information, pseudonymization, anonymization) |
| 0x40 | Bit-depth truncation optimization; the pictures for which this SEI message persists have been pre-processed or encoded in which the bit-depth of one or more of the colour component of the sample values is truncated. |

The variables EoiObjectBasedFlag, EoiTemporalResamplingFlag, EoiSpatialResamplingFlag, EoiTemporalQualityFlag, EoiSpatialQualityFlag, and EoiPrivacyProtectionFlag, and EoiBitDepthTruncationFlag, specifying whether eoi_type indicates the type of the optimization to include object-based optimization, temporal resampling optimization, spatial resampling optimization, temporal quality optimization, spatial quality optimization, and privacy protection optimization, and bit-depth truncation optimization, respectively, are derived as follows:

EoiObjectBasedFlag = ( ( eoi_type & 0x01 ) > 0 ) ? 1 : 0
EoiTemporalResamplingFlag = ( ( eoi_type & 0x02 ) > 0 ) ? 1 : 0
EoiSpatialResamplingFlag = ( ( eoi_type & 0x04 ) > 0 ) ? 1 : 0	(xx)
EoiTemporalQualityFlag = ( ( eoi_type & 0x08 ) > 0 ) ? 1 : 0
EoiSpatialQualityFlag = ( ( eoi_type & 0x10 ) > 0 ) ? 1 : 0
EoiPrivacyProtectionFlag = ( ( eoi_type & 0x20 ) > 0 ) ? 1 : 0
EoiBitDepthTruncationFlag = ( ( eoi_type & 0x40 ) > 0 ) ? 1 : 0

NOTE – For example, when certain highest temporal sublayers have been encoded with such coarse quantization that human viewers perceive the quality fluctuation annoying, but machine task performance is not compromised, eoi_for_human_viewing_flag and eoi_for_machine_analaysis_flag can be set equal to 0 and 1, respectively, and eoi_type can be set equal to a value that causes EoiTemporalQualityFlag to be equal to 1.

When eoi_persistence_flag is equal to 0, it is a requirement of bitstream conformance that EoiTemporalResamplingFlag shall be equal to 0 and EoiTemporalQualityFlag shall be equal to 0.

…

eoi_num_bit_shift_luma specifies the number of arithmetic bit right shift that is applied to the luma component of the sample values of the associated picture prior to encoding.

eoi_num_bit_shift_cb specifies the number of arithmetic bit right shift that is applied to the cb component of the sample values of the associated picture prior to encoding.

eoi_num_bit_shift_cr specifies the number of arithmetic bit right shift that is applied to the cr component of the sample values of the associated picture prior to encoding.

It is constrained that the values of eoi_num_bit_shift_luma, eoi_num_bit_shift_cb, and eoi_num_bit_shift_cr, if present, shall be the same throughout a CVS.

…

Patent rights declaration(s)

LG Electronics Inc. and Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).