| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0116 |
| --- | --- |

| Title: | EE2-3.1/2 related: Additional chained motion vector prediction candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangna.cynthia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

EE2-3.1 introduces a chained motion vector prediction (CMVP) into inter merge candidate list construction. CMVP candidates are inserted after HMVP candidates.

EE2-3.1a is the chained motion vector prediction without temporal buffer restriction.

EE2-3.1b is EE2-3.1a with the restriction of only motion fetched within one-CTU-row from collocated pictures.

In EE2-3.2a/b, the one-CTU-row temporal buffer constraint is removed/imposed for all relevant tools in ECM-12.0.

EE2-3.2c is a combination test of EE2-3.2a and EE2-3.1a.

EE2-3.2d is a combination test of EE2-3.2b and EE2-3.1b.

In this contribution, additional CMVP candidates are introduced.

On top of the EE2-3.2c, simulation results of the proposed method are reported as below:

RA: Overall: {0.00%, -0.09%, -0.06%, , , 100.0%, 100.0%}.

Class F: {-0.03%, -0.14%, 0.04% , , 99.9%, 100.2%}.

Class TGM: {-0.09%, -0.15%, -0.17% , , 99.7%, 100.2%}.

LB: Overall: {-0.14%, -0.33%, 0.03% , , 100.0%, 100.1%}.

Class F: {-0.14%, -0.50%, -0.78% , , 100.3%, 99.9%}.

Class TGM: {-0.14%, -0.17%, 0.03% , , 100.2%, 99.9%}.

# Introduction

In ECM, up to 4 pairwise candidates are added after the first ARMC reordering in a pairwise merge list. After deriving the pairwise candidates, they will be reordered together with the candidates in the initial merge candidate list after the first ARMC reordering under a second ARMC reordering. Then diversity reordering is performed for the reordered candidates of the initial merge candidate list and pairwise merge list after all the first and second ARMC reordering. At last, the first N (e.g., N is 10 for regular merge mode, and 4 for TM merge mode) merge candidates of the initial merge candidate list and pairwise merge list after all the reordering are selected to generate the final merge candidate list.

In EE2-3.1, CMVP candidates are inserted after HMVP candidates. CMVP may not have enough positions in the inter merge candidate list.

# Proposed methods

In this contribution, additional CMVP candidates are introduced.

It is proposed to add some CMVP candidates after pairwise candidates in the pairwise merge candidate list. The maximum size of the pairwise merge candidate list is increased. It will reserve some positions for the CMVP candidates.

# Experimental Results

The proposed method has been implemented on top of the EE-2-3.2c [1]. Testing results of the proposed method under RA/LB configurations are summarized in Table 1. The simulations were performed following the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over Test_3.2c |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.00% | -0.11% |  |  | 100.0% | 99.9% |
| Class A2 | 0.03% | -0.07% | -0.05% |  |  | 100.0% | 99.7% |
| Class B | 0.01% | -0.18% | -0.05% |  |  | 100.0% | 100.1% |
| Class C | 0.00% | -0.05% | -0.05% |  |  | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.09% | -0.06% |  |  | 100.0% | 100.0% |
| Class D | -0.02% | -0.04% | 0.01% |  |  | 100.1% | 100.0% |
| Class F | -0.03% | -0.14% | 0.04% |  |  | 99.9% | 100.2% |
| Class TGM | -0.09% | -0.15% | -0.17% |  |  | 99.7% | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over Test_3.2c |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | -0.11% | -0.34% |  |  | 100.2% | 99.9% |
| Class C | -0.18% | -0.34% | 0.28% |  |  | 100.0% | 100.2% |
| Class E | -0.14% | -0.69% | 0.32% |  |  | 99.9% | 100.2% |
| Overall | -0.14% | -0.33% | 0.03% |  |  | 100.0% | 100.1% |
| Class D | -0.18% | -0.05% | -0.21% |  |  | 99.8% | 100.0% |
| Class F | -0.14% | -0.50% | -0.78% |  |  | 100.3% | 99.9% |
| Class TGM | -0.14% | -0.17% | 0.03% |  |  | 100.2% | 99.9% |

- Conclusions

In this contribution, additional CMVP candidates are introduced. The proposed method can bring some coding gains with reasonable coding time change. It is recommended to study this method in EE2.

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/jvet-ag-ee2/ECM/-/tree/TEST_3.2?ref_type=heads

[2] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Oct. 2023.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).