| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0188-v1 |
| --- | --- |

| Title: | AHG9/AHG16: Showcase for picture fusion for generative face video SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Showcase |  |  |
| Author(s) or Contact(s): | Shiv Gehlot, Guan-Ming Su, Peng Yin, Sean McCarthy, Gary J. Sullivan (Dolby) | Email: | {pyin, smcca}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution describes a showcase for picture fusion for the generative face video SEI message included in the technologies under consideration for future extensions of VSEI. Specifically, this contribution demonstrates the case when gfv_drive_pic_fusion_flag is set equal to 1. It is demonstrated that gfv_drive_pic_fusion_flag equal to 0 could result in entanglement between foreground (face) and background. In the decoded video sequence, the background surrounding the face also moves with the face. This contribution demonstrates that gfv_drive_pic_fusion_flag equal to 1 can effectively isolate the foreground from background and improve video quality.

# Introduction

The generative face video (GFV) SEI message is included in JVET-AG2032 [1]: Technologies under consideration for future extensions of VSEI (version 3) to support generative face video coding (GFVC). Two cases are supported based on the syntax gfv_drive_pic_fusion flag, as shown in Figure 1: (a) supports the GFVC without picture fusion when gfv_drive_pic_fusion_flag=0; (b) supports GFVC with picture fusion when gfv_drive_pic_fusion_flag=1.

GFVC and the GFV SEI message are being studied in AHG16 of JVET. Test conditions and evaluation procedures for GFVC are described in JVET-AG2035. GFVC software is provided at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gfvc/gfvc_v1/. The software provides functionality for gfv_drive_pic_fusion_flag = 0. This contribution describes a showcase for gfv_drive_pic_fusion_flag = 1.

(a) gfv_drive_pic_fusion_flag=0

(b) gfv_drive_pic_fusion_flag=1

Figure 1 GFV SEI message

# GFVC fusion

## Overview of GFVC

The objective of GFVC is to significantly reduce bitrate for face video such as used in video conferencing, for example.

The framework of the GFVC software is shown in Figure 2. At the encoder, VTM compresses the base picture of face video. Subsequently, driving pictures are input to the neural network (NN) analysis models for key points/compact features extraction, which are then coded into the bitstream. At the decoder side, the decoded base picture along with decoded key points/compact features are input to the NN synthesis model to reconstruct the driving pictures.

Figure 2 Overview of encoding and decoding process of GFVC

## Current limitation of GFVC without picture fusion

One limitation we observed for the above approach is entanglement between foreground (face) and background. In the decoded video sequence, background surrounding the face also moves with the face, as shown in Figure 3.

Figure 3 Entanglement between face and background

## GFVC with picture fusion

This contribution describes the use of picture fusion to mitigate the entanglement of face and background, as illustrated in Figure 4. In this contribution, CFTE [3] is used for generative coding of the face and a static background picture is used as the fusion picture.

Figure 4 GFVC with picture fusion (CFTE face coding and a static fusion picture are used as an example)

Figure 5 Bitstream: K: base picture; B: background base picture; C: GFV SEI messages

In the example illustrated in Figure 4, the processing operations on the encoder side are as follows:

- For the base picture

  - The base picture is encoded with VTM as an IRAP picture (denoted as K in Figure 5)

  - In parallel, the base picture is input to a segmentation NN model to generate a face segment and a background segment for the base picture

  - The background segment of the base picture is input to an inpainting NN model to produce a background base picture that is encoded with VTM (denoted as B in Figure 5)

  - The regions of the base picture corresponding to the background of the face segment are filled with a constant sample value to produce a segmented base picture. For example, the constant sample value could be equal to the mean of RGB values in the face segment.

  - Compact features are extracted from the segmented base picture and conveyed in the GFV SEI message associated with the base picture (denoted as the C associated with picture K in Figure 5)

- For driving pictures

  - Each driving picture is input to a segmentation NN model to generate a face segment for the driving picture

  - The regions of the driving picture corresponding to the background of the face segments are filled with a constant sample value to produce a segmented driving picture.

  - Compact features are extracted from each segmented driving picture and conveyed in a GFV SEI message associated with the background base picture. The value of the GFV SEI message syntax element gfv_cnt is incremented by 1 for each driving picture (denoted as the series of C associated with picture B in Figure 5)

  - The syntax element gfv_drive_pic_fusion_flag is set equal to 1 in the GFV SEI message

The corresponding processing operations on the decoder side are as follows:

- For the base picture

  - The base picture is decoded for display and for input to other processes as described below

  - The background base picture is decoded for input to a fusion process

  - The decoded base picture is input to a segmentation NN model to generate a reconstructed face segment for the decoded base picture

  - The regions of the decoded base picture corresponding to the face segment are filled with a constant sample value to produce a reconstructed segmented base picture

  - The GFV SEI message associated with the base picture is parsed to obtain the compact features for the base picture

- For driving pictures

  - The GFV SEI message associated with the driving picture is parsed to obtain the compact features for the driving picture

  - The reconstructed segmented base picture and the compact features of the base picture and driving pictures are input to a motion estimation and frame generation NN model with output of generated segmented driving picture, followed by a segmentation process to produce reconstructed segmented driving pictures

  - The reconstructed segmented driving picture and decoded background base picture are input to a fusion process to produce a background stabilized driving picture for display

For the examples provided in this contribution, the segmentation NN model is described in [4], the inpainting NN model is described in [5], and alpha blending was used for the fusion process as follows:

fused_image=alpha_mask*foreground_picture+1-alpha_mask*background_picture

where alpha_mask is derived from the segmentation mask.

# Showcase demonstration of GFVC with picture fusion

Examples are provided to demonstrate the effectiveness of picture fusion for GFVC. Figure 6 shows reduced background warping for GFVC with picture fusion (right panel) compared to GFVC without picture fusion (middle panel). Additional examples are presented in the accompanying PowerPoint file .

Original              CFTE without picture fusion (QP=22)   CFTE with picture fusion (QP=22)

Figure 6 Reduced background warping for GFVC with picture fusion

# Conclusion

This contribution describes a showcase for picture fusion for the generative face video SEI message. Examples are provided to illustrate the mitigation of background warping for GFVC with picture fusion compared to GFVC without picture fusion. The examples presented are limited to fusion pictures representing static backgrounds. The fusion pictures used in the examples presented were generated by known segmentation and inpainting NN models. It is noted, however, that it would be beneficial if segmentation, inpainting, and fusion technologies were further improved, particularly for dynamic backgrounds. It is proposed that technologies for picture fusion for GFVC be further studied in AHG16.

# References

- S McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, and Hendry, “Technologies under consideration for future extensions of VSEI (version 3)”, JVET-AG2032, online meeting, Jan, 2024

- S McCarthy, B. Chen, “Test conditions and evaluation procedures for generative face video coding”, JVET-AG2032, online meeting, Jan, 2024

- B. Chen et al., “Beyond keypoint coding: Temporal evolution inference with compact feature representation for talking face video compression,” in Data Compression Conference, 2022, pp. 13–22.

- Xiangguang Chen, et al., “Robust Human Matting via Semantic Guidance,” Proceedings of the Asian Conference on Computer Vision (ACCV), 2020

- Weihuang Liu, et al., “CoordFill: Efficient High-Resolution Image Inpainting via Parameterized Coordinate Querying”, AAAI 2023.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).