| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0122-v2 |
| --- | --- |

| Title: | EE2-3.6: Reference filtering for inter-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy(Ofinno) | Tel: Email: | afilippov@ofinno.com vrufitskiy@ofinno.com |
| Source: | Ofinno LLC |  |  |

_____________________________

# Abstract

This contribution presents the simulation results of the Reference filtering for inter-prediction technique, which was studied in Test EE2-3.6. On top of ECM-12.0, the following BD-rate gain results and encoding/decoding runtimes are reportedly obtained for test EE2-3.6a {Y/U/V/EncT/DecT}: {-0.09%, -0.16%, -0.08%, 105%, 101%} for RA configuration and {-0.06%, -0.45%, -0.17%, 107%, 101%} for LDB configuration.

# Introduction

In [1] and [2], a technique of reference filtering for inter-prediction was proposed. This technique aims at improving inter-prediction mechanisms. In this contribution, we report the simulation results of the Reference filtering for inter-prediction studied in Test EE2-3.6.

# Proposed methods

In this test, a linear filter is applied to interpolated reference area to obtain predicted samples. Linear filter parameters are derived from template areas of current and reference blocks using spatially collocated samples. The test description of the studied technique is provided in [3]. In this test a linear filter is applied for uni-predicted non-affine blocks when motion vectors are coded using AMVP (non-merge) mode.
A shape of the linear filter and how this filter is applied to template area is shown in Figure 1.

Figure 1. Shape of a linear filter

When coefficients of the linear filter are derived at positions “NW”, “N”, “NE”, “W”, “E”, ”SW”, “S”, “SE”, they are constrained to be equal to each other. This constraint is introduced to guarantee symmetricity of the spatial filter, so as to avoid subsample displacements that may be introduced by this filter if it is spatially asymmetric.

# Simulation results

The proposed methods are implemented on top of ECM-12.0 and are tested under CTC for enhanced compression tool testing [4] with Random Access and Low Delay B configurations.

Table 1. Simulation results of EE2-3.6a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.26% | -0.08% | 105.8% | 100.8% |
| Class A2 | -0.10% | -0.22% | -0.17% | 104.7% | 101.3% |
| Class B | -0.08% | -0.08% | -0.08% | 105.9% | 100.4% |
| Class C | -0.08% | -0.16% | 0.00% | 104.0% | 101.9% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.16% | -0.08% | 105.1% | 101.1% |
| Class D | -0.05% | -0.37% | -0.11% | 104.3% | 100.1% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.05% | -0.51% | 0.11% | 107.3% | 102.5% |
| Class C | -0.09% | -0.30% | -0.09% | 107.2% | 99.1% |
| Class E | -0.03% | -0.56% | -0.76% | 107.6% | 100.9% |
| Overall | -0.06% | -0.45% | -0.17% | 107.3% | 101.0% |
| Class D | -0.17% | 0.35% | 0.12% | 107.1% | 100.5% |

# References

- A. Filippov, V. Rufitskiy, and K. Suverov “Non-EE2: Reference filtering for inter-prediction,” JVET-AG0194, October 2023.

- A. Filippov, V. Rufitskiy, and K. Suverov “Non-EE2: Reference filtering for inter-prediction,” JVET-AE0140, July 2023.

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu and K. Zhang, “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AG2024, Jan. 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).