| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0123-v1 |
| --- | --- |

| Title: | AHG9: On applying the SEI processing order to generated pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Nokia Technologies, Tampere, Finland | Email: | {miska.hannuksela ¦ francesco.cricri}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

When a processing chain described by an SEI processing order (SPO) SEI message includes a picture rate upsampling neural-network post-filter (NNPF), it is asserted that it is unclear which pictures the subsequent processing stages in the same processing chain apply. The input document presents use cases where the subsequent processing stages do not apply to both the current picture and all the pictures that were generated through the picture rate upsampling.

This input document proposes to add indications in the processing order nesting (PON) SEI message to indicate which ones of the candidate pictures, consisting of the current picture and the interpolated and extrapolated pictures generated in any previous processing stages, the semantics of the PON-nested SEI messages apply. Thus, for example, a PON-nested NNPFA SEI message can be indicated to apply to only the interpolated pictures but not the current picture.

Two options are proposed:

- pon_curr_pic_association_flag and pon_gen_pic_association_all_flag indicating, when equal to 1, that the PON-nested SEI messages apply to the current picture and all the generated pictures, respectively.

- In addition to the functionality of option 1, optional additions in the PON SEI message are proposed to indicate per each generated picture whether the PON-nested SEI messages apply.

# Problem statement

When a processing chain includes picture rate upsampling, it is asserted that it is unclear which pictures the subsequent processing stages in the same processing chain apply. Use cases where the subsequent processing stages do not apply to both the current picture and all the pictures that were generated through picture rate upsampling include the following:

- An interpolated picture is enhanced since the picture rate upsampling NNPF (also called the interpolator NNPF) has low complexity and produces relatively poor picture quality. Thus, a visual enhancement filter is to be applied only on the interpolated pictures, but not the cropped decoded pictures.

- Consecutive pictures in output order belong to different temporal sublayers and have different qualities in a hierarchical fashion. An interpolator NNPF generates a picture 1, 3, 5, … between each pair of cropped decoded pictures 0, 2, 4, 6, etc.  Interpolated picture 1 will likely be of high quality and needs not be filtered by a quality enhancement NNPF. Interpolated picture 3 will likely be of lower quality than interpolated picture 1, and thus is to be visually enhanced with the quality enhancement NNPF.

Additionally, it is asserted that the current design does not allow an encoder to make a decision between the base NNPF or an updated NNPF for interpolated pictures, but rather the choice between the base NNPF or an updated NNPF that is indicated for a cropped decoded picture also applies to the interpolated pictures. An example:

- An encoder finetunes a quality-enhancement NNPF based on interpolated pictures of a random access segment and sends an NNPF update through an NNPFC SEI message. Ideally, an encoder would indicate the use of the base quality-enhancement NNPF for the cropped decoded pictures and the updated NNPF for the interpolated pictures, but this is not possible with the current design.

Furthermore, it is asserted that when a processing chain includes a picture rate upsampling NNPF, the current design does not allow an encoder to indicate to which pictures in a list of generated pictures the subsequent processing stages in the same processing chain apply. Example use cases are as follows:

- An interpolator NNPF generates four pictures based on two input pictures, e.g., generates pictures 1, 2, 3, 4 based on pictures 0 and 5. It is likely that the inner-most generated pictures, e.g., generated pictures 2 and 3, are of lower quality than the other generated pictures in the list of generated pictures, e.g., than generated pictures 1 and 4. In order to enhance only the inner-most generated pictures, e.g., generated pictures 2 and 3, an encoder would need to indicate the use of a quality-enhancement NNPF for those inner-most generated pictures only.

- As a further extension of example 4, the encoder may finetune a quality-enhancement NNPF based only on the inner-most generated pictures, and thus it would need to indicate the use of the base quality-enhancement NNPF for the non-inner-most generated pictures and the use of the updated NNPF for the inner-most generated pictures.

# Overview of the proposal

This input document proposes to add indications in the PON SEI message to indicate which ones of the candidate pictures, consisting of the current picture and the interpolated and extrapolated pictures generated in any previous processing stages, the semantics of the PON-nested SEI messages apply. Thus, for example, a PON-nested NNPFA SEI message can be indicated to apply to only the interpolated pictures but not the current picture.

Two options are proposed:

- pon_curr_pic_association_flag and pon_gen_pic_association_all_flag indicating, when equal to 1, that the PON-nested SEI messages apply to the current picture and all the generated pictures, respectively.

With this option, examples 1 to 3 can be realized.

- In addition to the functionality of option 1, optional additions in the PON SEI message are proposed to indicate per each generated picture whether the PON-nested SEI messages apply.

With this option, all examples 1 to 5 can be realized.

# Proposed specification text

## Option 1: Nesting association to the current picture and/or all generated candidate pictures

| processing_order_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| pon_num_po_ids_minus1 | u(6 8) |
| pon_curr_pic_association_flag | u(1) |
| pon_gen_pic_association_all_flag | u(1) |
| for( i = 0; i <= pon_num_po_ids_minus1; i++ ) |  |
| pon_target_po_id[ i ] | u(8) |
| pon_num_seis_minus1 | u(8) |
| for( i = 0; i <= pon_num_seis_minus1; i++ ) { |  |
| pon_processing_order[ i ] | u(8) |
| sei_message( ) |  |
| } |  |
| } |  |

For each value of pon_processing_order[ i ] present in this SEI message, the following applies:

–	Let the list of pictures, denoted candGenPicList, consist of the following pictures in output order:

–	The pictures that have been generated by applying the post-processing stages of the associated SPO SEI message that have po_sei_processing_order[ i ] less than pon_processing_order[ i ] in this SEI message for the current picture and are associated with an output time that is not among the output times of the decoded pictures. When several pictures in candGenPicList have the same output time, they are ordered in the same order in candGenPicList as they appear in the NNPF output tensor.

–	Let the processed current picture be the picture that is associated with the same output time as that of the current picture and has been output by the post-processing stage of the associated SPO SEI message that has the greatest value of po_sei_processing_order[ i ] less than pon_processing_order[ i ] in this SEI message and outputs a picture that has an output time equal to that of the current picture.

pon_curr_pic_association_flag equal to 1 specifies that the semantics of the PON-nested SEI messages in this PON SEI message apply to the processed current picture. pon_curr_pic_association_flag equal to 0 specifies that the semantics of the PON-nested SEI messages in this PON SEI message do not apply to the processed current picture.

pon_gen_pic_association_all_flag equal to 1 specifies that the semantics of the PON-nested SEI messages in this PON SEI message apply to all the pictures in candGenPicList. pon_gen_pic_association_flag equal to 0 specifies that the semantics of the PON-nested SEI messages in this PON SEI message do not apply to all the pictures in candGenPicList.

It is a requirement of bitstream conformance that pon_curr_pic_association_flag + pon_gen_pic_association_all_flag shall be greater than or equal to 1.

## Option 2: Nesting association to any candidate picture

| processing_order_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| pon_num_po_ids_minus1 | u(7 8) |
| pon_association_all_flag | u(1) |
| for( i = 0; i <= pon_num_po_ids_minus1; i++ ) |  |
| pon_target_po_id[ i ] | u(8) |
| if( !pon_association_all_flag ) { |  |
| pon_curr_pic_association_flag | u(1) |
| if( !pon_curr_pic_association_flag ) |  |
| pon_gen_pic_association_all_flag | u(1) |
| if( !pon_gen_pic_association_all_flag ) { |  |
| pon_association_cnt_minus1 | ue(v) |
| for( i = 0; i <= pon_association_cnt_minus1; i++ ) |  |
| pon_association_diff_minus1[ i ] | ue(v) |
| } |  |
| while( !byte_aligned( ) ) |  |
| pon_alignment_zero_bit | u(1) |
| } |  |
| pon_num_seis_minus1 | u(8) |
| for( i = 0; i <= pon_num_seis_minus1; i++ ) { |  |
| pon_processing_order[ i ] | u(8) |
| sei_message( ) |  |
| } |  |
| } |  |

For each value of pon_processing_order[ i ] present in this SEI message, the list of pictures candGenPicList and the set of pictures candPicSet are derived as follows:

–	Let the list of pictures, denoted candGenPicList, consist of the following pictures in output order:

–	The pictures that have been generated by applying the post-processing stages of the associated SPO SEI message that have po_sei_processing_order[ i ] less than pon_processing_order[ i ] in this SEI message for the current picture and are associated with an output time that is not among the output times of the decoded pictures. When several pictures in candGenPicList have the same output time, they are ordered in the same order in candGenPicList as they appear in the NNPF output tensor.

–	Let the set of pictures, denoted candPicSet, consist of the following pictures:

–	All pictures in candGenPicList.

–	The picture that is associated with the same output time as that of the current picture and has been output the post-processing stage of the associated SPO SEI message that has the greatest value of po_sei_processing_order[ i ] less than pon_processing_order[ i ] in this SEI message and outputs a picture that has an output time equal to that of the current picture. This picture is called the processed current picture below.

pon_association_all_flag equal to 1 specifies that the semantics of each PON-nested SEI message in this PON SEI message apply individually to each picture in candPicSet. pon_association_all_flag equal to 0 specifies that the semantics of the PON-nested SEI messages in this PON SEI message may not apply to each picture in candPicSet.

pon_curr_pic_association_flag equal to 1 specifies that the semantics of the PON-nested SEI messages in this PON SEI message apply to the processed current picture. pon_curr_pic_association_flag equal to 0 specifies that the semantics of the PON-nested SEI messages in this PON SEI message do not apply to the processed current picture.

pon_gen_pic_association_all_flag equal to 1 specifies that the semantics of the PON-nested SEI messages in this PON SEI message apply to all the pictures in candGenPicList. pon_gen_pic_association_flag equal to 0 specifies that the semantics of the PON-nested SEI messages in this PON SEI message do not apply to all the pictures in candGenPicList. When pon_curr_pic_association_flag is present and equal to 1, pon_gen_pic_association_all_flag is inferred to be equal to 0.

pon_association_cnt_minus1 plus 1 specifies the count of pictures among candGenPicList to which the semantics of the PON-nested SEI messages in this PON SEI message apply.

pon_association_diff_minus1[ i ] is used to specify the i-th picture among candGenPicList to which the semantics of the PON-nested SEI messages in this PON SEI message apply. The variable assocGenPic[ j ], which specifies the j-th index of the picture among candGenPicList to which the semantics of the PON-nested SEI messages in this PON SEI message apply, is derived as follows:

assocGenPic[ 0 ] = pon_association_diff_minus1[ 0 ]
for( j = 1; j <= pon_association_cnt_minus1; j++ )		(xx)
	assocGenPic[ j ] = assocGenPic[ j − 1 ] + pon_association_diff_minus1[ j ] + 1

pon_alignment_zero_bit shall be equal to 0.

# Examples

The examples presented in Section 1 can be realized with the proposed syntax as described below.

## Example 1

Example 1: An interpolated picture is enhanced since the interpolator NNPF has low complexity and produces relatively poor picture quality. Thus, a visual enhancement filter is to be applied only on the interpolated pictures, but not to the cropped decoded pictures.

- Each picture unit for which the interpolator NNPF is activated includes a PON SEI message with a PON-nested NNPFA SEI message to activate the quality enhancement NNPF and the following syntax element values of the PON SEI message:

  - pon_association_all_flag = 0 (option 2 only)

  - pon_curr_pic_association_flag = 0

  - pon_gen_pic_association_all_flag = 1

## Example 2

Example 2: Consecutive pictures in output order belong to different temporal sublayers and have different qualities in a hierarchical fashion. An interpolator NNPF generates a picture 1, 3, 5, … between each pair of cropped decoded pictures 0, 2, 4, 6, etc.  Interpolated picture 1 will likely be of high quality and needs not be filtered by a quality enhancement NNPF. Interpolated picture 3 will likely be of lower quality than interpolated picture 1, and thus is to be visually enhanced with the quality enhancement NNPF.

- Picture unit 2 does not contain NNPFA SEI message to activate the quality enhancement NNPF.

- Picture unit 4 contains a PON SEI message with a PON-nested NNPFA SEI message and the following syntax element values of the PON SEI message:

  - pon_association_all_flag = 0 (option 2 only)

  - pon_curr_pic_association_flag = 0

  - pon_gen_pic_association_all_flag = 1

## Example 3

Example 3: An encoder finetunes a quality-enhancement NNPF based on interpolated pictures of a random access segment and sends an NNPF update through an NNPFC SEI message. An encoder indicates the use of the base quality-enhancement NNPF for the current picture and the updated NNPF for the interpolated pictures that have been generated by activating the picture rate upsampling filter for the current picture as follows:

- The current picture unit includes a PON SEI message ponA including an NNPFA SEI message that indicates the use of the base quality-enhancement NNPF for the current picture as follows:

  - pon_association_all_flag = 0 (option 2 only)

  - pon_curr_pic_association_flag = 1

  - pon_gen_pic_association_all_flag = 0 (inferred)

- The current picture unit also includes a PON SEI message ponB including an NNPFA SEI message that indicates the use of an updated quality-enhancement NNPF for the interpolated pictures as follows:

  - pon_association_all_flag = 0 (option 2 only)

  - pon_curr_pic_association_flag = 0

  - pon_gen_pic_association_all_flag = 1

## Example 4

Example 4: An interpolator NNPF generates four pictures based on two input pictures, e.g., generates pictures 1, 2, 3, 4 based on pictures 0 and 5. It is likely that the inner-most generated pictures, e.g., generated pictures 2 and 3, are of lower quality than other generated pictures in the list of generated pictures, e.g., than generated pictures 1 and 4. In order to enhance only the inner-most generated pictures, e.g., generated pictures 2 and 3, an encoder would need to indicate the use of a quality-enhancement NNPF for those inner-most generated pictures only.

This example can be addressed with Option 2.

- Each picture unit for which the interpolator NNPF is activated includes a PON SEI message with a PON-nested NNPFA SEI message to activate the quality enhancement NNPF and the following syntax element values of the PON SEI message:

  - pon_association_all_flag = 0

  - pon_curr_pic_association_flag = 0

  - pon_gen_pic_association_all_flag = 0

  - pon_association_cnt_minus1 + 1 = 2

  - pon_association_diff_minus1[ 0 ] = 1 (picture 2)

  - pon_association_diff_minus1[ 1 ] = 0 (picture 3)

## Example 5

Example 5: As a further extension of example 4, the encoder may finetune a quality-enhancement NNPF based only on the inner-most generated pictures, and thus it would need to indicate the use of the base quality-enhancement NNPF for the non-inner-most generated pictures and the use of the updated NNPF for the inner-most generated pictures.

This example can be addressed with Option 2.

- The current picture unit includes a PON SEI message ponA including an NNPFA SEI message that indicates the use of the base quality-enhancement NNPF for generated pictures 1 and 4 as follows:

  - pon_association_all_flag = 0

  - pon_curr_pic_association_flag = 0

  - pon_gen_pic_association_all_flag = 0

  - pon_association_cnt_minus1 + 1 = 2

  - pon_association_diff_minus1[ 0 ] = 0 (picture 1)

  - pon_association_diff_minus1[ 1 ] = 2 (picture 4)

- The current picture unit also includes a PON SEI message ponB including an NNPFA SEI message that indicates the use of an updated quality-enhancement NNPF for generated pictures 2 and 3 as follows:

  - pon_association_all_flag = 0

  - pon_curr_pic_association_flag = 0

  - pon_gen_pic_association_all_flag = 0

  - pon_association_cnt_minus1 + 1 = 2

  - pon_association_diff_minus1[ 0 ] = 1 (picture 2)

  - pon_association_diff_minus1[ 1 ] = 0 (picture 3)

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).