| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0124-v1 |
| --- | --- |

| Title: | AHG9: Latency signalling in the SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Nokia Technologies, Tampere, Finland | Email: | {miska.hannuksela ¦ francesco.cricri}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Neural network post-filters (NNPFs) may use multiple input pictures, may generate pictures through temporal interpolation or extrapolation, and need not output an NNPF output picture for each input picture. Multiple NNPFs may be cascaded into a processing chain using the SEI processing order (SPO) SEI message. It is asserted that it may not be easy for a decoding system to conclude the latency required by a processing chain, because the latency depends on how NNPFs are activated by NNPFA SEI messages.

This contribution proposes to include po_sei_max_latency as an optional syntax element in the SPO SEI message. po_sei_max_latency specifies the maximum latency required for the processing chain specified by this SPO SEI message in terms of the maximum number of pictures that follow any cropped decoded picture picA in the current layer in output order and are needed as input to at least one processing stage of the processing chain to derive a picture that results from the processing chain and has the same output time as that of picA or any picture preceding the next cropped decoded picture following picA in output order, if any.

It is asserted that when the count of cropped decoded pictures output by the decoder is greater than or equal to po_sei_max_latency, the decoding system can perform the processing chain indicated by the SPO SEI message with all required input pictures being available.

# Problem Statement

Figure 1 presents an example of a processing chain that has the following two processing stages:

- Quality enhancement NNPF, which takes three consecutive cropped decoded pictures, in output order, as input and filters the mid-most picture. The quality enhancement NNPF is activated so that the mid-most input picture has a lower quality than the surrounding input pictures and hence the quality of the mid-most input picture is enhanced to be closer to the quality of the surrounding input pictures.

- Picture rate upsampling NNPF, which takes two consecutive cropped decoded or processed pictures, in output order, as input and generates one intermediate picture as output.

Figure 1 – A processing chain of a quality enhancement NNPF followed by a picture rate upsampling NNPF.

Since picture 1 is generated based on the processed picture 2 as input, and the processed picture 2 is filtered by taking cropped decoded pictures 0, 2, and 4 as input, the post-processing involves a latency increase of two pictures in output order. In other words, the post-processing can be performed in an uninterrupted manner with a latency of two additional pictures following the decoder output.

It is asserted that it may not be easy for a decoding system to conclude the latency required by a processing chain, because the latency depends on how NNPFs are activated by NNPFA SEI messages. For instance, in the presented example each NNPF individually has a latency of one picture (i.e., the next picture in output order is needed to produce the filtering result) but the presented way of activating the NNPFs causes a total latency of two pictures.

It is asserted that it is important for a decoding system to know the latency that is required for buffering cropped decoded pictures to perform the processing chain so that no playback pauses or no use of wrong input pictures to a processing stage are caused due to too short initial buffering.

It is therefore proposed to add signalling of the latency to the SPO SEI message.

# Proposal

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(7 8) |
| po_sei_latency_flag | u(1) |
| if( po_sei_latency_flag ) |  |
| po_sei_max_latency | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

po_sei_latency_flag equal to 1 specifies that po_sei_max_latency is present. po_sei_latency_flag equal to 0 specifies that po_sei_max_latency is not present and the latency caused by the processing chain specified by this SPO SEI message is unknown or unspecified.

po_sei_max_latency specifies the maximum latency required for the processing chain specified by this SPO SEI message in terms of the maximum number of pictures that follow any cropped decoded picture picA in the current layer in output order and are needed as input to at least one processing stage of the processing chain to derive a picture that results from the processing chain and has the same output time as that of picA or any picture preceding the next cropped decoded picture following picA in output order, if any.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).