| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, 17-24 April 2024 | Document: JVET-AH0126-v1 |
| --- | --- |

| Title: | [AHG9]: Neural network post-filter for recovering images from energy-aware images |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | C.H. Demarty F. Aumont L. Blondé E. Reinhard O. Le Meur | Email: | {claire-helene.demarty, franck.aumont, laurent.blonde, erik.reinhard, olivier.lemeur}@interdigital.com |
| Source: | InterDigital |  |  |  |

_____________________________

# Abstract

The neural-network post-filter characteristics (NNPFC) SEI message includes the nnpfc_purpose syntax element, aiming to define and to trigger one or more neural-network post-filters. Several purposes have been defined such as picture rate upsampling, bit depth upsampling or colourization.

This contribution proposes to add a new purpose, called reverse energy-aware reconstruction. It is assumed that the received content has been preprocessed by a preprocessing method (e.g., [2,3,4]), so that its energy consumption is reduced when displayed on a screen. The amount of reduction R is defined at the encoder side; it represents the energy reduction ratio in percentage and is transmitted to configure the NNPFC. This new post-filter aims to recover from the decoded image an image close to the original image this image having been preprocessed prior to encoding to consume less energy when displayed onscreen.

# Introduction

Neural network post filter (NNPF) SEI messages [1] have 6 different purposes: general visual quality improvement, chroma upsampling , resolution resampling (increasing or decreasing the width or height), picture rate upsampling, bit depth upsampling, colourization, temporal extrapolation and those combinations.

This contribution extends the signification of the nnpfc_purpose metadata field of the post- filter. The proposed purpose, called reverse energy-aware reconstruction, consists in recovering from the decoded image an image close to the original image, this image having been preprocessed prior to encoding to consume less energy when displayed onscreen (see [4] for a benchmark of energy-aware methods, i.e., methods that modify images so that they consume less when displayed onscreen). The amount of energy recovery the method can achieve is equal to the amount of reduction R that has been applied during the preprocessing operation. The proposed contribution is illustrated in Figure 1.

Figure 1: Flow chart of the proposed contribution illustrating the new purpose “reverse energy-aware reconstruction”

By recovering an image close to the original image, the quality of experience is increased as well as the energy consumption of the display. This post-filter is therefore well tailored to use cases for which the energy consumption at the end-users (mostly related to the display) is not constrained.

# Proposal

A new value is added to the nnpfc_purpose field to indicate the activation of the process to recover an image with a quality close to that of the input content, at the expense of more energy consumption.

| bitMask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x40 | Temporal extrapolation (i.e., generating one or more future pictures) |
| 0x80 | Reverse energy aware reconstruction (i.e., reconstructing an image with a quality close to that of the original image while increasing the energy consumption) |

Table 1: definition of nnpfc_purpose

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 1, where ( nnpfc_purpose & bitMask ) not equal to 0 indicates that the NNPF has the purpose associated with the bitMask value in  Table 1.

The value of nnpfc_purpose shall be in the range of 0 to 127255, inclusive, in bitstreams conforming to this edition of this document. Values of 128256 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 128256 to 65 535, inclusive.

The SEI message syntax is therefore modified as followed (modifications are highlighted in yellow):

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( nnpfc_num_input_pics_minus1 > 0 ) { |  |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_filtering_flag[ i ] | u(1) |
| nnpfc_absent_input_pic_zero_flag | u(1) |
| } |  |
| if( ChromaUpsamplingFlag ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ColourizationFlag ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ResolutionResamplingFlag ) {Est ce |  |
| nnpfc_pic_width_num_minus1 | ue(v) |
| nnpfc_pic_width_denom_minus1 | ue(v) |
| nnpfc_pic_height_num_minus1 | ue(v) |
| nnpfc_pic_height_denom_minus1 | ue(v) |
| } |  |
| if(ReverseEnergyAwareFlag) { |  |
| nnpfc_energy_recovering_ratio | u(7) |
| } |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| if( nnpfc_inp_order_idc != 1 ) |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_inp_order_idc > 0 ) |  |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| if( nnpfc_out_order_idc != 1 ) |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_out_order_idc != 0 ) |  |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_matrix_coeffs | u(8) |
| nnpfc_full_range_flag | u(1) |
| } |  |
| } |  |
| if( nnpfc_out_order_idc > 0 ) |  |
| nnpfc_chroma_loc_info_present_flag | u(1) |
| if( nnpfc_chroma_loc_info_present_flag ) |  |
| nnpfc_chroma_sample_loc_type_frame | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| if( nnpfc_inp_order_idc != 1 ) |  |
| nnpfc_luma_padding_val | ue(v) |
| if( nnpfc_inp_order_idc != 0 ) { |  |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| } |  |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| nnpfc_num_metadata_extension_bits | ue(v) |
| if( nnpfc_num_metadata_extension_bits > 0 ) |  |
| nnpfc_reserved_metadata_extension | u(v) |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

Semantic:

The variables ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, ColourizationFlag, TemporalExtrapolatingFlag, and ReverseEnergyAwareFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, colourization, temporal extrapolation, and reverse the energy-aware reconstruction respectively, are derived as follows:

ChromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
ResolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
PictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(75)
BitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
ColourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
TemporalExtrapolationFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : 0

ReverseEnergyAwareFlag = ( ( nnpfc_purpose & 0x80 ) > 0 ) ? 1 : 0

nnpfc_energy_recovering_ratio, when ReverseEnergyAwareFlag is equal to 1, specifies the amount of energy recovery the post-filter should achieve. Its value is between 0 and 100, coded over 7 bits.

# Conclusion

In this contribution, a new NNPFC purpose is proposed. It consists in recovering from the decoded image an image close to the original image, this image having been preprocessed prior to encoding to consume less energy when displayed onscreen.  This contribution offers not only new service opportunities but is also timely, as the EU Energy Efficiency Directive [5] identifies the Information & Communication Technology (ICT) sector as an energy-hungry sector [6]. Indeed, this new purpose assumes that an energy-aware video, i.e. consuming less energy when displayed, is transmitted, leading to significant energy savings [4]. Thanks to the proposed purpose, it is possible to recover an image close to the original image [2] in case the energy consumption at the decoder side is relaxed.

It is therefore suggested to include this new purpose in the VSEI specification.

# References

[1] 		S. McCarthy, T. Chujoh, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, "Additional SEI messages for VSEI (Draft 4), " JVET-AD2006, April, 2023

[2] 		Le Meur, Olivier, and Claire-Hélène Demarty. "Invertible Energy-Aware Images. " IEEE Signal Processing Letters (2023).

[3] 		Le Meur, Olivier, Claire-Hélène Demarty, and Laurent Blondé. "Deep-learning-based energy aware images." 2023 IEEE International Conference on Image Processing (ICIP). IEEE, 2023.

[4]		Sampaio, Emmanuel, Claire-Hélène Demarty, and Olivier Le Meur. "How to make images less power-hungry: An objective benchmark study." Proceedings of the Second International ACM Green Multimedia Systems Workshop. 2024.

[5]		Energy Efficiency Directive (europa.eu)

[6]		Digitalisation of the energy systems (europa.eu)

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).