| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0127-v1 |
| --- | --- |

| Title: | AHG9/AHG16: The SEI message design for scalable representation and layered reconstruction for generative face video compression |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Bolin Chen,Yan Ye, Ru-Ling Liao (Alibaba) Shanzhi Yin, Shiqi Wang (CityU) | Email: | {jiechen.cj, chenbolin.chenboli, yan.ye, ruling.lrl}@alibaba-inc.com shanzhyin3-c@my.cityu.edu.hk shiqwang@cityu.edu.hk |
| Source: | Alibaba and City University of Hong Kong |  |  |

_____________________________

- Abstract

The generative face video (GFV) SEI message was included in the technologies under consideration for future extension of VSEI (version 3) in last meeting. And in this contribution, the SEI message design of generative face video compression (GFVC) was extended to support the scalable representation and layered reconstruction of GFVC. The following two options are provided in the contribution.

- Option 1: extend the current GFV SEI message by introducing a gating flag to control whether to additionally signal the enhancement layer features

- Option 2: introduce a new SEI message named generative face video enhancement (GFVE) SEI message to signal the enhancement layer features

Introduction

Versions of the GFV SEI message have been proposed sine the 29th JVET meeting (JVET-AC0088 [1], JVET-AD0051 [2], JVET-AE0080[3], JVET-AE0083[4], JVET-AE0280[5], JVET-AF0234 5[6] and JVET-AG0203[7] ) and was adopted in the draft 3 of technologies under consideration for future extensions of VSEI [8] at last meeting.

In the current GFV SEI message, facial parameters of a variety of feature representations including 2D keypoints, 2D landmarks, 3D keypoints, facial semantics, and other formats can be carried. And a facial parameter translator neural network may be signaled to convert the signaled facial parameters into the fixed format of the parameters which can be used by the generative neural network in decoder side to generate the face pictures.

In this meeting, JVET-AH0110 [9] proposed a scalable representation and layered reconstruction (SRLR) for GFVC to extend the bitrate and quality range that the methods in the current AHG16 software can cover. It was reported in JVET-AH0110 that the proposed SRLR scheme can extend the quality and bitrate ranges for all three GFVC methods supported in the AHG16 software for all quality metrics under the GFVC test conditions. The proposed SRLR scheme consists of a base layer and an enhancement layer, of which the base layer is the current GFVC method supported in the AHG16 software and the facial parameters can be signalled in the current GFV SEI message. To support SRLR scheme, the facial parameters of enhancement layer need to be additionally signaled.

Thus, in this contribution, the SEI message design of GFVC was extended to support additional signaling of enhancement layer facial parameters. By doing this, the proposed SRLR scheme can be supported with SEI message approach.

Proposal

To support SRLR scheme of GFVC by additionally signaling the enhancement layer features, two options are provided in this contribution.

option 1: extend the current GFV SEI message

In the option 1, the current GFV SEI message is extended to support signaling of enhancement layer facial parameters. So, in this option, the base layer facial parameters and the enhancement layer facial parameters are signaled in the same SEI message. To make the enhancement layer signaling optional, a gating flag is introduced. The enhancement layer facial parameters are only signaled when the gating flag is on. At the decoder side, when the gating flag is on, after decoding the enhancement layer parameters, the decoder may use the enhancement neural network to improve the quality of the face picture generated by the generative model (i.e., the output of the base layer).

The proposed text changes are in section 3.1.

option 2: introduce a new SEI message named GFVE SEI message

In this option, a new SEI message named generative face video enhancement SEI message was introduced for enhancement layer facial parameters signaling. So, in this option, the base layer facial parameters and the enhancement layer facial parameters are signalled in two different SEI messages. To identify the associated GFV SEI message for each GFVE SEI message, gfv_id and gfv_cnt of the associated GFV SEI message are signaled in the GFVE SEI message.

The proposed text changes are in section 3.2.

The proposed text changes

The proposed text for option 1

The text changes are highlighted with yellow.

Table 1 - generative face video SEI message syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| … |  |
| gfv_enhance_info_present_flag | u(1) |
| if(gfv_enhance_info_present ) { |  |
| gfv_enhance_matrix_element_precision_factor | ue(v) |
| gfv_num_enhance_matrice_minus1 | ue(v) |
| for(i=0; i <= gfv_num_enhance_matrice_minus1; i++){ |  |
| gfv_enhance_matrix_height_minus1[ i ] | ue(v) |
| gfv_ enhance_matrix_width_minus1[ i ] | ue(v) |
| for( j = 0; j <= gfv_enhance_matrix_height_minus1[ i ]; j++ ) |  |
| for( k = 0; k <= gfv_enhance_matrix_width_minus1[ i ]; k++ ) { |  |
| gfv_enhance_matrix_element[ i ][ j ][ k ] | u(1) |
| if( !gfv_enhance_matrix_element[ i ][ j ][ k ]) |  |
| gfv_enhance_matrix_element_sign_flag[ i ][ j ][ k ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( gfv_nn_present_flag ) |  |
| if( gfv_nn_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| gfv_nn_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

The generative face video (GFV) SEI message indicates facial parameters and specifies a facial parameter translator network, denoted as TranslatorNN( ), that may be used to convert various formats of facial parameters signaled in the SEI message into a fixed format of parameters, and a face picture generator neural network, denoted as GenerativeNN( ), that may be used to generate output pictures using the fixed format of facial parameters and previously decoded output pictures. Optionally, a face picture enhancer EnhanceNN() may be used to enhance the pictures generated by the GenerativeNN().

NOTE 1 – Facial parameters could be determined from source pictures prior to encoding. Such source pictures may be referred to as driving pictures.

NOTE 2 – Previously decoded output pictures input to GenerativeNN( ) may be a base picture (a decoded output picture that provides the reference texture from which the face pictures may be generated) and, optionally, a picture that can be fused by GenerativeNN( ) to improve background texture and facial details. When the current picture is not a base picture, the GFV SEI message may be used to generate a face picture based on the previously decoded base picture, the facial parameters conveyed by the GFV SEI message, and, optionally, the current decoded picture for fusion pupose.

NOTE 3 – After GenerativeNN() generates the face picture, if enhancement layer information is present, enhanced face pictures may be obtained by feeding the generated face pictures from GenerativeNN( ) and  corresponding enhancement layer information to EnhanceNN( ).

…

gfv_enhance_info_present_flag equal to 1 indicates the enhancement layer matrix is present, gfv_enhance_info_present_flag equal to 0 indicates the enhancement layer matrix is not present.

gfv_matrix_element_precision_factor indicates the quantization factor of enhancement matrix elements.

gfv_num_enhance_matrices_minus1 plus 1 specifies the number of enhancement matrices signalled in the SEI message. The value of gfv_num_enhance_matrices_minus1 shall be in the range of 0 to 216 – 1, inclusive.

gfv_enhance_matrix_height_minus1[ i ] plus 1 indicates the height of the i-th enhancement matrix.

gfv_ enhance_matrix_width_minus1[ i ] plus 1 indicates the width of the i-th enhancement matrix.

gfv_enhance_matrix_element[ i ][ j ][ k ] is used to derive the value of the element at position (k, j) of the i-th matrix.

gfv_enhance_matrix_element_sign_flag[ i ][ j ][ k ] indicates the sign of the matrix element at position (k, j) of the i-th matrix.

The variable matrixEnhanceElementVal[ i ][ j][ k ] representing the value of the matrix element at position (k, j) of the i-th matrix is derived as follows:

matrixEnhanceElementVal[ i][ j ][ k ] = 1-2*gfv_enhance_matrix_element_sign_flag[ i ][ j ][ k ]*gfv_enhance_matrix_element[ i ][ j ][ k ]1<< gfv_enhance_matrix_element_precision_factor

For a particular gfv_id value, the following process is used in increasing order of gfv_cnt to generate a video picture per each GFV SEI message that has gfv_base_pic_flag equal to 0 and a unique value of gfv_cnt within a picture unit:

DeriveSigParam( ) 
TranslatorNN (sigKeyPoint , sigMatrix) 
DeriveInputTensors( ) 
if( gfv_base_pic_flag == 0 && gfv_drive_pic_fusion_flag == 0) {
  if(ChromaFormatIdc == 0 ) 
    GenerativeNN( inputBaseY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
  else 
    GenerativeNN( inputBaseY, inputBaseCb, inputBaseCr, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
}
else if(gfv_base_pic_flag == 0 && gfv_drive_pic_fusion_flag == 1) {
  if(ChromaFormatIdc == 0 ) 
    GenerativeNN( inputBaseY, inputDriveY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
  else 
    GenerativeNN( inputBaseY, inputBaseCb, inputBaseCr, inputDriveY, inputDriveCb, inputDriveCr , inputBaseKeyPoint, inputBaseMatrix,, inputDriveKeyPoint, inputDriveMatrix) 
}
if(gfv_enhance_info_present_flag){
  if(ChromaFormatIdc == 0 ) 
    EnhanceNN( inputBaseY, genY, inputEnhanceMatrix) 
  else 
    EnhanceNN ( inputBaseY, inputBaseCb, inputBaseCr, genY, genCb, genCr, inputEnhanceMatrix) 
StoreOutputTensors( )

…

The process DeriveInputTensors( ) for deriving the inputs of GenerativeNN ( ) is specified as follows:

…
When gfv_enhance_info_present_flag is equal to 1, the enhancement matrices are derived as follows:

if( gfv_enhance_info_present_flag ) {
  for ( i = 0; i <= gfv_num_enhance_matrices_minus1; i++ ) {
    for ( j = 0; j <= gfv_enhance_matrix_height_minus1[ i ]; j++) {
      for( k = 0; k <= gfv_enhance_matrix_width_minus1[ i ]; k++ ) {
        inputEnhanceMatrix[ i ][ j ][ k ] = matrixEnhanceElementVal[ i ][ j][ k ] 
      }
    }
  }
}

….

EnhanceNN ( ) is a process to enhance the sample values of an generated picture corresponding to a driving picture. It is only invoked when gfv_enhance_info_present_flag is equal to 1. Input values to EnhanceNN( ) and output values from EnhanceNN( ) are real numbers.

Inputs to EnhanceNN() are:

- When ChromaFormatIdc is equal to 0: inputBaseY, genY, inputEnhanceMatrix

- When ChromaFormatIdc is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, genY, genCb, genCr, inputEnhanceMatrix

Outputs of EnhanceNN( ) are:

- A luma sample array enhanceY

- When ChromaFormatIdc is not equal to 0, two chroma sample arrays enhanceCb and enhanceCr.

The process StoreOutputTensors( ) for deriving the output is specified as follows:

when gfv_base_pic_flag is equal to 0, the output sample array outYPic[ x ][ y ], outCbPic[ x ][ y ], and outCrPic[ x ][ y ] are derived as follows:

for(x=0; x< CroppedWidth; x++){
  for(y=0; y< CroppedHeight; y++){
    outputYPic[ x ][ y ] = gfv_enhance_info_present_flag ? OutY(enhanceY[ x ][ y ]) : OutY( genY[ x ][ y ] )
  }
}

if(ChromaFormatIdc != 0) {
  for(x=0; x< CroppedWidth/ SubWidthC; x++){
    for(y=0; y< CroppedHeight/ SubHeightC; y++){
      outputCbPic[ x ][ y ] = gfv_enhance_info_present_flag ? OutC(enhanceCb[ x ][ y ]) : OutC( genCb[ x ][ y ] )
      outputCrPic[ x][ y ] = gfv_enhance_info_present_flag ? OutC(enhanceCr[ x ][ y ]) : OutC( genCr[ x ][ y ] )
    }
  }
}

…

where the functions OutY( ) and OutC( ) are specified as follows:

OutY( x ) = Clip3( 0, ( 1  <<  BitDepthY ) – 1 , x * ( ( 1  <<  BitDepthY ) – 1 ) 
OutC( x ) = Clip3( 0, ( 1  <<  BitDepthC ) – 1 , x * ( ( 1  <<  BitDepthC ) – 1 )

The proposed text for option 2

The following text are newly added.

Table 2 - generative face video enhancement SEI message syntax

| generative_face_video_enhancement ( payloadSize ) { | Descriptor |
| --- | --- |
| gfve_id | ue(v) |
| gfve_gfv_id | ue(v) |
| gfve_gfv_cnt | ue(v) |
| gfve_nn_present_flag | u(1) |
| if( gfve_nn_present_flag ) { |  |
| gfve_nn_base_flag | u(1) |
| gfve_nn_mode_idc | ue(v) |
| if( gfve_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfve_nn_reserved_zero_bit_a | u(1) |
| gfve_nn_tag_uri | st(v) |
| gfve_nn_uri | st(v) |
| } |  |
| } |  |
| gfve_matrix_element_precision_factor | ue(v) |
| gfve_num_matrices_minus1 | ue(v) |
| for(i=0; i <= gfve_num_matrices_minus1; i++){ |  |
| gfve_matrix_height_minus1[ i ] | ue(v) |
| gfve_matrix_width_minus1[ i ] | ue(v) |
| for( j = 0; j <= gfve_matrix_height_minus1[ i ]; j++ ) |  |
| for( k = 0; k <= gfve_matrix_width_minus1[ i ]; k++ ) { |  |
| gfve_matrix_element[ i ][ j ][ k ] | u(1) |
| if( !gfve_matrix_element[ i ][ j ][ k ]) |  |
| gfve_matrix_element_sign_flag[ i ][ j ][ k ] | ue(v) |
| } |  |
| } |  |
| if( gfve_nn_present_flag ) |  |
| if( gfve_nn_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfve_nn_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| gfve_nn_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

The generative face video enhancement (GFVE) SEI message indicates enhancement facial parameters and specifies a enhancement network, denoted as EnhancerNN( ), that may be used to enhance the visual quality of the face pictures generated with GFV SEI message.

NOTE 1 – Enhancement facial parameters could be determined from source pictures prior to encoding.

NOTE 2 – When the current picture is not a base picture, the GFV SEI message may be used to generate a face picture based the facial parameters conveyed by the GFV SEI message, and the GFVE SEI message may be further used to enhance the generated face picture to improve the visual quality.

Use of this SEI message requires the definition of the following variables:

- Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

- Luma sample array baseCroppedYPic and chroma sample arrays baseCroppedCbPic and baseCroppedCrPic for a decoded output picture, denoted as BasePicture, corresponding to a base picture.

- Luma sample array genCroppedYPic and chroma sample arrays genCroppedCbPic and genCroppedCrPic for a generated picture with associated GFV SEI message, denoted as GenPicture, corresponding to a driving picture.

- Bit depth BitDepthY for the luma sample array of the input pictures.

- Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

- A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

gfve_id contains an identifying number that may be used to identify GFVE SEI message and specify a neural network that may be used as EnhancerNN( ). The value of gfve_id shall be in the range of 0 to 232− 2, inclusive. Values of gfve_id from 256 to 511, inclusive, and from 231 to 232− 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering a GFVE SEI message with gfve_id in the range of 256 to 511, inclusive, or in the range of 231 to 232− 2, inclusive, shall ignore the SEI message.

NOTE – Different values of gfve_id in different GFVE SEI messages could be used to identify different enhancement model that may be used to enhance the face picture.

gfve_gfv_id and gfve_gfv_cnt specifies gfv_id and gfv_cnt of the GFV SEI message used to generate the face picture that the current GFVE SEI message is to enhance.

For a GFVE SEI message, the following applies:

- The associated GFV SEI message is a GFV SEI message in a same picture unit with gfv_id equal to gfve_gfv_id, gfv_cnt equal to gfve_gfv_cnt. The GFVE SEI message shall be present in the same picture unit with the associated GFV SEI message. And the associated GFV SEI message shall be precede the GFVE SEI message in the decoding order. When the associated GFV SEI message is not present in the picture unit containing the GFVE SEI message, the GFVE SEI message shall be ignored.

- When a GFVE SEI message is the first GFVE SEI message with a particular value of gfve_id is present in an IRAP picture unit or in a picture unit that follows IRAP picture unit in output order and is not preceded in output order by any picture unit that follows the IRAP picture unit in output order and has a GFVE SEI message with that particular value of gfve_id, gfve_nn_present_flag shall be present and equal to 1.

- If a GFV SEI message GA is associated with a GFVE SEI message GEA, and a GFV SEI message GB is associated with a GFVE SEI message GEB, and GFV SEI message GA precede the GFV SEI message GB in decoding order, the GFVE SEI GEA shall precede GFVE SEI message GEB in decoding order.

- If a GFVE SEI message GEA with gfve_gfv_id equal to gfveGfvIdA, gfve_gfv_cnt value equal to gfveGfvCntA, and a GFVE SEI message GEB with gfve_gfv_id equal to gfveGfvIdB, gfve_gfv_cnt value equal to gfveGfvCntB, are present in the same picture unit, and gfveGfvIdA is equal to gfveGfvIdB and gfveGfvCntA is less than gfveGfvCntB, the GFVE SEI message GEA shall precede the GFVE SEI message GEB in decoding order.

- If a GFVE SEI message GEA with gfve_gfv_id equal to gfveGfvIdA, gfve_gfv_cnt value equal to gfveGfvCntA, and a GFV SEI message GB with gfv_id equal to gfvIdB, gfv_cnt value equal to gfvCntB, are present in the same picture unit, if gfveGfvIdA is equal to gfvIdB, gfveGfvCntA is less than gfvCntB, the GFVE SEI message GEA shall precede the GFV SEI message GB in decoding order.

- When either of the following conditions is true, GFVE SEI messages shall have the same SEI payload content:

  - The GFVE SEI messages are present in the same picture unit, have gfve_nn_base_flag present and equal to 1 and have the same value of gfve_id, gfve_gfv_id, gfve_gfv_cnt.

  - The GFVE SEI messages are present in the same picture unit, have gfve_nn_base_flag not present and have the same value of gfve_id, gfve_gfv_id, gfve_gfv_cnt.

gfve_nn_present_flag equal to 1 indicates a neural network that may be used as a EnhancerNN( ) is contained or indicated by the SEI message. gfve_nn_present_flag equal to 0 indicates a neural network that may be used as a EnhancerNN( ) is not contained or indicated by the SEI message. When gfve_nn_present_flag is not present, it is inferred to be 0.

gfve_nn_base_flag, gfve_nn_mode_idc, gfve_nn_reserved_zero_bit_a, gfve_nn_tag_uri, gfve_nn_uri, gfve_nn_payload_byte[ i ] specify a neural network that may be used as a TranslatorNN( ). gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] have the same syntax and semantics as nnpfc_base_flag, nnpfc_mode_idc, nnpfc_reserved_zero_bit_a, nnpfc_tag_uri, nnpfc_uri, nnpfc_payload_byte[ i ], respectively.

gfve_matrix_element_precision_factor indicates quantization factor of matrix elements signalled in the SEI message.

gfve_num_matrices_minus1 plus 1 specifies the number of matrices signalled in the SEI message. The value of gfv_num_matrices_minus1 shall be in the range of 0 to 216 – 1, inclusive.

gfve_matrix_height_minus1[ i ] plus 1 indicates the height of the i-th matrix.

gfve_matrix_width_minus1[ i ] plus 1 indicates the with of the i-th matrix.

gfv_matrix_element[ i ][ j ][ k ] indicates the inv-quantized value of the element at position (k, j) of the i-th matrix.

gfve_matrix_element_sign_flag[ i ][ j ][ k ] in indicates the sign of the matrix element at position (k, j) of the i-th matrix.

The variable matrixElementVal[ i ][ j][ k ] representing the value of the matrix element at position (k, j) of the i-th matrix is derived as follows:

matrixElementVal[ i][ j ][ k ] = 1-2*gfve_matrix_element_sign_flag[ i ][ j ][ k ]*gfve_matrix_element[ i ][ j ][ k ]1<< gfve_matrix_element_precision_factor

The following process is invoked for each GFVE SEI message to enhance the picture generated with the associated GFV SEI message:

DeriveInputTensors( ) 
if(ChromaFormatIdc == 0 ) 
  EnhancerNN( inputBaseY, inputGenY, inputMatrix) 
else 
  EnhancerNN( inputBaseY, inputBaseCb, inputBaseCr, inputGenY, inputGenCb, inputGenCr, inputMatrix) 
}
StoreOutputTensors( )

The process DeriveInputTensors( ) for deriving the inputs of EnhancerNN ( ) is specified as follows:

The BasePicture input tensor inputBaseY, inputBaseCb and inputBaseCr are derived as follows:

for( x = 0; x < CroppedWidth; x++ ) {
  for ( y = 0; y < CroppedHeight; y++ ) {
    inputBaseY[ x ][ y ] = InpY( baseCroppedYPic[ x ][ y ] ) 
  }
}
if (ChromaFormatIdc !=0) {
  for( x = 0; x < CroppedWidth/ SubWidthC; x++ ) {
    for ( y = 0; y < CroppedHeight/ SubHeightC; y++ ) {
      inputBaseCb[ x][ y ] = InpC( baseCroppedCbPic[ x][ y ] )
      inputBaseCr[ x][ y ] = InpC( baseCroppedCrPic[ x ][ y ] )
    }
  }
}

The GenPicture input tensor inputGenY, inputGenCb and inputGenCr are derived as follows:

for( x = 0; x < CroppedWidth; x++ ) {
  for ( y = 0; y < CroppedHeight; y++ ) {
    inputGenY[ x ][ y ] = InpY( genCroppedYPic[ x ][ y ] ) 
  }
}
if (ChromaFormatIdc !=0) {
  for( x = 0; x < CroppedWidth/ SubWidthC; x++ ) {
    for ( y = 0; y < CroppedHeight/ SubHeightC; y++ ) {
      inputGenCb[ x][ y ] = InpC( genCroppedCbPic[ x][ y ] )
      inputGenCr[ x][ y ] = InpC( genCroppedCrPic[ x ][ y ] )
    }
  }
}

The matrix input tensor inputMatrix is derived as follows:

for ( i = 0; i <= gfve_num_matrices_minus1; i++ ) {
  for ( j = 0; j <= gfve_matrix_height_minus1[ i ]; j++) {
    for( k = 0; k <= gfve_matrix_width_minus1[ i ]; k++ ) {
      inputMatrix[ i ][ j ][ k ] = matrixElementVal[ i ][ j][ k ] 
    }
  }
}

where the functions InpY( ) and InpC( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) – 1 )

InpC( x ) = x ÷ ( ( 1  <<  BitDepthC ) – 1 )

EnhancerNN ( ) is a process to enhance the sample values of an generated picture that is generated with the associated GFV SEI message. Input values to GenerativeNN( ) and output values from GenerativeNN( ) are real numbers.

Inputs to EnhancerNN () are:

- When ChromaFormatIdc is equal to 0: inputBaseY, inputGenY, inputMatrix

- When ChromaFormatIdc is not equal to 1: inputBaseY, inputBaseCb, inputBaseCr, inputGenY, inputGenCb, inputGenCr, inputMatrix

Outputs of EnhancerNN ( ) are:

- A luma sample array enhanceY

- When ChromaFormatIdc is not equal to 0, two chroma sample arrays enhanceCb and enhanceCr.

The process StoreOutputTensors( ) for deriving the output is specified as follows:

the output sample array outYPic[ x ][ y ], outCbPic[ x ][ y ], and outCrPic[ x ][ y ] are derived as follows:

for(x=0; x< CroppedWidth; x++){
  for(y=0; y< CroppedHeight; y++){
    outputYPic[ x ][ y ] = OutY(enhanceY[ x ][ y ]) 
  }
}

if(ChromaFormatIdc != 0) {
  for(x=0; x< CroppedWidth/ SubWidthC; x++){
    for(y=0; y< CroppedHeight/ SubHeightC; y++){
      outputCbPic[ x ][ y ] = OutC(enhanceCb[ x ][ y ]) 
      outputCrPic[ x][ y ] = OutC(enhanceCr[ x ][ y ]) 
    }
  }
}

where the functions OutY( ) and OutC( ) are specified as follows:

OutY( x ) = Clip3( 0, ( 1  <<  BitDepthY ) – 1 , x * ( ( 1  <<  BitDepthY ) – 1 ) 
OutC( x ) = Clip3( 0, ( 1  <<  BitDepthC ) – 1 , x * ( ( 1  <<  BitDepthC ) – 1 )

Conclusion

It is proposed to extend the SEI message design for GFVC to support scalable representation and layered reconstruction of GFVC. Two options are provided in the contributions, of which the first one is to extend the current GFV SEI message and the second one is to introduce new GFVE SEI message. It is suggested to include one of the proposed options into the next draft of VSEI TuC.

References

- Chen B, Chen J, et al. “AHG9: Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AC0088, January 2023.

- Chen B, Chen J, et al. “AHG9: Common SEI Message for Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AD0051, April 2023.

- McCarthy S, et al. “AHG9: Generative Face Video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0080, July 2023.

- Chen B, Chen J, et al. “AHG9: Common SEI Message of Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0083, July 2023.

- Chen B, Chen J, et al. “AHG9: Common Text for Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0280, July 2023.

- Chen B, Chen J. et al, “AHG9: Common Text for Proposed Generative Face Video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AF0234, October 2023.

- Chen J, Chen B, et al, “AHG9/AHG16: Common text for proposed generative face video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AG0203, January 2024.

- McCarthy S., Chen J. et all, “Technologies under consideration for future extensions of VSEI (version 3),” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AG2032, March 2024.

- Chen B. Y. Ye, et all, “AHG16: Scalable Representation and Layered Reconstruction for Generative Face Video Compression”, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AH0110, April 2024

Patent rights declaration(s)

Alibaba and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).