| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 14–24 April 2024 | Document: JVET-AH0131-v1 |
| --- | --- |

| Title: | AhG12: Fix on BV Filtering for IntraTMP Fusion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seonjae Kim Dongsan Jun (Dong-A University) 550 Nakdongdae-ro, Saha-gu, Busan, 49315, KOREA | Tel: Email: | +82-51-200-5593 sjkim@donga.ac.kr dsjun@dau.ac.kr |
| Source: | Dong-A University (DAU) |  |  |

_____________________________

- Abstracts

The IntraTMP fusion prediction that combines multiple matched blocks derived from the intra template matching process was proposed in JVET-AD0083 and adopted to ECM-12.0. While calculating the number of fusion predictors (fusionNum), a threshold is used to judge whether it should be used for fusion. This contribution investigates the effect of changing the block vector filtering procedure of fusion candidates. Experiment results of the proposed method on top of ECM-12.0 are reported as follows:

Test1 (Inherit fusionNum of set 0 as other sets, Encoder clean-up)

AI: {0.00% Y, 0.00% U, 0.00% V, 100.0% EncT, 100.1% DecT}

Test2 (Calculate fusionNum for each fusion set)

AI: {0.00% Y, -0.01% U, -0.02% V, 100.2% EncT, 100.4% DecT}

Introduction

Intra template matching prediction (TMP) fusion method was presented in JVET-AD0086 [1] and adopted in ECM-12.0 [2]. In the template matching process, the encoder/decoder collects 19 neighboring block vectors (BV) in the search area based on their template cost (SAD). For IntraTMP fusion, the BVs of 3 candidates' fusion sets are obtained from five consecutive BVs in an SAD-based sorted BV list. Through the fusion process combining multiple matched blocks using weighted sum or Weiner filter, IntraTMP fusion derives the final prediction block.

In order to calculate the number of fusion predictors (fusionNum), a threshold is used for each of the 3 candidates' fusion sets to judge whether it should be used for fusion. If the template cost is higher than the threshold, corresponding matched blocks will not be combined. The threshold in each candidate of the fusion set can be calculated as follows:

- Fusion set 0: Thresholdset0=SADBV0≪1

- Fusion set 1: Thresholdset1=SADBV5*1.2

- Fusion set 2: Thresholdset2=SADBV10*1.2

However, there is an issue in the current implementation of ECM-12.0 where fusion sets 1 and 2 are referred to as template costs of fusion set 0 instead of each template cost when calculating fusionNum. As a result, entire fusion sets have the same fusionNum, which depends on the template costs of fusion set 0. Moreover, the encoder should calculate the same fusionNum repeatedly for each fusion set, as shown in Figure 1.

Figure 1. Example of the implementation issue in the IntraTMP fusion

Proposed methods

In this contribution, the block vector filtering procedure for IntraTMP fusion prediction is fixed from two aspects.

Test1: Inherit fusionNum of set 0 as other sets (Encoder clean-up).

The encoder calculates fusionNum only once in fusion set 0, and other fusion sets are inherited fusionNum of set 0. This method does not change the ECM-12.0 anchor performance.

Figure 2. Example of the test1 (Inherit fusionNum of set 0 as other sets)

Test2: Calculate fusionNum for each fusion set.

In order to calculate fusionNum individually, each fusion set refers to corresponding block vectors and template costs. Threshold scales are the same as the ECM-12.0 implementation (2 / 1.2 / 1.2).

Figure 3. Example of the test2 (Calculate fusionNum for each fusion set)

Experimental results

The tables below summarize the coding efficiency, runtime, and peak memory consumption of proposed methods in ECM 12.0 using the JVET common test conditions [3].

Test1: Inherit fusionNum of set 0 as other sets (Encoder clean-up).

Table 1. Performance of the test1 on top of ECM-12.0 (AI)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | 0.00% | 100.3% | 100.1% | 100.0% | 100.0% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100.0% | 100.1% | 100.0% | 100.0% |
| Class B | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class C | 0.00% | 0.00% | 0.00% | 100.1% | 100.0% | 100.0% | 100.0% |
| Class E | 0.00% | 0.00% | 0.00% | 99.9% | 100.3% | 100.0% | 100.0% |
| Overall | 0.00% | 0.00% | 0.00% | 100.0% | 100.1% | 100.0% | 100.0% |
| Class D | 0.00% | 0.00% | 0.00% | 99.7% | 100.2% | 100.0% | 100.0% |
| Class F | 0.00% | 0.00% | 0.00% | 99.9% | 100.2% | 100.0% | 100.0% |
| Class TGM | 0.00% | 0.00% | 0.00% | 99.8% | 100.1% | 100.0% | 100.0% |

Test2: Calculate fusionNum for each fusion set.

Table 2. Performance of the test2 on top of ECM-12.0 (AI)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.07% | -0.01% | 100.3% | 100.5% | 101.7% | 100.0% |
| Class A2 | -0.01% | 0.06% | 0.02% | 100.2% | 100.5% | 100.2% | 100.0% |
| Class B | -0.01% | 0.02% | -0.01% | 100.1% | 100.2% | 99.6% | 100.2% |
| Class C | 0.02% | 0.02% | -0.06% | 100.3% | 100.2% | 100.0% | 100.0% |
| Class E | 0.00% | -0.14% | -0.01% | 100.1% | 100.6% | 99.8% | 100.0% |
| Overall | 0.00% | -0.01% | -0.02% | 100.2% | 100.4% | 100.2% | 100.0% |
| Class D | 0.00% | -0.04% | -0.02% | 100.1% | 100.3% | 99.9% | 100.0% |
| Class F | 0.02% | -0.02% | 0.06% | 100.0% | 100.5% | 98.0% | 100.0% |
| Class TGM | 0.00% | -0.01% | -0.09% | 100.0% | 100.4% | 103.8% | 100.0% |

Conclusion

This contribution presents two fixes for block vector filtering process of IntraTMP fusion prediction. Experimental results reportedly show marginal coding gain without encoder/decoder runtime change. It is recommended to Test2 in next version ECM.

References

- F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang, Y. Ma, H. Zhang, J. Huo, F. Yang, P. Lin, J. Lin, V. Seregin, M. Karczewicz, X. Li, R. Liao, J. Chen, and Y. Ye, “EE2-1.20i/j: Combination of IntraTMP tests”, JVET-AD0086, 30th meeting, April 2023.

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, 32nd meeting, Oct. 2023.

Patent rights declaration(s)

Dong-A University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).