| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 14–24 April 2024 | Document: JVET-AH0131-v2 |
| --- | --- |

| Title: | AhG12: Fix on BV Filtering for IntraTMP Fusion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seonjae Kim Dongsan Jun (Dong-A University) 550 Nakdongdae-ro, Saha-gu, Busan, 49315, KOREA | Tel: Email: | +82-51-200-5593 sjkim@donga.ac.kr dsjun@dau.ac.kr |
| Source: | Dong-A University (DAU) |  |  |

_____________________________

- Abstracts

IntraTMP fusion was proposed in JVET-AD0083 and implemented on top of ECM-12.0. For each fusion set, the SAD-based thresholds are used to discriminate whether the blocks are combined or not in the fusion set. Current ECM-12.0 has a SW bug where both fusion set1 and set2 calculate block vector (BV) filtering using the SAD-based threshold from fusion set0. This contribution fixes the SW bug in the BV filtering of IntraTMP fusion. Experiment results on top of ECM-12.0 are reported as below:

AI: {0.00% Y, -0.01% U, -0.02% V, 100.2% EncT, 100.3% DecT}

Introduction

Intra template matching prediction (TMP) fusion method was presented in JVET-AD0086 [1] and adopted in ECM-12.0 [2]. In the template matching process, the encoder/decoder collects 19 neighboring block vectors (BVs) within the search area through the template cost (SAD) comparisons. IntraTMP fusion method makes 3 fusion sets with the reordering 19 BVs and each set initially has the 5 BVs. For each fusion set, the SAD-based thresholds are used to discriminate whether the block indicated by a BV will be used to or not.

The threshold of each fusion set can be calculated as below:

- Fusion set 0: Thresholdset0=SADBV0≪1

- Fusion set 1: Thresholdset1=SADBV5*1.2

- Fusion set 2: Thresholdset2=SADBV10*1.2

Figure1. Example of the block vector filtering in IntraTMP fusion

As shown in Figure 2, we report that current ECM-12.0 has a SW bug where all fusion sets calculate the BV filtering from fusion set0. As a result, all fusion sets have the same fusionNum, which depends on the template costs of fusion set0. Moreover, the encoder should calculate the same fusionNum repeatedly for each fusion set.

Figure2. Example of the SW bug on ECM12.0

Proposed methods

In this contribution, it is proposed to fix the BV filtering process of IntraTMP fusion with two SW modifications.

Experimental results

The table below summarizes the coding efficiency, runtime, and peak memory consumption of proposed methods under the JVET common test conditions [3].

Table 1. Performance of the proposed fix on top of ECM-12.0 (AI)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.07% | -0.01% | 100.3% | 100.2% | 101.7% | 100.0% |
| Class A2 | -0.01% | 0.06% | 0.02% | 100.2% | 100.3% | 100.2% | 100.0% |
| Class B | -0.01% | 0.02% | -0.01% | 100.1% | 100.3% | 99.6% | 100.2% |
| Class C | 0.02% | 0.02% | -0.06% | 100.3% | 100.2% | 100.0% | 100.0% |
| Class E | 0.00% | -0.14% | -0.01% | 100.1% | 100.4% | 99.8% | 100.0% |
| Overall | 0.00% | -0.01% | -0.02% | 100.2% | 100.3% | 100.2% | 100.0% |
| Class D | 0.00% | -0.04% | -0.02% | 100.1% | 100.2% | 99.9% | 100.0% |
| Class F | 0.02% | -0.02% | 0.06% | 100.0% | 100.4% | 98.0% | 100.0% |
| Class TGM | 0.00% | -0.01% | -0.09% | 100.0% | 100.4% | 103.8% | 100.0% |

Conclusion

This contribution presents SW fix of IntraTMP fusion prediction. Experimental results show marginal chroma coding gain with negligible encoder/decoder runtime changes. It is recommended to incorporate the proposed fix in the next ECM.

References

- F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang, Y. Ma, H. Zhang, J. Huo, F. Yang, P. Lin, J. Lin, V. Seregin, M. Karczewicz, X. Li, R. Liao, J. Chen, and Y. Ye, “EE2-1.20i/j: Combination of IntraTMP tests”, JVET-AD0086, 30th meeting, April 2023.

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, 32nd meeting, Oct. 2023.

Patent rights declaration(s)

Dong-A University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).