| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0133-v1 |
| --- | --- |

| Title: | EE1-4.3: RPR new input for super-resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiedong Ye, Xu Li, Yiqing Zhu, Qiong Liu (HUST) Chuan Zhou, M. Rafie, Zhuoyi Lv (vivo) | Email: | ye_jd@hust.edu.cn q.liu@hust.edu.cn chuan.zhou@vivo.com |
| Source: | Huazhong University of Science and Technology vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the EE1-4.3 test results of NN-based low complexity super-resolution (SR) with new RPR input. Based on JVET-AG0114, this contribution further restructures the network from a high complexity to a low complexity version. First we retain the reference picture resampling (RPR) picture as a new input to the neural network, and ablation experiment is performed to demonstrate that the new inputs do bring performance gains. Second, we use NNVC-8.0 to regenerate a new training dataset with NN tools (NNintra + NNlf_LOP). We use 2 dataset (NN-off dataset and NN-on dataset) to train our low complexity SR model separately.

All tests are based on the nnvc_anchor, activate SR model with NN tools (NNintra + NNlf_LOP) and calculate BD-rates savings compared to NNVC-8.0-nnvc_anchor.

Test EE1-4.3.1.a: SR model trained by NN-off dataset are shown {xx%, xx%, xx%} and {-0.79%, 1.27%, 1.79%} BD-rate savings under RA and AI, respectively.

Test EE1-4.3.1.b: ablation SR model trained by NN-off dataset are shown {xx%, xx%, xx%} and {xx%, xx%, xx%} BD-rate savings under RA and AI, respectively.

Test EE1-4.3.2.a: SR model trained by NN-on dataset are shown {xx%, xx%, xx%} and {xx%, xx%, xx%} BD-rate savings under RA and AI, respectively.

Test EE1-4.3.2.b: ablation SR model trained by NN-on dataset are shown {xx%, xx%, xx%} and {xx%, xx%, xx%} BD-rate savings under RA and AI, respectively.

Introduction

JVET-AG0114[1] presents a high complexity SR model with RPR new input, whose complexity is 452 kMAC/pixel.

In this contribution, the SR model is restructured from a high complexity to a low complexity version, whose complexity is 20.2 kMAC/pixel and the RPR is retained as a new input. In addition to this we conducted additional experiments. First, to prove that the RPR new input is effective, we performed an ablation experiment by removing the RPR input from the network and compensating the complexity to be the same as before. Second, considering that the inference phase activates the NN tools, we also activated the NN tools on NNVC-8.0 to generate a new dataset to retrain our SR model.

In this contribution, the following test results are reported: SR model trained by NN-off dataset (test 4.3.1.a), ablation SR model trained by NN-off dataset (test 4.3.1.b), SR model trained by NN-on dataset with NN tools (test 4.3.2.a), ablation SR model trained by NN-on dataset with NN tools (test 4.3.2.b).

Proposed method

Low complexity network architecture with new input

In unified SR model in NNVC-8.0, reconstruction picture, prediction picture, slice QP and so on are fed into network. The RPR image is only used at the end of the network as a baseline for upsampling results.

Figure 1 Low complexity network architecture with new input

The unified SR network architecture and parameters setting are shown in Figure 1. We believe that the content of RPR images can also provide valid information for SR networks. Therefore, we feed it as a new additional input to the network, similar to reconstruction picture and prediction picture, also extract its shallow features using 1-layer 3×3 convolution. Considering the resolution of the RPR image, the stride of its convolution is set to 2. Then all the shallow features are concatenated together and fed into the deep feature extraction network.

Ablation experiment network architecture

Figure 2 Ablation experiment network architecture

In the ablation experiment, we remove the RPR input. To make the experimental conditions fairer, we increased the number of channels in the reconstruction image branch from 32 to 48 to make the complexity consistent with the original model. The network complexity of ablation model is 20.2 kMAC/pixel.

Training stage information

Table 1 Network Information for our LOP SR model

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~1000 |
|  | Batch size: | 32 |
|  | Loss function: | L1 |
|  | Training time (for 1 model): | ~5 days |
|  | Training data information: | TVD_4K, BVI-DVC_4K, DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0_nnvc-6.0, QP{17, 22, 27, 32, 37, 42} Or NNVC-8.0 with NN tools on, QP{17, 22, 27, 32, 37, 42} |
| Optional | Number of iterations |  |
|  | Patch size | 72*72 |
|  | Learning rate: | 2e-4 |
|  | Learning rate update strategy | Reduced 3 times in the later part of training |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flip, rotation |
|  | Mini-batch selection process: | random cropping |
|  | Training data update strategy: |  |
|  | Other information: |  |

Inference stage information

Table 3 Inference information for our LOP SR model

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | Not used |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 0.08M |
|  | Total Number of Parameters (All Models) | 0.08M |
|  | Parameter Precision (Bits) | int16 |
|  | Memory Parameter (MB) | 0.2 |
|  | Multiply Accumulate (MAC/pixel) | 20.2 K |
|  | Calculation Method | thop |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 256*256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Experiment results

This section will show the coding performance of our two sets of experiments in the AI and RA. The proposed CNN-based up-sampling filter is tested on top of NNVC 8.0, according to the common test conditions [2]. The tested QP values are {22, 27, 32, 37, 42}.

Test 4.3.1.a

Our LOP SR model trained by NN-off dataset reportedly shows on average {xx%, xx%, xx%} and {-0.79%, 1.27%, 1.79%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

Table 4 Results of SR model trained by NN-off dataset in RA configurations

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 NNVC anchor + NNSR |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C |  |  |  |
| Class E |  |  |  |
| Overall |  |  |  |

Table 5 Results of SR model trained by NN-off dataset in AI configurations

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 NNVC anchor + NNSR |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.70% | 2.45% | 3.57% |
| Class A2 | -3.06% | 5.17% | 7.15% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% |
| Overall | -0.79% | 1.27% | 1.79% |

Test 4.3.1.b

Our ablation model trained by NN-off dataset reportedly shows on average {xx%, xx%, xx%} and {xx%, xx%, xx%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

Table 6 Results of ablation model trained by NN-off dataset in RA configurations

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 NNVC anchor + NNSR |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C |  |  |  |
| Class E |  |  |  |
| Overall |  |  |  |
| Result of Test 4.3.1.a (float) | -0.84% | 1.54% | 1.74% |

Table 7 Results of ablation model trained by NN-off dataset in AI configurations

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 NNVC anchor + NNSR |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C |  |  |  |
| Class E |  |  |  |
| Overall |  |  |  |
| Result of Test 4.3.1.a (float) | -0.68% | 1.42% | 1.95% |

Test 4.3.2.a

Our LOP SR model trained by NN-on dataset with NN-tools reportedly shows on average {xx%, xx%, xx%} and {xx%, xx%, xx%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

Table 8: Results of SR model trained by NN-on dataset in RA configurations

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 NNVC anchor + NNSR |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C |  |  |  |
| Class E |  |  |  |
| Overall |  |  |  |

Table 9: Results of SR model trained by NN-on dataset in AI configurations

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 NNVC anchor + NNSR |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C |  |  |  |
| Class E |  |  |  |
| Overall |  |  |  |

Test 4.3.2.b

Our ablation model trained by NN-on dataset with NN-tools reportedly shows on average {xx%, xx%, xx%} and {xx%, xx%, xx%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

Table 8: Results of ablation model trained by NN-on dataset in RA configurations

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 NNVC anchor + NNSR |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C |  |  |  |
| Class E |  |  |  |
| Overall |  |  |  |
| Result of Test 4.3.2.a (float) |  |  |  |

Table 9: Results of ablation model trained by NN-on dataset in AI configurations

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 NNVC anchor + NNSR |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C |  |  |  |
| Class E |  |  |  |
| Overall |  |  |  |
| Result of Test 4.3.2.a (float) |  |  |  |

Conclusion

In this contribution, we reduce the model complexity of our previous reported network architecture, and propose a new RPR input to further improve the coding performance of the super-resolution (SR) model.

In addition to this, firstly, in order to verify the effectiveness of the new RPR input, we perform an ablation experiment. Secondly, considering that the NN tools are activated in inference stage, we regenerate the dataset with the NN tools activated to train the model.

The results show promising coding performance with new RPR input. The performance of the models obtained by training with the NN-on dataset is greatly improved when used with the NN tools. It is recommended to adopt the proposed methods in NNVC.

Reference

[1] Jiedong Ye, Xu Li, Yiqing Zhu, Qiong Liu, Chuan Zhou, M. Rafie, Zhuoyi Lv, “AHG11: On new input and backbone enhancement of model for super-resolution”, JVET-AG0114

[2] E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AF2016.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).