| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0134-v1 |
| --- | --- |

| Title: | [AHG9] Improvement on the SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Gao Ping Wu Yaxian Bai Shaowei Xie Menghu Jia Weihong Niu Cheng Huang | Email: | {gao.ying, ping.wu, bai.yaxian, xie.shaowei, jia.menghu, niu.weihong, huang.cheng5} @zte.com.cn |
| Source: | ZTE |  |  |

_____________________________

- Abstract

This contribution is a follow-up of previous meeting input document JVET-AG0105. One potential use case extended from the example in JVET-AE0156 was given and explained in this version of contribution.

Furthermore, some minor changes to the proposed syntax and semantics in JVET-AG0105 have been done in response to the update in JVET-AG2027 from its previous version.

Introduction

Background

In 33rd JVET meeting, JVET-AG0105[1] proposed to add a flag (po_sei_prcessing_degree_flag[i]) to specify whether or not the decoder should ignore the entire SPO SEI message if it cannot interpret or does not support the functionality indicated by any indicated SEI message that has po_sei_importance_flag[ i ] equal to 1. A few use cases, associated with po_sei_important_flag eaqual to 1, can be found in Dolby’s contribution in JVET-AE0156 [2].

After last meeting, there were some changes made to JVET-AG2027[3] compared to its previous version of document, and one of the updates is as follows:

“Delete“However, if po_sei_wrapping_flag[ i ] is equal to 0 and no SEI message is present with payloadType equal to po_sei_payload_type[ i ] or po_sei_wrapping_flag[ i ] is equal to 0 and po_sei_prefix_flag[ i ] is equal to 1 and no SEI message is present with with payloadType equal to po_sei_payload_type[ i ] that has prefix data that matches the values of po_sei_prefix_data_bit[ i ][ j ], the following applies:

- If po_sei_importance_flag[ i ] is equal to 1, the decoder should ignore the entire SEI processing order SEI message.

- Otherwise, the decoder should ignore all data associated with the loop variable value of i.”

Based on the above change, it can be clarified that if the decoding system cannot interpret or does not support the functionality indicated by any indicated SEI message that has po_sei_importance_flag[ i ] equal to 1, it should ignore the entire SPO SEI message regardless of whether this important message is wrapped or not .

Examples of usage

For the explanation of the benefit by adding one flag in JVET-AG0105[1], one example of the use of the proposed scheme with the SEI messages in their processing order is as follows:

- A post-filter hint (PFH) SEI message (to improve picture quality), intended as discretionary, with:

po_sei_importance_flag[ 0 ] = 0

po_sei_processing_order[ 0 ] = 0 (this is the first stage in the SPO)

- A film grain characteristics (FGC) SEI message (to achieve the expected artistic effect or to mask imperfections in digital footage),  with:

po_sei_importance_flag[ 1] = 0

po_sei_processing_order[ 1 ] =1 (this is the second stage in the SPO)

- A tone mapping information (TMI) SEI message (to adapt to some alternative display), necessary for appropriate interpretation of the following data, with:

po_sei_importance_flag[ 2 ] = 1 (to indicate that this process is fundamental to the purpose of this SPO)

po_sei_processing_degree_flag[2] = 0 (to indicate that if this process is not executed, it will only affect subsequent SEI message in the SPO)

po_sei_processing_order[ 2 ] = 2 (this is the third stage in the SPO)

- A content light level information (CLL) SEI message (to indicate the light level after applying the tone mapping information)

po_sei_importance_flag[3 ] = 0 (to indicate that this indicator is not necessary for interpretation of the output of the second stage)

po_sei_processing_order[ 3 ] = 3 (this is the forth stage in the SPO)

In this example, a film grain characteristic SEI message is added into the processing chain based on the original example provided in JVET-AE0156[2]. As described in JVET-AE0156[2], since the indicated CLL information is only valid if the tone mapping process indicated in the TMI SEI message is applied, so it is important that the CLL SEI message should not be used unless the tone mapping processing has been applied.

According to the existing design of the SPO SEI message as indicated in JVET-AG2027 [3], if the decoding system cannot interpret or does not support the tone mapping functionality indicated by the TMI SEI message, it should ignore the entire SPO SEI message. That is to say, the decoder can process the PFH SEI message and the FGC SEI message in random order, which means that the decoder may process the FGC SEI message before the PFH SEI message. The possible result of this is that the synthetic film grain added to the picture may be mistakenly treated as noise and filtered by post-filtering operation. Obviously, it seems more reasonable to process the PFH SEI message and the FGC SEI message in the preferred order indicated in the SPO SEI message even if the TMI SEI message is not applied.

Additionally, although the encoder can send another SPO SEI message containing only PFH SEI message and the FGC SEI message in the processing chain to achieve similar purpose, it remains to be discussed whether it is appropriate for the encoder to consider the problems that the decoder may encounter. At the same time, it is also necessary to consider how to determine the relationship between these two SPO SEI messages when the decoder receives the SPO SEI message containing only PFH SEI message and the FGC SEI message mentioned above and the SPO SEI message containing the complete processing chain.

Based on the above analysis, it is recommended to add a flag (processing_degree_flag) to provide a simple and flexible mechanism to determine whether the entire processing chain should be ignored or only a part of the processing chain should be ignored when the decoder cannot process an important SEI message that has po_sei_importance_flag equal to 1 in the processing chain indicated by the SPO SEI message.

Proposed syntax and semantics

Because JVET-AG2027[3] has been updated since the last meeting, the following syntax and semantic information have been slightly modified (removed the constraint that po_sei_wrapping_flag [i] is equal to 0 in the if condition) compared to JVET-AG0105[1] .

Proposed change of syntax of the SEI processing order SEI message

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| if(po_sei_importance_flag[ i ] && !po_sei_wrapping_flag[ i ]) |  |
| po_sei_processing_degree_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| ...... |  |
| } |  |

Proposed change of semantics of the SPO SEI message

po_sei_importance_flag[ i ] indicates the degree of importance determined by the encoder for the type of SEI message with index i.

If the decoding system cannot interpret or does not support the functionality indicated by any indicated SEI message that has po_sei_importance_flag[ i ] equal to 1, it should ignore the entire SPO SEI message process the SPO SEI message based on the value of po_sei_processing_degree_flag[i].

po_sei_processing_degree_flag[i] (when present) indicates the degree of processing for SPO SEI message by the decoder.

If the functionality indicated by the SEI message that has po_sei_importance_flag[ i ] equal to 1 cannot be interpreted or supported by the decoding system, the following applies:

- po_sei_processing_degree_flag[i] equal to 1 indicates that the decoder should ignore the entire SPO SEI message.

- po_sei_processing_degree_flag[i] equal to 0 indicates that the decoder should process those SEI messages which has po_sei_processing_order[j] less than po_sei_processing_order[i] based on the preferred order in the SPO SEI message.

References

- Y. Gao, Y.-X. Bai, S.-W. Xie, M.-H. Jia, C. Huang, P. Wu (ZTE), "AHG9: On the SEI processing order SEI message", JVET-AG0105.

- G. J. Sullivan, S. McCarthy, P. Yin (Dolby), "[AHG9] Message wrapping and importance indication for the SEI processing order SEI message", JVET-AE0156.

- G. J. Sullivan, M. M. Hannuksela, Y.-K. Wang “SEI processing order and processing order nesting SEI messages in VVC (draft 7)” JVET-AG2027

Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).