| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0138-v1 |
| --- | --- |

| Title: | AHG9/AHG16: Pupil position SEI message for Generative Face Video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anthony Trioux, Yayun Yao, Faming Ma, Fuzheng Yang, Fang Xing, Zhikui Wang | Email: | anthony_triox@xidian.edu.cn yaoyayun@stu.xidian.edu.cn fmma@stu.xidian.edu.cn fzhyang@mail.xidian.edu.cn xingfang@hisense.com wangzhikui@hisense.com |
| Source: | Xidian University，Hisense Group Co., Ltd. |  |  |

# Abstract

At the 33rd JVET meeting, one of the tasks of AHG16 was to “Coordinate with AHG9 on further development of the GFVC SEI message in JVET-AG2032[1]”. In this contribution, towards the goal of improving realism in generative face video coding, we propose to convey the concept of eye movement information and propose to add the GFV SEI message corresponding to the pupil center position, which represents an important information.

# Introduction

Figure 1 shows the general block diagram of generative face video compression (GFVC). A base picture, also known as key frame, is first passed through a color format converter and then classically encoded for transmission through VVC. The following pictures, or subsequent pictures, are passed through an analysis model to extract facial representation (landmarks, keypoints, etc.), which are then encoded by arithmetic coding and transmitted. At the receiver-side, the base picture follows the reverse processing. Then, a generative model is used to regenerate the subsequent pictures, whose inputs include the decoded facial representation and the reconstructed base picture.

Figure 1 - General block diagram of GFVC.

In this contribution, we point out the importance of eye movement when dealing with face video contents. Both blinking and changes in gaze direction can make face videos more realistic. As stated in JVET-AG0042[2], in current generative face video coding architectures, facial information is often represented through: 2D keypoints, 2D landmarks, 3D keypoints, facial semantics or compact temporal feature. Such representations cannot accurately represent eye movements. In addition, in JVET-AG2032[1] only an Eye matrix is used to represent the open-close status and level of eyes, which do not convey the changes of pupil position information. By using the official software implementation (https://github.com/Berlin0610/GFVC_Software), we show in Figure 2, the difficulties of the generative model to generate accurate eye positions, highlighting the importance of transmitting such information as part of the SEI information.

Figure 2 - Incorrect pupil position in current method

The eye movement information can be obtained by the analysis model together with the facial representation and transmitted at the decoding side to obtain more realistic and accurate eye movements through the generative model. Therefore, we propose to add the pupil center position information to the SEI information of the generated face video.

# Pupil position representation

In the process of generative face video coding, the pupil position changes occur only in a small part of each video frame, therefore, to reduce the amount of transmitted information, we use the relative coordinate information of the pupil position with respect to the inner eye corner of the eye, taken as the reference point.

Specifically, as shown in Figure 3, we define a coordinate system by taking the inner corner of the eye as the origin, the line connecting the inner and outer corners of the eye as the x-axis, and the perpendicular of the x-axis as the y-axis to establish a rectangular coordinate system. The pupil center coordinates (x, y), correspond to the distance between the pupil center and the x-axis and y-axis, respectively. Left, right or both pupil center position(s) can be transmitted using this representation. The pupil center position information can be transmitted either as direct coordinate values (x, y) or as a predicted value with respect to the pupil center position in the base picture (dx, dy). The x-axis and y-axis are normalized by the distance between the two corners of the eye.

Figure 3 - Coordinate system of pupil center position with respect to the corner of the eye.

# Proposal

We propose to add the pupil center position in the GFV SEI information, and propose the corresponding changes relative to JVET-AG2032[1].

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| if( gfv_base_pic_flag ) { /*specify TranslatorNN( )*/ |  |
| gfv_nn_present_flag | u(1) |
| if( gfv_nn_present_flag ) { |  |
| gfv_nn_base_flag | u(1) |
| gfv_nn_mode_idc | ue(v) |
| if( gfv_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_a | u(1) |
| gfv_nn_tag_uri | st(v) |
| gfv_nn_uri | st(v) |
| } |  |
| } |  |
| } else /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_fusion_flag /*indicate if DrivePicture is input to GenerativeNN( )*/ | ue(v) |
| gfv_coordinate_present_flag | u(1) |
| if( gfv_coordinate_present_flag ) { |  |
| gfv_pupil_coordinate_present_idx | u(2) |
| if (gfv_ pupil_coordinate_present _idx != 0){ |  |
| gfv_pupil_coordinate_precision_factor_minus1 | ue(v) |
| gfv_pupil_pred_flag | u(1) |
| if (gfv_ pupil_coordinate_present _idx == 1 ¦¦ gfv_ pupil_coordinate_present _idx == 3) { |  |
| if (gfv_pupil_pred_flag) { |  |
| gfv_pupil_left_eye_dx_coordinate_abs | ue(v) |
| gfv_pupil_left_eye_dx_coordinate_sign_flag | u(1) |
| gfv_pupil_left_eye_dy_coordinate_abs | ue(v) |
| gfv_pupil_left_eye_dy_coordinate_sign_flag | u(1) |
| } else { |  |
| gfv_pupil_left_eye_x_coordinate | ue(v) |
| gfv_pupil_left_eye_y_coordinate | se(v) |
| } |  |
| } |  |
| if (gfv_ pupil_coordinate_present _idx == 2 ¦¦ gfv_ pupil_coordinate_present _idx == 3) { |  |
| if (gfv_pupil_pred_flag) { |  |
| gfv_pupil_right_eye_dx_coordinate_abs | ue(v) |
| gfv_pupil_right _eye_dx_coordinate_sign_flag | u(1) |
| gfv_pupil_right_eye_dy_coordinate_abs | ue(v) |
| gfv_pupil_right_eye_dy_coordinate_sign_flag | u(1) |
| } else { |  |
| gfv_pupil_right_eye_x_coordinate | ue(v) |
| gfv_pupil_right_eye_y_coordinate | se(v) |
| } |  |
| } |  |
| } |  |
| gfv_coordinate_precision_factor_minus1 | ue(v) |
| gfv_num_kp_minus1 | ue(v) |
| gfv_kp_pred_flag | u(1) |
| gfv_coordinate_z_present_flag | u(1) |
| ... |  |

Use of this SEI message requires the definition of the following variables:

- Distance between the corners denoted herein by CornersDistance.

- The inner eye corner of the eye, as the reference point, denoted as LeftCornerReferenceCoordinateX, LeftCornerReferenceCoordinateY, RightCornerReferenceCoordinateX and RightCornerReferenceCoordinateY.

- The pupil center position in the base picture, denoted as BaseLeftPupilPositionX, BaseLeftPupilPositionY, BaseRightPupilPositionX and BaseRightPupilPositionY.

gfv_ pupil_coordinate_present_idx: indicates which pupil position information is transmitted. When equal to 0, it means that the pupil position information does not exist; when equal to 1, it means that it contains the left eye pupil position information; when equal to 2, it means that it contains the right eye pupil position information; when equal to 3, it means that it contains both left eye and right eye pupil position information. If gfv_pupil_ coordinate_present _idx does not exist, it is inferred to be equal to 0.

gfv_pupil_coordinate_precision_factor_minus1: plus 1 indicates the length, in bits, of syntax elements gfv_pupil_left_eye_ dx_coordinate_abs, gfv_pupil_left_eye_dy_coordinate_abs, gfv_pupil_left_eye_x_coordinate, gfv_pupil_left_eye_y_coordinate, gfv_pupil_right_eye_dx_coordinate_abs, gfv_pupil_right_eye_dy_coordinate_ abs, gfv_pupil_right_eye_x_coordinate and gfv_pupil_right_eye_y_coordinate.

gfv_pupil_pred_flag: equal to 1 indicates syntax elements gfv_pupil_left_eye_dx_coordinate_abs,  gfv_pupil_left_ eye_dy_coordinate_abs,  gfv_pupil_right_eye_dx_coordinate_abs,  gfv_pupil_right_eye_dy_coordinate_abs, gfv_ pupil_left_eye_dx_coordinate_sign_flag, gfv_pupil_left_eye_dy_coordinate_sign_flag, gfv_pupil_right_eye_dx_ coordinate_sign_flag and gfv_pupil_right_eye_dy_coordinate_sign_flag may be present. gfv_pupil_pred_flag equal to 0 indicates syntax elements gfv_pupil_left_eye_x_coordinate, gfv_pupil_left_eye_y_coordinate, gfv_pupil_right_eye_x_ coordinate and gfv_pupil_right_eye_y_coordinate may be present.

gfv_pupil_left_eye_dx_coordinate_abs: indicates the absolute difference value of the normalized value of the x-axis coordinate of the left eye.

gfv_pupil_left_eye_dx_coordinate_sign_flag: specifies the sign of the difference value of the x-axis coordinate of the left eye. When equal to 0, LeftPupilCoordinateDeltaX is positive. When equal to 1, LeftPupilCoordinateDeltaX is negative. When gfv_pupil_left_eye_dx_coordinate_sign_flag is not present, it is inferred to be equal to 0.

gfv_pupil_left_eye_dy_coordinate_abs: indicates the absolute difference value of the normalized value of the y-axis coordinate of the left eye.

gfv_pupil_left_eye_dy_coordinate_sign_flag: specifies the sign of the difference value of the y-axis coordinate of the left eye. When equal to 0, LeftPupilCoordinateDeltaY is positive. When equal to 1, LeftPupilCoordinateDeltaY is negative. When gfv_pupil_left_eye_dy_coordinate_sign_flag is not present, it is inferred to be equal to 0.

The variables LeftPupilCoordinateDeltaX and LeftPupilCoordinateDeltaY indicating the delta x-axis coordinate and delta y-axis coordinate of the left pupil center position, respectively, are derived as follows:

LeftPupilCoordinateDeltaX

= (1-2 * gfv_pupil_left_eye_dx_coordinate_sign_flag) * gfv_pupil_left_eye_dx_coordinate_abs * CornersDistance1<< (gfv_pupil_coordinate_precision_factor_minus1 +1)

LeftPupilCoordinateDeltaY

= (1-2 * gfv_pupil_left_eye_dy_coordinate_sign_flag) * gfv_pupil_left_eye_dy_coordinate_abs * CornersDistance1<< (gfv_pupil_coordinate_precision_factor_minus1 +1)

gfv_pupil_left_eye _x_coordinate: indicates the normalized value of the x-axis coordinate of the left eye.

gfv_pupil_left_eye _y_coordinate: indicates the normalized value of the y-axis coordinate of the left eye.

The variables leftPupilCoordinateX and leftPupilCoordinateY indicating the x-axis coordinate and y-axis coordinate of the left pupil center position, respectively, are derived as follows:

When gfv_pupil_pred_flag is equal to 0,

LeftPupilCoordinateX

= gfv_pupil_left_eye_x_coordinate * CornersDistance1<< (gfv_pupil_coordinate_precision_factor_minus1 +1)+LeftCornerReferenceCoordinateX

LeftPupilCoordinateY

= gfv_pupil_left_eye_y_coordinate * CornersDistance1<< (gfv_pupil_coordinate_precision_factor_minus1 +1)+LeftCornerReferenceCoordinateY

When gfv_pupil_pred_flag is equal to 1,

LeftPupilCoordinateX

= LeftPupilCoordinateDeltaX+BaseLeftPupilPositionX + LeftCornerReferenceCoordinateX

LeftPupilCoordinateY

= LeftPupilCoordinateDeltaY+BaseLeftPupilPositionY + LeftCornerReferenceCoordinateY

gfv_pupil_right_eye_dx_coordinate_abs: indicates the absolute difference value of the normalized value of the x-axis coordinate of the right eye.

gfv_pupil_right_eye_dx_coordinate_sign_flag: specifies the sign of the difference value of the x-axis coordinate of the right eye. When equal to 0, RightPupilCoordinateDeltaX is positive.When equal to 1, RightPupilCoordinateDeltaX is negative. When gfv_pupil_right_eye_dx_coordinate_sign_flag is not present, it is inferred to be equal to 0.

gfv_pupil_right_eye_dy_coordinate_abs: indicates the absolute difference value of the normalized value of the y-axis coordinate of the right eye.

gfv_pupil_right_eye_dy_coordinate_sign_flag: specifies the sign of the difference value of the y-axis coordinate of the right eye. When equal to 0, RightPupilCoordinateDeltaY is positive.When equal to 1, RightPupilCoordinateDeltaY is negative. When gfv_pupil_right_eye_dy_coordinate_sign_flag is not present, it is inferred to be equal to 0.

The variables RightPupilCoordinateDeltaX and RightPupilCoordinateDeltaY indicating the delta x-axis coordinate and delta y-axis coordinate of the right pupil center position, respectively, are derived as follows:

RightPupilCoordinateDeltaX

= (1-2 * gfv_pupil_right_eye_dx_coordinate_sign_flag) * gfv_pupil_right_eye_dx_coordinate_abs * CornersDistance1<< (gfv_pupil_coordinate_precision_factor_minus1 +1)

RightPupilCoordinateDeltaY

= (1-2 * gfv_pupil_right_eye_dy_coordinate_sign_flag) * gfv_pupil_right_eye_dy_coordinate_abs * CornersDistance1<< (gfv_pupil_coordinate_precision_factor_minus1 +1)

gfv_pupil_right_eye_x_coordinate: indicates the normalized value of the x-axis coordinate of the right eye.

gfv_pupil_right_eye_y_coordinate: indicates the normalized value of the y-axis coordinate of the right eye.

The variables RightPupilCoordinateX and RightPupilCoordinateY indicating the x-axis coordinate and y-axis coordinate of the right pupil center position, respectively, are derived as follows:

When gfv_pupil_pred_flag is equal to 0,

RightPupilCoordinateX

= gfv_pupil_right_eye_x_coordinate * CornersDistance1<< (gfv_pupil_coordinate_precision_factor_minus1 +1)+RightCornerReferenceCoordinateX

RightPupilCoordinateY

= gfv_pupil_right_eye_y_coordinate * CornersDistance1<< (gfv_pupil_coordinate_precision_factor_minus1 +1)+RightCornerReferenceCoordinateY

when gfv_pupil_pred_flag is equal to 1,

RightPupilCoordinateX

= RightPupilCoordinateDeltaX+BaseRightPupilPositionX + RightCornerReferenceCoordinateX

RightPupilCoordinateY

= RightPupilCoordinateDeltaY+BaseRightPupilPositionY + RightCornerReferenceCoordinateY

# References

- S. McCarthy, J. Chen, S. Deshpande, M. Hannuksela, Hendry, “Technologies under consideration for future extensions of VSEI (version 3)” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AG2032, January 2024.

- B. Chen, J. Chen, R.-L. Liao, Y. Ye, S. Wang, “AHG 16: Proposed Common Software Tools and Testing Conditions for Generative Face Video Compression,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AG0042, January 2024.

# Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Hisense Group Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).