| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0145-v1 |
| --- | --- |

| Title: | AHG12: Follow up on normative DMVR fix for ECM as in JVET-AG0125 (Parallel friendly use of boundary distortion for DMVR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K.Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

At the last meeting it was suggested to arrange a subjective viewing of JVET-AG0125 to verify its subjective impact. Therefore, a set of sequences from JVET-AH0041 have been tested for the normative solution to address visual issues with DMVR in ECM-12.0.

Objective BDR performance of JVET-AG0125 on top of ECM-12.0 vs ECM-12.0:

|  | Y | U | V |
| --- | --- | --- | --- |
| Procession_3840x2160_60p_420_10 | -0,08% | -0,10% | -0,04% |
| BeachMountain2_3840x2160_30fps_ | -0,06% | -0,08% | -0,28% |
| CrossRoad2s500_3840x2160_50fps_ | -0,02% | -0,03% | -0,10% |
| DrivingPOV3_3840x2160_60fps_10b | 0,05% | -0,08% | 0,00% |
| Average | -0,03% | -0,07% | -0,11% |

Subjectively it is asserted that visual quality can be improved.

Additionally, it is also provided results when enabling the DMVR encoder control in both ECM and VTM11_ANC and the average BDR loss is about 0.76% respectively 0.92%. The encoder control can also improve subjective quality but not as good as the normative solution.

It is proposed to include the normative solution for DMVR to ECM to improve robustness of DMVR in ECM.

# Introduction

DMVR can introduce visual artifacts and to combat that encoder methods have been included to both the VVC reference software and ECM software. Since ECM is exploratory work and not a final standard it is believed to be more appropriate to deal with the issue on both encoder and decoder side.

# Proposal

To have a design which inherently aims to make the motion vector determination of DMVR more robust, it is proposed to introduce a refinement step as a final step after bilateral matching search. The refinement step selects a motion vector pair from the initial motion vector pair and the best bilateral matching candidate based on a boundary distortion metric. The approach is used for blocks with size 16xN, Mx16 and MxN where M and N are equal to 32 or greater, as the DMVR visual artifacts are more likely to appear for those blocks.

The candidate motion vector pair with least total distortion is selected as the refined motion vector pair for the subblock. This is asserted to make the motion selection at the block boundary more robust.

Further details can be found in JVET-AG0125[1].

# Results

## ECM

Objective performance of JVET-AG0125 on top of ECM12-.0 vs ECM-12.0:

|  | Y | U | V |
| --- | --- | --- | --- |
| Procession_3840x2160_60p_420_10 | -0,08% | -0,10% | -0,04% |
| BeachMountain2_3840x2160_30fps_ | -0,06% | -0,08% | -0,28% |
| CrossRoad2s500_3840x2160_50fps_ | -0,02% | -0,03% | -0,10% |
| DrivingPOV3_3840x2160_60fps_10b | 0,05% | -0,08% | 0,00% |
| Average | -0,03% | -0,07% | -0,11% |

Subjectively it is asserted that visual quality can be improved.

Objective performance of DMVR encoder control (DMVREncMvSelect=1) vs ECM-12.0:

|  | Y | U | V |
| --- | --- | --- | --- |
| Procession_3840x2160_60p_420_10 | 1,16% | 1,22% | 1,41% |
| BeachMountain2_3840x2160_30fps_ | 0,41% | 0,46% | 0,55% |
| CrossRoad2s500_3840x2160_50fps_ | 0,63% | 0,63% | 0,63% |
| DrivingPOV3_3840x2160_60fps_10b | 0,74% | 0,82% | 0,88% |
| Average | 0,73% | 0,78% | 0,87% |

Subjectively it is asserted that visual quality can be improved but the normative solution is generally better.

## VTM11_ANC

Objective performance of enabling the DMVR encoder control (DMVREncMvSelect=1) on VTM11_ANC using same QP settings as for VTM11_ANC in JVET-AH0041:

|  | Y | U | V |
| --- | --- | --- | --- |
| Procession_3840x2160_60p_420_10 | 1,43% | 1,54% | 1,63% |
| DrivingPOV3_3840x2160_60fps_10b | 1,49% | 2,09% | 1,50% |
| BeachMountain2_3840x2160_30fps_ | 0,30% | 0,51% | 0,82% |
| CrossRoad2s500_3840x2160_50fps_ | 0,46% | 0,47% | 0,42% |
| Average | 0,92% | 1,15% | 1,09% |

Subjectively it is asserted that visual quality can be improved.

# Conclusion

It is proposed to include the normative method to ECM to make sure that DMVR is robust.

# References

[1] K. Andersson, R. Yu, “AHG12: Parallel friendly use of boundary distortion for DMVR”, JVET-AG0125, Online meeting, January 2024.

[2] M. Wien, “AHG4: AHG meeting report on ECM expert viewing preparations”, JVET-AH0041, Rennes France, April 2024.

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).