| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0148-v1 |
| --- | --- |

| Title: | AHG9/AHG16: On key point coordinates calculation for the generative face video SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kaifa Yang1, Ye-Kui Wang2 Yiling Xu1, Yue Li1 1Shanghai Jiao Tong University, China 2Bytedance Inc., US | Email: | sekiroyyy@sjtu.edu.cn yekui.wang@bytedance.com yl.xu@sjtu.edu.cn yueli983@sjtu.edu.cn |
| Source: | Shanghai Jiao Tong University, Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes on the signalling and calculation of key point coordinates for the generative face video SEI message:

- Remove the syntax element gfv_coordinate_z_max_value_minus1.

- Change the equations for calculation of the key point coordinates to be independent of CroppedWidth, CroppedHeight, and gfv_coordinate_z_max_value_minus1.

# Introduction

In the current text for the generative face video (GFV) SEI message, the calculation of key point coordinates coordinateX[ i ], coordinateY[ i ], and coordinateZ[ i ] involves CroppedWidth, CroppedHeight, and gfv_coordinate_z_max_value_minus1, as can be seen from the following equations:

coordinateX[i] = (1-2 * gfv_coordinate_x_sign_flag[ i ]) * gfv_coordinate_x_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateY[i] = (1-2 * gfv_coordinate_y_sign_flag[ i ]) * gfv_coordinate_y_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateZ[i] = (1-2 * gfv_coordinate_z_sign_flag[ i ]) * gfv_coordinate_z_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

However, according to the current GFV Software, the input coordinates to the GFV network [3][4][5][6] are actually not related to the picture width and height. Furthermore, according to the semantics of gfv_coordinate_x_abs[ i ], gfv_coordinate_y_abs[ i ], gfv_coordinate_z_abs[ i ], they should indicate the normalized absolute value of the x-, y-, and z-axis coordinates of the i-th key point, while the equations above would result in value ranges [0, CroppedWidth], [0, CroppedHeight], and [0, gfv_coordinate_z_max_value_minus1 + 1], respectively, instead of [0, 1], [0, 1], and [0, 1], respectively.

Therefore, the changes as summarized in the abstract are proposed to make the signalling and calculation of the coordinates to be independent of CroppedWidth, CroppedHeight, and gfv_coordinate_z_max_value_minus1.

# Proposal

The proposed syntax and semantics changes are as follows:

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| … |  |
| gfv_coordinate_present_flag | u(1) |
| if( gfv_coordinate_present_flag ) { |  |
| gfv_coordinate_precision_factor_minus1 | ue(v) |
| gfv_num_kp_minus1 | ue(v) |
| gfv_kp_pred_flag | u(1) |
| gfv_coordinate_z_present_flag | u(1) |
| if(gfv_coordinate_z_present_flag ) |  |
| gfv_coordinate_z_max_value_minus1 | ue(v) |
| for( i = 0; i <= num_kp_minus1; i++ ) { |  |
| ... |  |
| … |  |
| } |  |

...

gfv_coordinate_z_present_flag equal to 1 indicates that z-axis coordinate information of the keypoints is present. coordinate_z_present_flag equal to 0 indicates that the z-axis coordinate information of the keypoints is not present.

gfv_coordinate_z_max_value_minus1 plus 1 indicates the maximum absolute value of z-axis coordinates of keypoints.

gfv_coordinate_x_abs[ i ] indicates the normalized absolute value of the x-axis coordinate of the i-th keypoint.

...

gfv_coordinate_dz_sign_flag[ i ] specifies the sign of the difference value of the z-axis coordinate of the i-th key point. When gfv_coordinate_dz_sign_flag[ i ] is not present, it is inferred to be equal to 0.

The variables coordinateDeltaX[ i ], coordinateDeltaY[ i ] and coordinateDeltaZ[ i ] indicating the delta x-axis coordinate, delta y-axis coordinate and delta z-axis coordinate of the i-th keypoint, respectively, are derived as follows:

coordinateDeltaX[ i ] = (1-2 * gfv_coordinate_dx_sign_flag[ i ]) * gfv_coordinate_dx_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateDeltaY[i] = (1-2 * gfv_coordinate_dy_sign_flag[ i ]) * gfv_coordinate_dy_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateDeltaZ[i] = (1-2 * coordinate_dz_sign_flag[ i ]) * gfv_coordinate_dz_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

The variables coordinateX[ i ], coordinateY[ i ] and coordinateZ[ i ] indicating the x-axis coordinate, y-axis coordinate and z-axis coordinate of the i-th keypoint, respectively, are derived as follows:

When gfv_kp_pred_flag is equal to 0,

coordinateX[ i ] = (1-2 * gfv_coordinate_x_sign_flag[ i ]) * gfv_coordinate_x_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateY[i] = (1-2 * gfv_coordinate_y_sign_flag[ i ]) * gfv_coordinate_y_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateZ[i] = (1-2 * gfv_coordinate_z_sign_flag[ i ]) * gfv_coordinate_z_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

...

# References

- M. M. Hannuksela, F. Cricri, H. Zhang (Nokia), “AHG9: On the generative face video SEI message”, JVET-AG0087.

- J. Chen, B. Chen, Y. Ye (Alibaba), S. Yin, S. Wang (CityU), S. McCarthy, P. Yin, G.-M. Su, A. K. Choudhury, W. Husak, G. J. Sullivan (Dolby), “AHG9/AHG16: Common text for proposed generative face video SEI message”, JVET-AG0203.

- Feng D, Huang Y, Zhang Y, et al. A generative compression framework for low bandwidth video conference[C]//2021 IEEE International Conference on Multimedia & Expo Workshops (ICMEW). IEEE, 2021: 1-6.

- Siarohin A, Lathuilière S, Tulyakov S, et al. First order motion model for image animation[J]. Advances in Neural Information Processing Systems, 2019, 32.

- Siarohin A, Woodford O J, Ren J, et al. Motion representations for articulated animation[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2021: 13653-13662.

- Wang T C, Mallya A, Liu M Y. One-shot free-view neural talking-head synthesis for video conferencing[C]//Proceedings of the IEEE/CVF conference on computer vision and pattern recognition. 2021: 10039-10049.

# Patent rights declaration(s)

Shanghai Jiao Tong University and Bytedance Inc. does not have any current or pending patent rights relating to the proposed technology described in this contribution.