| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0150-v1 |
| --- | --- |

| Title: | AHG9: On the proposed MPII SEI message for non-parallel MPI layers |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li1, Ye-Kui Wang2, Yiling Xu1, Kaifa Yang1 1Shanghai Jiao Tong University, China 2Bytedance Inc., US | Email: | yueli983@sjtu.edu.cn yekui.wang@bytedance.com yl.xu@sjtu.edu.cn sekiroyyy@sjtu.edu.cn |
| Source: | Shanghai Jiao Tong University, Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution provides a comparison of the rendering complexity between traditional MPI and MPI with non-parallel layers. Two demos showing the rendering effects of the two methods are provided in the attachments. It is proposed to add the support of non-parallel layers into the MPII SEI message.

# Introduction

At the JVET-AG meeting, JVET-AG0232 [1] proposed an extension to the multiplane image information (MPII) SEI message, such that the multiplane image layers can be either parallel or non-parallel to each other. Further study was requested to compare differences in rendering complexity vs. parallel planes and to justify the need for standardization for this variant of MPI, including providing information about open-source software for rendering MPI non-parallel planes.

Therefore, we investigated relevant open source codes and did a complexity comparison. Furthermore, we created two demos showing the rendering effects of the two methods (Subsection 2.2 includes a decription of what to see in the demos).

## Summary of MPII SEI message extension for non-parallel layers

As noted in JVET-AG0232, the current MPII SEI message only supports MPI with fronto-parallel layers which is prone to discretization artifacts in novel views. The syntax proposed in JVET-AG0232 is shown in Table 1. Besides the information from JVET-AF2032 [2], the MPII SEI message extension also conveys normal vector to support MPI with non-parallel layers.

Table 1 – MPII SEI message syntax extension from JVET-AG0232

| multiplane_image_information( payloadSize ) { | Descriptor |
| --- | --- |
| mpii_num_layers_minus1 | ue(v) |
| mpii_layers_parallel_flag | u(1) |
| mpii_layer_depth_equal_distance_flag | u(1) |
| if( mpii_layer_depth_equal_distance_flag ) { |  |
| depth_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| depth_rep_info_element( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| } else |  |
| for( i = 0; i <= mpii_num_layers_minus1; i++ ) |  |
| depth_rep_info_element( ZSign[ i ], ZExp[ i ], ZMantissa[ i ], ZManLen[ i ] ) |  |
| if( !mpii_layers_parallel_flag ) |  |
| normal_vector_len_idc | u(2) |
| for( i = 0; i <= mpii_num_layers_minus1; i++ ) { |  |
| fronto_parallel_flag[ i ] | u(1) |
| if( !fronto_parallel_flag[ i ] ) { |  |
| normal_vector_x[ i ] | i(v) |
| normal_vector_y[ i ] | i(v) |
| } |  |
| } |  |
| mpii_texture_opacity_interleave_flag | u(1) |
| if( mpii_texture_opacity_interleave_flag = = 0 ) |  |
| mpii_texture_opacity_arrangement_flag /* 0: Top-and-Bottom, 1: Side-by-Side */ | u(1) |
| mpii_picture_num_layers_in_height_minus1 | ue(v) |
| } |  |

# Findings

## Open-source codes for MPI with non-parallel layers

An open source code [4] for rendering MPI with non-parallel layers was found associated with the paper  “Structural Multiplane Image: Bridging Neural View Synthesis and 3D Reconstruction” [3], which was published in CVPR 2023. We found this paper has been cited by two other papers [5][6]. The open source code provides the generation of MPI with non-parallel layers based on plane segmentation using either single view image or multi-view images.

## Comparison of the rendering complexity and visual quality between traditional MPI and MPI with non-parallel layers

Different from the standard MPI having a global back-to-front rendering order of layers for each pixel due to the fronto-parallel layers, the MPI with non-parallel layers has different rendering orders for pixels because layers could intersect with each other as shown in Figure 1. Each pixel has its own rendering order. First, the depth value for each pixel on each layer is calculated, and then we rearrange the RGBα images with the depth order for each pixel. Finally, we apply the standard alpha composition with the new order to render the image.

Figure 1 MPI with non-parallel layers[3]

We selected two scenes from ScanNet [7] to compare rendering time and rendering quality in novel views with multi-view images as input. In the two scenes, rendering the standard MPI with 32 layers (the size for each layer is 384x256) in a novel view costs about 6ms, and the time for MPI with non-parallel layers is about 12ms. Figure 2 shows two frames from the demo videos, and each frame represents the rendering results of the standard MPI (left) and MPI with non-parallel layers (right) in different perspectives. The visual quality in novel views is much better with MPI with non-parallel layers, especially in the plane area such as the wall where severe discretization artifacts appear with standard MPI.

Figure 2 Rendering quality in novel views for standard MPI (left) and MPI with non-parallel layers (right)

# References

- Y. Li,Y.-K. Wang, Y. Xu, K. Yang, “AHG9: Support of non-parallel MPI layers in the MPII SEI message (draft 2)” , JVET-AG0232, Jan. 2024

- M. M. Hannuksela, J. Chen, S. Deshpande, S. McCarthy, “Technologies under consideration for future extensions of VSEI (draft 2)”, JVET-AF2032, Sept. 2023

- Mingfang Zhang, Jinglu Wang, Xiao Li, Yifei Huang, Yoichi Sato, and Yan Lu, “Structural Multiplane Image: Bridging Neural View Synthesis and 3D Reconstruction”, CVPR 2023

- Structural-MPI code: https://github.com/mf-zhang/Structural-MPI

- Wang Y, Lira W, Wang W, et al, “Slice3D: Multi-Slice, Occlusion-Revealing, Single View 3D Reconstruction”, arXiv 2023

- Lee Y C, Zhang Z, Blackburn-Matzen K, et al, “Fast View Synthesis of Casual Videos”, arXiv 2023

- Angela Dai, Angel X. Chang, Manolis Savva, Maciej Halber, Thomas Funkhouser, and Matthias Nießner, “Scannet:Richly-annotated 3d reconstructions of indoor scenes”, CVPR 2017

# Patent rights declaration(s)

Shanghai Jiao Tong University and Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex A
Proposed MPII SEI message syntax and semantics from JVET-AG0232

The text changes are provided below with modification highlighted in yellow.

| multiplane_image_information( payloadSize ) { | Descriptor |
| --- | --- |
| mpii_num_layers_minus1 | ue(v) |
| mpii_layers_parallel_flag | u(1) |
| mpii_layer_depth_equal_distance_flag | u(1) |
| if( mpii_layer_depth_equal_distance_flag ) { |  |
| depth_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| depth_rep_info_element( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| } else |  |
| for( i = 0; i <= mpii_num_layers_minus1; i++ ) |  |
| depth_rep_info_element( ZSign[ i ], ZExp[ i ], ZMantissa[ i ], ZManLen[ i ] ) |  |
| if( !mpii_layers_parallel_flag ) |  |
| normal_vector_len_idc | u(2) |
| for( i = 0; i <= mpii_num_layers_minus1; i++ ) { |  |
| fronto_parallel_flag[ i ] | u(1) |
| if( !fronto_parallel_flag[ i ] ) { |  |
| normal_vector_x[ i ] | i(v) |
| normal_vector_y[ i ] | i(v) |
| } |  |
| } |  |
| mpii_texture_opacity_interleave_flag | u(1) |
| if( mpii_texture_opacity_interleave_flag = = 0 ) |  |
| mpii_texture_opacity_arrangement_flag /* 0: Top-and-Bottom, 1: Side-by-Side */ | u(1) |
| mpii_picture_num_layers_in_height_minus1 | ue(v) |
| } |  |

The multiplane image information (MPII) SEI message specifies the multiplane image (MPI) scene representation information that may be used for view synthesis.

When an MPII SEI message is present in any AU of a CLVS, an MPII SEI message shall be present in the first AU of the CLVS and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an MPII SEI message is output that follows the current picture in output order.

NOTE 1 – This SEI message can work together with the multiview acquisition information SEI message for view synthesis. The multiview acquisition information SEI message specifies the intrinsic and extrinsic parameters for current camera view. When multiple views are available, the reconstructed novel views can be rendered from multiplane images of nearby views.

Use of this SEI message requires the definition of the following variables:

–	Cropped decoded output picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	A cropped decoded picture array decPicCurr0[ cIdx ][ x ][ y ], with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..( cIdx = = 0 ) ? CroppedWidth : CroppedWidth / SubWidthC − 1, y = 0..( cIdx = = 0 ) ? CroppedHeight : CroppedHeight / SubHeightC − 1.

–	In output order a temporally following cropped decoded picture array decPicCurr1[ cIdx ][ x ][ y ], with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..( cIdx = = 0 ) ? CroppedWidth : CroppedWidth / SubWidthC − 1, y = 0..( cIdx = = 0 ) ? CroppedHeight : CroppedHeight / SubHeightC − 1.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

mpii_num_layers_minus1 plus 1 specifies the number of texture and opacity layers for the MPI representation.

mpii_layers_parallel_flag equal to 1 indicates that all layers are parallel. mpii_layers_parallel_flag equal to 0 indicates that all layers may or may not be parallel.

mpii_layer_depth_equal_distance_flag equal to 1 indicates that equal distance is used to generate MPI layers and depth parameter for each layer Z[ i ] can be derive using nearest depth value ZNear and farthest depth value ZFar.

The depth value for i-th MPI layer, Z[ i ], is derived as follows:

Z[ i ] = i * ( ZFar − Znear ) ÷ (mpi_num_layers_minus1 ) + ZNear	(xx)

mpii_layer_depth_equal_distance_flag equal to 0 indicates that the depth information for each layer follows next in the SEI message.

The variables in the x column of Table xx are derived from the respective variables in the s, e, n and v columns of Table xx as follows:

–	If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1)s * 2( e − 31 ) * ( 1 + n ÷ 2v ).

–	Otherwise (e is equal to 0), x is set equal to ( −1)s * 2−( 30 + v ) * n.

NOTE 2 – The above specification is similar to that found in IEC 60559:1989.

Table xx – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| Z[ i ] | ZSign[ i ] | ZExp | ZMantissa | ZManLen |

NOTE 3 – In some applications, disparity is used instead of depth (the disparity value D and depth value Z relationship is D = 1 ÷ Z). Corresponding to Equation (xx), the disparity value for the i-th MPI layer is D[ i ] = i * ( DFar − Dnear ) ÷ (mpi_num_layers_minus1 ) + DNear.

normal_vector_len_idc specifies the number of bits, denoted by the variable lenInbits, used for coding of the normal_vector_x[ i ] and normal_vector_y[ i ] syntax elements. The value of lenInbits is derived to be equal to 2normal_vector_len_idc + 2.

fronto_parallel_flag[ i ] equal to 1 indicates that the i-th MPI layer is parallel to the image plane and the normal vector of the i-th MPI layer is (0.0, 0.0, 1.0). fronto_parallel_flag[ i ] equal to 0 indicates that the i-th MPI layer is not parallel to the image plane and the normal vector of the i-th layer can be derived from normal_vector_x[ i ] and normal_vector_y[ i ].

normal_vector_x[ i ] and normal_vector_y[ i ] are used to specify the x and y components, respectively, of the normal vector of the i-th MPI layer. normal_vector_x[ i ] and normal_vector_y[ i ] are coded using lenInbits bits. The values of normal_vector_x[ i ] and normal_vector_y[ i ] shall be in the range of −2lenInbits − 2 to 2lenInbits − 2, inclusive.

When mpii_layers_parallel_flag is equal to 0, the three components of the normal vector of the i-th MPI layer, normalVectorX[ i ], normalVectorY[ i ], and normalVectorZ[ i ], are derived as follows:

if( fronto_parallel_flag[ i ] ) {
	normalVectorX[ i ] = 0.0
	normalVectorY[ i ] = 0.0
	normalVectorZ[ i ] = 1.0
} else {
	normalVectorX[ i ] = normal_vector_x[ i ] ÷ 2lenInbits − 2
	normalVectorY[ i ] = normal_vector_y[ i ] ÷ 2lenInbits − 2
	normalVectorZ[ i ] = Sqrt( 1 − normalVectorX[ i ]2 − normalVectorY[ i ]2 )
}

mpii_texture_opacity_interleave_flag equal to 1 indicates decoded output pictures correspond to temporally interleaved texture and opacity constituent pictures in output order as illustrated in Figure XX. mpii_texture_opacity_interleave_flag equal to 0 indicates decoded output pictures correspond to spatially packed texture and opacity constituent pictures as illustrated in Figures XX and XX.

mpii_texture_opacity_arrangement_flag equal to 0 indicates decoded output pictures represent texture and opacity constituent pictures in a top-bottom packing arrangement as illustrated in Figure XX. mpii_texture_opacity_arrangement_flag equal to 1 indicates decoded output pictures represent texture and opacity constituent pictures in a side-by-side packing arrangement as illustrated in Figure XX.

For each specified picture packing arrangement scheme, there are two constituent pictures that are referred to as picture 0 and picture 1. When mpii_texture_opacity_interleave_flag is equal to 0, the constituent picture associated with the upper-left sample of the decoded picture is considered to be constituent picture 0 and the other constituent picture is considered to be constituent picture 1. When mpii_texture_opacity_interleave_flag is equal to 1, the first decoded picture in the current CLVS is constituent picture 0 and the next decoded picture in output order is constituent picture 1 and the display time of the constituent picture 0 should be delayed to coincide with the display time of constituent picture 1. The two constituent pictures form the spatially packed texture and opacity picture of a MPI, with picture 0 being associated with the spatially packed texture picture and picture 1 being associated with the spatially packed opacity picture.

mpii_picture_num_layers_in_height_minus1 plus 1 specifies the number of spatially packed layers in height for picture 0 and picture 1. The variable hLayers is set equal to mpii_picture_num_layers_in_height_minus1 + 1 and the variable wLayers is set equal to (mpii_num_layers_minus1 + 1) / hLayers.

Let variable fWidth and fHeight specify the width and height of picture 0 and picture 1, respectively, and are derived as follows:

–	If mpii_texture_opacity_interleave_flag is equal to 1, the following applies:

fWidth = CroppedWidth

fHeight = CroppedHeight

–	Otherwise (mpii_texture_opacity_interleave_flag is equal to 0)

–	If mpii_texture_opacity_arrangement_flag is equal to 0, the following applies:

fWidth = CroppedWidth , fHeight = CroppedHeight / 2

–	Otherwise (mpii_texture_opacity_arrangement_flag is equal to 1), the following applies:

fWidth = CroppedWidth / 2 , fHeight = CroppedHeight

Let variable cWidth = fWidth / subWidthC and variable cHeight = fHeight / subHeightC.

Let array picture0[ cIdx ][ x ][ y ] specify samples in picture 0 and array picture1[ cIdx ][ x ][ y ] specify samples in picture 1, with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..( cIdx = = 0 ) ? fWidth: cWidth − 1, y = 0..( cIdx = = 0 ) ? fHeight : cHeight − 1 and are derived as follows:

–	If mpii_texture_opacity_interleave_flag is equal to 1, the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr1[ cIdx ][ x ][ y ]

–	Otherwise (mpii_texture_opacity_interleave_flag is equal to 0)

–	Let variable cW = ( cIdx = = 0 )? fWidth : cWidth

–	Let variable cH = ( cIdx = = 0 )? fHeight : cHeight

–	If mpii_texture_opacity_arrangement_flag is equal to 0, the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y + cH ]

–	Otherwise (mpii_texture_opacity_arrangement_flag is equal to 1), the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x + cW ][ y ]

Let variable layerWidth and layerHeight specify the width and height for decoded MPI layer, respectively. The variables are derived as follows:

layerWidth = fWidth / wLayers

layerHeight = fHeight / hLayers

The reconstruction of MPI process is described as follows:

The outputs of this process are:

–	a 4D MPI texture layer array recTextureLayer[ i ][ cIdx ][ w ][ h ] with i = 0..mpii_num_layers_minus1, cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, w = 0..( cIdx = = 0 ) ? layerWidth : layerWidth / SubWidthC − 1, and h = 0..( cIdx = = 0 ) ? layerHeight : layerHeight / SubHeightC − 1.

–	a 3D MPI opacity layer array recOpacityLayer[ i ][ w ][ h ] with i = 0..mpii_num_layers_minus1, x = 0..layerWidth − 1, and y = 0..layerHeight − 1.

The array recTextureLayer and array recOpacityLayer are derived as follows:

for( i = 0; i  <=  mpii_num_layers_minus1; i++ ) {
	k = i % wLayers
	m = ( i − k ) / hLayers
	for( cIdx = 0; cIdx < ChromaFormatIdc = = 0 ) ? 1 : 3; cIdx++ )
		for( h = 0; h < ( cIdx = = 0 ) ? layerHeight : layerHeight / SubHeightC ; h++ )
			for( w = 0; w < ( cIdx = = 0 ) ? layerWidth : layerWidth / SubWidthC ; w++ ) {
				u = k * ( cIdx  = =  0 ) ? layerWidth : layerWidth / SubWidthC + w
				v = m * ( cIdx  = =  0 ) ? layerHeight : layerHeight / SubHeightC + h
				recTextureLayer[ i ][ cIdx ][ w ][ h ] = picture0[ cIdx ][ u ][ v ]
			}
	for( h = 0; h < layerHeight; h++ )
		for( w = 0; w < layerWidth; w++ )
			recOpacityLayer[ i ][ w ][ h ] = picture1[ 0 ][ k * layerWidth + w ][ m * layerHeight + h ]
}