| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0154-v1 |
| --- | --- |

| Title: | Carriage of depth and alpha maps as HEVC single-layer bitstreams |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Emmanouil Potetsianakis Evangelos Alexiou Ramin Ghaznavi-Youvalari Mohsen Abdoli Alexandre Tissier Mary-Luc Champel | Tel: Email: | thomase@xiaomi.com emmanouil@xiaomi.com alexiou@xiaomi.com ramin.youvalari@xiaomi.com mabdoli@xiaomi.com alexandret@xiaomi.com champelmaryluc@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

It is proposed to enable the carriage of alpha planes and depth pictures as a HEVC single layer bitstream along with the corresponding SEI messages. It is asserted that the existing SEI messages can be reused provided the semantics of the messages permit the presence of the SEI in a single-layer context. In addition, for the mapping of the alpha planes and depth pictures with the internal picture format, it is proposed to leverage existing colour primaries and matrix coefficients defined in the CICP specification.

# Introduction

The HEVC multi-layer extension enables the carriage of alpha planes and depth pictures as auxiliary pictures in dedicated layers. Those auxiliary layers must by definition be associated with a layer carrying the primary pictures, i.e. non-auxiliar pictures. In addition, the HEVC specifications define SEI message to describe properties of the alpha planes and depth pictures. Those SEI messages are currently constrained to be present only in layers that are auxiliary pictures of the type corresponding to the SEI message, i.e. AUX_ALPHA or AUX_DEPTH.

At JVET #33, two contributions, JVET-AG0144 [1] and JVET-AG0308 [2], were submitted to allow a single-layer design in HEVC to carry alpha planes and depth pictures which received the general comment:

It is agreed that this is a desirable functionality (to be supported with AVC, HEVC and VVC), but further study appears necessary to understand what is the best way (e.g., JVET-AG0308 has the same target) of achieving it, and which changes would be necessary for the different specs.

As a result, this contribution consolidates a solution based on both contributions to allow the carriage of alpha planes and depth pictures as a HEVC single layer bitstream along with the corresponding SEI messages. In addition, solution also conveys the mapping of the alpha planes and depth pictures with the internal picture format by leveraging existing colour primaries and matrix coefficients defined in the CICP specification.

# Study of comments received during the previous JVET meeting

At JVET #33, the JVET-AG0144 received the following comment.

Comment

It was commented by another expert that in principle this would already be possible by sending a depth layer without a base layer using Main 10 profile with independent layer decoding flag (and rewriting layer id in NAL header).

Answer

Assuming rewriting operations are possible on a given receiver (e.g. HMD, smartphone), and for a given application context (sometimes with encrypted content), those operations are unnecessarily increasing the motion to photo delay which is detrimental for split rendering use cases. In addition, after the hypothetical rewriting of the layer id with the value 0, this doesn’t change the issue that the depth and alpha SEI messages would be present in a primary layer which is not yet permitted by the current HEVC specification.

At JVET #33, the JVET-AG038 received the following comment:

Comment

It needs to be clarified if the existing SEI message for depth maps can be used in single-layer streams in AVC and HEVC, and how it could be associated with the depth map, or if an extension would be necessary.

Answer

The same SEI messages can be reused provided the semantics of the messages permit the presence of the SEI in a single-layer context, i.e. the SEI message can be present in a layer whose id value is 0.

# Proposed solutions

## Overview

In order to enable the above use case, this contribution presents possible solutions for each design goal:

- Design goal: Information indicating that the bitstream carries a depth map sequence or an alpha map sequence.

  - Solution: Defining code points in colour primaries for depth maps and alpha maps as colour primaries in the VUI (cf. 3.2.1).

- Design goal: Information indicating the mapping used between the depth/alpha map (mono-channel) and the luma and chroma components.

  - Solution: Defining new code points in matrix coefficients for mono-channel (luma only) in CICP with signalling in the VUI (cf. 3.2.2).

- Design goal: Information specifying how to interpret the sample values from the decoded pictures to reconstruct the depth/alpha maps.

  - Solution: Reuse of existing HEVC SEI messages with modifications be allowed in a single layer context (cf. 3.2.3 and 3.2.4).

## Text changes

### Change #1 - CICP: Depth and alpha maps as colour primaries

colour_primaries indicates either, when the image is a texture image, the chromaticity coordinates of the source primaries as specified in Table E.3 in terms of the CIE 1931 definition of x and y as specified in ISO 11664-1 or, when the image is not a texture image the type of the image.

When the colour_primaries syntax element is not present, the value of colour_primaries is inferred to be equal to 2 (the chromaticity is unspecified or is determined by the application). Values of colour_primaries that are identified as reserved in Table E.3 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders shall interpret reserved values of colour_primaries as equivalent to the value 2.

Table E.3 — Colour primaries interpretation using colour_primaries syntax element

| Value | Primaries |  |  |  | Informative remark |
| --- | --- | --- | --- | --- | --- |
| 0 | Reserved |  |  |  | For future use by ITU-T ¦ ISO/IEC |
| 1 | primary | x | y |  | Rec. ITU-R BT.709-6 Rec. ITU-R BT.1361-0 conventional colour gamut system and extended colour gamut system (historical) IEC 61966-2-1 sRGB or sYCC IEC 61966-2-4 SMPTE RP 177 (1993) Annex B |
|  | green | 0.300 | 0.600 |  |  |
|  | blue | 0.150 | 0.060 |  |  |
|  | red | 0.640 | 0.330 |  |  |
|  | white D65 | 0.312 7 | 0.329 0 |  |  |
| … |  |  |  |  |  |
| 22 | primary | x |  | y | EBU Tech. 3213-E (1975) |
|  | green | 0.295 |  | 0.605 |  |
|  | blue | 0.155 |  | 0.077 |  |
|  | red | 0.630 |  | 0.340 |  |
|  | white D65 | 0.312 7 |  | 0.329 0 |  |
| 23..127 | Reserved |  |  |  | For future use by ITU-T ¦ ISO/IEC |
| 128 | Alpha map |  |  |  | This signal is an alpha map |
| 129 | Depth map |  |  |  | This signal is a depth picture |
| 130..255 | Reserved |  |  |  | For future use by ITU-T ¦ ISO/IEC |

### Change #2 - CICP: Monochromatic matrix coefficients

Table E.5 — Matrix coefficients interpretation using the matrix_coeffs syntax element

| Value | Matrix | Informative remark |
| --- | --- | --- |
| 0 | Identity | The identity matrix. Typically used for GBR (often referred to as RGB); however, may also be used for YZX (often referred to as XYZ) IEC 61966-2-1 sRGB SMPTE ST 428-1 (2006) See Formulae (E-31) to (E-33) |
| 1 | KR = 0.212 6; KB = 0.072 2 | ITU-R Rec. BT.709-6 ITU-R Rec. BT.1361-0 conventional colour gamut system and extended colour gamut system (historical) IEC 61966-2-1 sYCC IEC 61966-2-4 xvYCC709 SMPTE RP 177 (1993) Annex B See Formulae (E-28) to (E-30) |
| … |  |  |
| 14 | ICTCP | Rec. ITU-R BT.2100-2 ICTCP See Formulae (E-62) to (E-64) for transfer_characteristics value 16 (PQ) See Formulae (E-65) to (E-67) for transfer_characteristics value 18 (HLG) |
| 15 | IPT-C2 | Society of Motion Picture and Television Engineers ST 2128 See Formulae (E-75) to (E-77) |
| 16 | YCgCo-Re | See Formulae (E-41) to (E-55) |
| 17 | YCgCo-Ro | See Formulae (E-41) to (E-55) |
| 18 | 1st colour component | A monochromatic representation with only the first colour component in the original representation present in the coded image. Typically used for Green only; however, may also be used for the Y tristimulus value (luminance), when the colour primaries are set to XYΖ, or to indicate that the representation contains only the first component of an image not representing texture. |
| 19 | 2nd colour component | A monochromatic representation with only the second colour component in the original representation present in the coded image. Typically used for Red only; however, may also be used for the X tristimulus value, when the colour primaries are set to XYΖ, or to indicate that the representation contains only the second component of an image not representing texture. |
| 20 | 3rd colour component | A monochromatic representation with only the third colour component in the original representation present in the coded image. Typically used for Blue only; however, may also be used for the Z tristimulus value, when the colour primaries are set to XYΖ, or to indicate that the representation contains only the third component of an image not representing texture. |
| 21 | Luma only KR = 0.2126; KB = 0.0722 | A monochromatic representation of the luma component of the Rec. ITU-R BT.709-6 representation. See Formula (E-28) |
| 22 | Luma only KR = 0.2627; KB = 0.0593 | A monochromatic representation of the luma component of the Rec. ITU-R BT.2020-2 non-constant luminance representation. See Formula (E-28) |
| 23 | I only - ICTCP | A monochromatic representation of the I component of ICTCP representation. See Formula (E-75) |
| 24..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

### Change #3 - HEVC: Allowing alpha channel information SEI message in a primary layer in HEVC

Summary of change #3:

- Defining an alpha picture as either an auxiliary picture of type AUX_ALPHA or a non-auxiliary picture representing an alpha map.

- The associated primary picture of an alpha picture can be defined by external means.

- F.14.3.8 Alpha channel information SEI message semantics

The alpha channel information SEI message provides information about alpha channel picture sample values and post-processing, an alpha blending process, applied to the decoded alpha planes coded in auxiliary pictures of type AUX_ALPHA or to alpha maps coded in non-auxiliary pictures, and one or more associated primary pictures. The alpha blending process is a process not specified by this document, in which an auxiliary alpha coded picture is used in combination with a primary coded picture and with other data not specified by this document in the display process. In an alpha blending process, the samples of an auxiliary alpha coded picture are interpreted as indications of the degree of opacity (or, equivalently, the degrees of transparency) associated with the corresponding luma samples of the primary coded picture.

For an auxiliary picture with nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] equal to AUX_ALPHA, an associated primary picture, if any, is a picture in the same access unit having AuxId[ nuhLayerIdB ] equal to 0 such that ScalabilityId[ LayerIdxInVps[ nuhLayerIdA ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ nuhLayerIdB ] ][ j ] for all values of j in the range of 0 to 2, inclusive, and 4 to 15, inclusive.

For a picture with nuh_layer_id equal to 0 and representing an alpha map, its associated primary picture, if any, is defined by external means.

For the remainder of this clause, an alpha picture refers to either an auxiliary picture of type AUX_ALPHA or a non-auxiliary picture representing an alpha map.

When an access unit contains an auxiliary alpha picture picA with nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] equal to AUX_ALPHA, the alpha channel picture sample values of picA persist in output order until one or more of the following conditions are true:

—	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

—	A CLVS containing the auxiliary alpha picture picA ends.

—	The bitstream ends.

—	If defined, a CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the alpha channel information SEI message applies.

alpha_channel_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous alpha channel information SEI message in output order that applies to the current layer. alpha_channel_cancel_flag equal to 0 indicates that alpha channel information follows.

Let currPic be the picture that the alpha channel information SEI message is associated with. The semantics of alpha channel information SEI message persist for the current layer in output order until one or more of the following conditions are true:

—	A new CLVS of the current layer begins.

—	The bitstream ends.

—	A picture picB with nuh_layer_id equal to targetLayerId in an access unit containing an alpha channel information SEI message with nuh_layer_id equal to targetLayerId is output having PicOrderCnt( picB ) greater than PicOrderCnt( currPic ), where PicOrderCnt( picB ) and PicOrderCnt( currPic ) are the PicOrderCntVal values of picB and currPic, respectively, immediately after the invocation of the decoding process for picture order count for picB.

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the decoded auxiliary alpha picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the decoded auxiliary alpha picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. Values greater than 2 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2.

alpha_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the auxiliary alpha picture. alpha_channel_bit_depth_minus8 shall be in the range 0 to 7 inclusive. When an alpha picture is an auxiliary picture, alpha_channel_bit_depth_minus8 shall be equal to bit_depth_luma_minus8 of the associated primary picture.

alpha_transparent_value specifies the interpretation sample value of a decoded auxiliary alpha picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered transparent for purposes of alpha blending. The number of bits used for the representation of the alpha_transparent_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_opaque_value specifies the interpretation sample value of a decoded auxiliary alpha picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered opaque for purposes of alpha blending. The number of bits used for the representation of the alpha_opaque_value syntax element is alpha_channel_bit_depth_minus8 + 9.

A value of alpha_opaque_value that is equal to alpha_transparent_value indicates that the auxiliary alpha coded picture is not intended for alpha blending purposes.

NOTE 1	For alpha blending purposes, alpha_opaque_value can be greater than alpha_transparent_value or it can be less than or equal to alpha_transparent_value.

alpha_channel_incr_flag equal to 0 indicates that the interpretation sample value for each decoded auxiliary alpha picture luma sample value is equal to the decoded auxiliary alpha picture sample value for purposes of alpha blending. alpha_channel_incr_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary alpha picture samples, any auxiliary alpha picture luma sample value that is greater than Min( alpha_opaque_value, alpha_transparent_value ) should be increased by one to obtain the interpretation sample value for the auxiliary alpha picture sample, and any auxiliary alpha picture luma sample value that is less than or equal to Min( alpha_opaque_value, alpha_transparent_value ) should be used without alteration as the interpretation sample value for the decoded auxiliary alpha picture sample value.

When alpha_transparent_value is equal to alpha_opaque_value or Log2( Abs( alpha_opaque_value − alpha_transparent_value ) ) does not have an integer value, alpha_channel_incr_flag shall be equal to 0.

alpha_channel_clip_flag equal to 0 indicates that no clipping operation is applied to obtain the interpretation sample values of the decoded auxiliary alpha picture. alpha_channel_clip_flag equal to 1 indicates that the interpretation sample values of the decoded auxiliary alpha picture are altered according to the clipping process described by the alpha_channel_clip_type_flag syntax element.

alpha_channel_clip_type_flag equal to 0 indicates that, for purposes of alpha blending, after decoding the auxiliary alpha picture samples, any auxiliary alpha picture luma sample that is greater than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to Max( alpha_transparent_value, alpha_opaque_value ) to obtain the interpretation sample value for the auxiliary alpha picture luma sample, and any auxiliary alpha picture luma sample that is less or equal than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to Min( alpha_transparent_value, alpha_opaque_value ) to obtain the interpretation sample value for the auxiliary alpha picture luma sample. alpha_channel_clip_type_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary alpha picture samples, any auxiliary alpha picture luma sample that is greater than Max( alpha_transparent_value, alpha_opaque_value ) is set equal to Max( alpha_transparent_value, alpha_opaque_value ) to obtain the interpretation sample value for the auxiliary alpha picture luma sample, and any auxiliary alpha picture luma sample that is less than or equal to Min( alpha_transparent_value, alpha_opaque_value ) is set equal to Min( alpha_transparent_value, alpha_opaque_value ) to obtain the interpretation sample value for the auxiliary alpha picture luma sample.

When both alpha_channel_incr_flag and alpha_channel_clip_flag are equal to one, the clipping operation specified by alpha_channel_clip_type_flag should be applied first followed by the alteration specified by alpha_channel_incr_flag to obtain the interpretation sample value for the auxiliary alpha picture luma sample.

Alpha blending composition is ordinarily performed with a background picture B, a foreground picture F, and a decoded auxiliary alpha picture A, all of the same size. Assume for purposes of example illustration that the chroma resolutions of B and F, if different from the luma resolution, have been upsampled to the same resolution as the luma. Denote corresponding samples of B, F and A by b, f and a, respectively. Denote luma and chroma samples by subscripts Y, Cb and Cr. Each component, e.g., Y, is also assumed for purposes of example illustration to have the same bit depth in each of the pictures B and F. However, different components, e.g., Y and Cb, can have different bit depths in this example. The samples of pictures B and F may alternatively represent green, blue and red component values (see E.3.1), although the formulae use the subscripts Y, Cb and Cr for the three components.

Define the variables alphaRange, alphaFwt and alphaBwt for each luma sample aY of the auxiliary alpha picture A as follows:

alphaRange = Abs( alpha_opaque_value − alpha_transparent_value )	(F-95)

alphaFwt = Abs( ay − alpha_transparent_value )	(F-96)

alphaBwt = Abs( ay − alpha_opaque_value )		(F-97)

A picture format that is often useful for editing or direct viewing, and that is commonly used, is called pre-multiplied-black video. Pre-multiplied-black video has the characteristic that the decoded picture F will appear the same regardless of whether it is viewed directly without alpha blending composition or is alpha blended with a black background. The use of alpha_channel_use_idc equal to 0 corresponds with source video that is not pre-multiplied-black video, and the use of alpha_channel_use_idc equal to 1 corresponds with source video that is pre-multiplied-black video.

An example of operation of the alpha blending composition process to produce a displayed picture D with sample values d from the pictures B and F is as follows:

—	If alpha_channel_use_idc is equal to 0, the samples d of the displayed picture D are calculated as follows:

dY = ( alphaFwt * fY + alphaBwt * bY + alphaRange / 2 ) / alphaRange	(F-98)

dCb = ( alphaFwt * fCb + alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(F-99)

dCr = ( alphaFwt * fCr + alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(F-100)

—	Otherwise when alpha_channel_use_idc is equal to 1, the samples d of the displayed picture D are calculated as follows:

dY  = fY + ( alphaBwt * bY + alphaRange / 2 ) / alphaRange	(F-101)

dCb = fCb + ( alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(F-102)

dCr = fCr + ( alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(F-103)

NOTE 2	In this case, it is expected that the encoder produces its pre-multiplied-black source video picture S with sample values s from some original input picture T with sample values t as expressed by Formulae (F-104) to (F-106), so that when the decoded picture F is a close approximation of the pre-multiplied-black source video picture S, the cascaded effect of Formulae (F-104) to (F-106) followed by Formulae (F-101) to (F-103) is approximately the same as expressed in Formulae (F-98) to (F-100).

sY = ( alphaFwt * tY ) / alphaRange		(F-104)

sCb = ( alphaFwt * tCb ) / alphaRange		(F-105)

sCr = ( alphaFwt * tCr ) / alphaRange		(F-106)

NOTE 3	In the event that the background picture B is represented using green, blue and red component values (see 7.3) in a manner such that the colour black is represented by all three component values being equal to 0, when the background picture B is black, the operation expressed by Formulae (F-101) to (F-103) becomes simply dY = fY, dCb = fCb, and dCr = fCr. This can help to explain the “pre-multiplied black” term, as the expressions in Formulae (F-104) to (F-106) are referred to as the pre-multiplication for the black background combination.

For the case with alpha_channel_use_idc equal to 1, somewhat modified processing should be applied when the colour representation domain is different from the use of green, blue, and red colour component values, or with the use of a non-zero black level. Unless the colour black is represented by all three component values bY, bCb, and bCr being equal to 0, Formulae (F-101) to (F-103) do not simplify to dY = fY, dCb = fCb, and dCr = fCr for pre-multiplied-black video content.

### Change #4 - HEVC: Allowing depth representation information SEI message in a primary layer in HEVC

Summary of change #4:

- Defining a depth picture as either an auxiliary picture of type AUX_DEPTH or a non-auxiliary picture representing a depth map.

- The associated primary picture of a depth picture can be defined by external means.

- G.14.3.4 Depth representation information SEI message semantics

- G.14.3.4.1 General

The syntax elements in the depth representation information SEI message specify various parameters for auxiliary pictures of type AUX_DEPTH or non-auxiliary pictures representing depth maps. Those parameters are signalled for the purpose of processing decoded primary and auxiliary pictures prior to rendering on a 3D display, such as view synthesis. Specifically, depth or disparity ranges for depth pictures are specified.

When present, the depth representation information SEI message shall be associated with either:

- one or more layers with AuxId value equal to AUX_DEPTH; or,

- one layer with nuh_layer_id equal to 0 with pictures representing depth maps

[Editor’s note: The depth SEI message does not define the persistence of decoded depth sample values as done for the alpha SEI message. Study if similar logic is needed for the depth SEI message.]

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the depth representation information SEI message applies.

When present, the depth representation information SEI message may be included in any access unit. It is recommended that, when present, the SEI message is included for the purpose of random access in an access unit in which the coded picture with nuh_layer_id equal to targetLayerId is an IRAP picture.

For an auxiliary picture with AuxId[ targetLayerId ] equal to AUX_DEPTH, an associated primary picture, if any, is a picture in the same access unit having AuxId[ nuhLayerIdB ] equal to 0 such that ScalabilityId[ LayerIdxInVps[ targetLayerId ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ nuhLayerIdB ] ][ j ] for all values of j in the range of 0 to 2, inclusive, and 4 to 15, inclusive.

For a picture with nuh_layer_id equal to 0 and representing a depth map, its associated primary picture, if any, is defined by external means.

For the remainder of this clause, a depth picture refers to either an auxiliary picture of type AUX_DEPTH or a non-auxiliary picture representing a depth map.

The information indicated in the SEI message applies to all the pictures with nuh_layer_id equal to targetLayerId from the access unit containing the SEI message up to but excluding the next picture, in decoding order, associated with a depth representation information SEI message applicable to targetLayerId or to the end of the CLVS of the nuh_layer_id equal to targetLayerId, whichever is earlier in decoding order.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of decoded luma samples of auxiliary depth pictures as specified in Table G.3. In Table G.3, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

The variable maxVal is set equal to ( 1 << ( 8 + bit_depth_luma_minus8 ) ) − 1, where bit_depth_luma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId.

Table G.3 — Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of a auxiliary depth picture represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each decoded luma sample value of a auxiliary depth picture represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| 2 | Each decoded luma sample value of a auxiliary depth picture represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents ZNear (specified below). |
| 3 | Each decoded luma sample value of a auxiliary depth picture represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[ i ]. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

disparity_ref_view_id specifies the ViewId value against which the disparity values are derived.

NOTE 1	disparity_ref_view_id is present only if d_min_flag is equal to 1 or d_max_flag is equal to 1 and is useful for depth_representation_type values equal to 1 and 3. The value of disparity_ref_view_id can correspond to a ViewId value which is not present in the current CVS.

The variables in the x column of Table G.4 are derived from the respective variables in the s, e, n, and v columns of Table G.4 as follows:

—	If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s * 2e − 31 * ( 1 + n ÷ 2v ).

—	Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 1	The above specification is similar to that found in IEC 60559:1989.

Table G.4 — Association between depth parameter variables and syntax elements

| x | S | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the coded picture with ViewId equal to ViewId of the auxiliary depth picture.

The units for the ZNear and ZFar values, when present, are identical but unspecified.

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piece-wise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piece-wise linear segments for mapping of decoded luma sample values of a auxiliary depth picture to a scale that is uniformly quantized in terms of disparity. The values of depth_nonlinear_representation_model[ 0 ] and depth_nonlinear_representation_model[ depth_nonlinear_representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 2	When depth_representation_type is equal to 3, a auxiliary depth picture contains nonlinearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform decoded depth sample values from the nonlinear representation to the linear representation, i.e. uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first ( 0, 0 ) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k <= depth_nonlinear_representation_num_minus1 + 1; k++ ) {
          pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )
          dev1 = depth_nonlinear_representation_model[ k ]
          pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )
          dev2 = depth_nonlinear_representation_model[ k + 1 ]	(G-10)

          x1 = pos1 − dev1
          y1 = pos1 + dev1
          x2 = pos2 − dev2
          y2 = pos2 + dev2

          for( x = Max( x1, 0 ); x <= Min( x2, maxVal ); x++ )
                           DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )
}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of a auxiliary depth picture in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

# Conclusion

We recommend adopting the proposed text changes in the corresponding draft specifications.

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JVET-AG0144, “Carriage of depth and alpha maps as HEVC single-layer bitstreams”, E. Thomas, E. Potetsianakis, E. Alexiou, R. Ghaznavi-Youvalari, M. Abdoli, M-L. Champel (Xiaomi)

- JVET-AG0308, “CICP extensions for depth and alpha signaling”, D. Podborski, A. Tourapis (Apple)