| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0155-v2 |
| --- | --- |

| Title: | AHG12: Restricting BT CUs to apply QT-like partitioning structure |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | W. Ahmad, P. Wennersten, K. Andersson | Tel: Email: | +46761046057 {waqas.ahmad, per.wennersten, kenneth.r.andersson} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes a restriction on BT CUs to avoid QT-like partitioning structure when QT split is allowed for such CUs that is claimed to reduce encoding runtime for ECM. One additional CABAC context is added to make this restriction also improve coding efficiency. The proposal is implemented on top of ECM 12.0 software and compared to ECM-12.0, the average BDR performance, and runtime reported in this contribution are as follows:

Y 0.00%, U 0.04%,	V 0.04%, EncTime 99.5%, DecTime 99.9% for the AI configuration,

Y -0.02%, U -0.02%, V -0.05%, EncTime 99.1%, DecTime 99.9% for the RA configuration,

Y -0.01%, U 0.02%, V 0.01%, EncTime 98.7%, DecTime 99.9% for the LDB configuration,

It is proposed to study the contribution in next round of EE.

# Introduction

ECM supports QT with nested MTT partitioning structures. The MTT structure contains BTH, BTV, TTH, and TTV splitting. The QT and MTT splitting structures in ECM provide support for rectangular CUs as well as square CUs, which allows a better fit to local picture content characteristics and thus improves coding gain. However, with the variety of partitioning structures in ECM, on the one hand, coding gain is improved, but on the other hand, it increases the encoding time. This contribution proposes to add a criterion to restrict BT CUs for applying a QT-like structure when a QT split is allowed for such CUs.

# Proposed method

In the current version of ECM (ECM-12.0), the BT CUs are allowed to apply a QT-like partitioning structure even though the QT split is allowed for such CUs. This redundant partitioning structure exists in two ways, firstly when a square CU is partitioned by BTH split and its each subblock is further partitioned by BTV splitting as shown in Figure 1(a). Secondly, when a square CU is partitioned by BTV split and its each subblock is further partitioned by BTH splitting as shown in Figure 1(b). With these two partitioning schemes, a square CU of size m×m is partitioned into four square subblocks of size m/2×m/2.

CU with size m×m

BTV

BTH

BTV

a) BTH split is applied on a square block followed by BTV splits on its both subblocks

CU with size m×m

BTH

BTH

BTV

b) BTV split is applied on a square block followed by BTH splits on its both subblocks

Figure 1 The proposed restriction on BT splitting to disallow QT like structure.

A restriction is proposed on BT CUs to forbid applying QT-like structure as shown in figure 1. When the binary split is applied on a luma channel CU, and the first subblock selects a different binary split compared to the binary split selected by its parent then the same BT split as chosen by the first subblock is disallowed for the second subblock.

In particular, when the BTH split is applied on a CU and its first subblock selects the BTV split then for the second subblock BTV split is disallowed. Similarly, when the BTV split is applied on a CU and its first subblock selects the BTH split then for the second subblock BTH split is disallowed. It should be noted that the proposed condition is only applied to a square CU for which QT split is allowed.

A new cabac context is added to signal split_cu_flag. The context is used when the binary split is applied on a luma channel CU, and the first subblock selects a different binary split compared to the binary split selected by its parent CU.

The proposed condition is applied based on the maximum CTU size. When the maximum CTU size is 256 the proposed condition is applied to 128x128 and 64x64 CUs. The proposed condition is applied to CU sizes 128x128, 64x64, and 32x32 when the maximum CTU size is 128.

# Results

The proposed scheme is implemented on top of the ECM-12.0 [1] and experiments are performed by following the common test conditions for ECM12.0 [2]. The encoding run time is estimated by running the proposed and anchor scheme on the same machine in parallel. An encoding time reduction is obtained for AI, RA and LDB configuration with improvements in coding efficiency for RA and LDB configurations.

Due to a technical problem with our cluster, QP22 simulations for three LDB sequences (BQTerrace, FlyingGraphic, and ArenaOfValor) and one RA sequence (ParkRunning3) are not completed. The ECM-12.0 anchor results are substituted for the missing sequences test results.

Table 1 Coding performances of the proposed modifications in ECM-12.0 for AI configuration.

|  | All Intra Main 10 Over ECM-12.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.02% | 0.04% | 100.1% | 99.9% | 99.9% | 100.2% |
| Class A2 | 0.00% | -0.02% | -0.01% | 100.0% | 99.9% | 100.0% | 100.0% |
| Class B | 0.00% | 0.07% | 0.11% | 99.2% | 100.1% | 100.0% | 100.0% |
| Class C | 0.01% | 0.10% | -0.03% | 99.6% | 99.9% | 99.6% | 100.0% |
| Class E | 0.00% | -0.03% | 0.05% | 98.9% | 99.6% | 100.1% | 99.9% |
| Overall | 0.00% | 0.04% | 0.04% | 99.5% | 99.9% | 99.9% | 100.0% |
| Class D | 0.00% | 0.16% | 0.02% | 99.5% | 100.1% | 99.8% | 100.0% |
| Class F | 0.03% | -0.14% | -0.01% | 99.7% | 100.0% | 99.1% | 100.0% |
| Class TGM | -0.02% | 0.00% | -0.10% | 99.6% | 100.1% | 99.1% | 100.1% |

Table 2 Coding performances of the proposed modifications in ECM-12.0 for RA configuration.

|  | Random Access Main 10 Over ECM-12.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.03% | -0.04% | 99.1% | 100.1% | 99.7% | 99.9% |
| Class A2* | -0.03% | -0.02% | 0.04% | 99.2% | 99.9% | 99.5% | 100.1% |
| Class B | -0.05% | -0.02% | -0.23% | 99.2% | 99.9% | 100.0% | 100.1% |
| Class C | 0.03% | 0.01% | 0.11% | 98.7% | 99.9% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.05% | 99.1% | 99.9% | 99.9% | 100.0% |
| Class D | -0.02% | -0.14% | 0.16% | 99.3% | 99.9% | 99.9% | 100.0% |
| Class F | -0.08% | -0.18% | -0.03% | 99.3% | 100.1% | 100.1% | 100.1% |
| Class TGM | -0.08% | -0.07% | -0.05% | 99.2% | 99.8% | 100.2% | 100.0% |

Table 3 Coding performances of the proposed modifications in ECM-12.0 for LDB configuration.

|  | Low delay B Main 10 Over ECM-12.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class B* | -0.04% | -0.06% | -0.14% | 98.6% | 99.8% | 99.8% | 99.0% |
| Class C | 0.02% | -0.04% | -0.16% | 98.9% | 99.8% | 99.6% | 100.1% |
| Class E | 0.01% | 0.24% | 0.48% | 98.6% | 100.0% | 99.4% | 100.2% |
| Overall | -0.01% | 0.02% | 0.01% | 98.7% | 99.8% | 99.6% | 99.7% |
| Class D | -0.22% | 0.31% | -0.35% | 99.2% | 99.8% | 99.8% | 100.0% |
| Class F* | 0.05% | -0.26% | -0.69% | 99.5% | 99.9% | 99.7% | 100.1% |
| Class TGM* | -0.24% | -0.33% | -0.14% | 99.3% | 99.9% | 100.2% | 100.3% |

# Conclusion

In this contribution, a condition is proposed on BT CUs to restrict applying a QT-like structure when a QT split is allowed for such CUs. A new CABAC context is added to signal split_cu_flag syntax when the proposed restriction is used. The proposed scheme is demonstrated on top of ECM-12.0 by following the CTC. The results show around 1% and 1.3% reduction in encoding time for RA and LDB configuration with 0.02% and 0.01% improvements in coding efficiency respectively.  It is suggested to include the proposed scheme in the next round of EE.

# References

- ECM-12.0 reference software: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

- M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Hannover, October 2023.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).