| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0156-v1 |
| --- | --- |

| Title: | AhG12: neural network-based intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Urban, T. Dumas, F. Galpin, E. François | Tel: Email: | thierry.dumas@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes a new intra prediction mode based on neural networks in ECM-12.0. It is built from the algorithm integrated into Neural Network Video Coding (NNVC) and described in JVET-AG2019. The following results are reported.

- RA:  -…%, -…% and -…% BD-rate PSNR Y, U and V gains, respectively for a complexity of …% and …% at encoder and decoder

- AI:   -…%, -…% and -…% BD-rate PSNR Y, U and V gains, respectively, for a complexity of …% and …% at encoder and decoder.

# Introduction

A single neural network-based intra prediction mode exists in NNVC. Both its inference inside NNVC and its entire training have already been cross-checked through two different EE1 rounds. The same algorithm has been implemented and evaluated on top of ECM-12.0.

# Description

The neural network-based intra prediction algorithm and models are the same as the ones described in JVET-AG2019 [1].

- SADL for inference

- 16-bit integer for model weights and intermediate results, using 32-bit integer for internal computation

- 7 models depending on the block size

- sparse weights enforced at training time for complexity reduction. SADL supports a simple sparse matrix storage (compressed row method) and sparse matrix multiplication algorithm, exploiting the matrices sparsity.

Table 1 shows the parameters count and storage memory required for each model. Memory usage reported is informative since it depends on the implementation for sparse matrix storage. Table 1 shows the total memory storage of the actual tested ECM implementation.

Table 1: parameters count and storage memory needed for each model.

| model | parameter’s count | non-zero parameters count | memory footprint |
| --- | --- | --- | --- |
| 4x4 4x8 4x16 4x32 8x8 8x16 16x16 | 1679k 1737k 1854k 2088k 2030k 2263k 4054k | 99k 111k 134k 181k 143k 190k 350k | 387 kB 433 kB 524 kB 706 kB 558 kB 741 kB 1369 kB |

The complexity measured in MACs/pixel for the different neural network models is reported in Table 2. The bottom part of the table shows the complexity for blocks predicted with down-sampled reference samples.

Table 2: complexity measured in MACs/pixel for the different neural network models.

| model | complexity (MACs/pixel) |
| --- | --- |
| 4x4 4x8 4x16 4x32 8x8 8x16 16x16 | 6.2k 3.5k 2.1k 1.4k 2.2k 1.5k 1.4k |
| 8x32 | 0.74k |
| 16x32 | 0.68k |
| 32x32 | 0.34k |
| 64x64 | 0.08k |
| 128x128 | 0.02k |

# Experiment results

The simulations of ECM-12.0 including the proposed intra prediction mode are run following the JVET CTC [2] and reported in Table 3.

Table 3: experimental results of ECM-12.0 including the proposed intra prediction mode over ECM-12.0.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.36% | -0.44% | -0.43% | 100.6% | 106.4% | 102.8% | 106.5% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.37% | -0.58% | -0.52% | 96.2% | 103.0% | 103.9% | 107.6% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.78% | -0.57% | -0.62% | 110.0% | 144.1% | 100.1% | 105.7% |
| Class C | -0.89% | -0.72% | -0.78% | 107.9% | 151.8% | 103.0% | 107.5% |
| Class E | -1.17% | -0.92% | -1.04% | 111.9% | 147.1% | 103.1% | 107.1% |
| Overall |  |  |  |  |  |  |  |
| Class D | -1.02% | -0.69% | -0.93% | 106.6% | 160.9% | 104.2% | 107.9% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Conclusion

This contribution presents a new intra prediction mode based on neural networks in ECM-12.0. It is reported that, on top of ECM-12.0, this intra prediction tool yields the following results.

- RA:  -…%, -…% and -…% BD-rate PSNR Y, U and V gains, respectively for a complexity of …% and …% at encoder and decoder

- AI:   -…%, -…% and -…% BD-rate PSNR Y, U and V gains, respectively, for a complexity of …% and …% at encoder and decoder.

It is suggested to study this intra prediction tool in the next round of EE2.

# Cited References

- JVET-AG2019, “Description of algorithms and software in neural network-based video coding (NNVC) version 6”, F. Galpin, Y. Li, D. Rasanovskyy, J. Ström, L. Wang. 33rd Meeting, by teleconference, 17–26 January 2024.

- JVET-AF2017, “Common test conditions and evaluation procedures for enhanced compression tool testing”, M. Karczewicz, Y. Ye, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).