| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0158-v1 |
| --- | --- |

| Title: | AHG 9: On association between SEI Processing Order and Processing Order Nesting SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung Hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the current design, SEI Processing Order (SPO) SEI message and Processing Order Nesting (PON) SEI message are tightly associated. This is asserted to be not only does not give any benefit, it also may cause confusion and problems when either SPO SEI message or PON SEI message is removed from the bitstream.

This contribution proposes to modify the presence of SPO SEI message and PON SEI message to be more independent. To do that, the following points are proposed:

- Remove the signalling that tightly associates SPO and PON SEI messages. The following syntax elements are removed from PON SEI message:

  - pon_num_po_ids_minus1

  - pon_target_po_id[ i ]

  - pon_processing_order[ i ]

- Add text in SPO SEI message semantics to describe that an SEI message with po_sei_wrapping_flag[ i ] equal to 1 may or may not present in the bitstream. When it is present, it is included in one or more PON SEI messages.

- Add text in PON SEI message semantics to describe that an SEI message contained in PON SEI message may or may not be associated with an SPO SEI message. When there is no associated SPO SEI message, the SEI message is treated as if it does not have known processing order.

# Introduction

In the current design, SEI Processing Order (SPO) SEI message and Processing Order Nesting (PON) SEI message are tightly associated. When an SEI message seiA (i.e., by including the payloadType and possibly some prefix bits of seiA) is included in an SPO SEI message with its associated po_sei_wrapping_flag[ i ] equal to 1, the semantics suggests that one or more PON SEI message that contains that seiA is present in the bitstream. Snippet from JVET-AG2027 below shows the part of the text that describe this association in SPO SEI message.

--------------------- Start of snippet ---------------------

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

po_sei_wrapping_flag[ i ] equal to 1 specifies that one or more processing order nesting SEI messages with both of the following constraints should be present:

–	pon_target_po_id[ j ] with any value of j is equal to po_id.

–	There is a k-th loop entry in the processing order nesting SEI message such that the payloadType of the k-th nested SEI message is equal to po_sei_payload_type[ i ] and pon_processing_order[ k ] is equal to po_sei_processing_order[ i ].

When po_sei_wrapping_flag[ i ] is equal to 0, an SEI message with payloadType equal to po_sei_payload_type[ i ] (and, when po_sei_prefix_flag[ i ] equal to 1, prefix data that matches the values of po_sei_prefix_data_bit[ i ][ j ]) should be present outside of the processing order nesting SEI message.

NOTE 2 – po_sei_wrapping_flag[ i ] equal to 1 enables SEI messages to be carried within the processing order nesting SEI message to prevent such SEI messages from being incorrectly interpreted by decoders that do not process the SPO SEI message. Thus, po_sei_wrapping_flag[ i ] equal to 1 is intended to be used when po_sei_wrapping_flag[ i ] equal to 0 can lead to unintended results being produced by such decoders.

--------------------- End of snippet ---------------------

The reverse connection is also present in the semantics of PON SEI message such that the po_id of SPO SEI message that contains an SEI message that is also a PON-nested SEI message in a PON SEI message is listed in the list of target po_id of the PON SEI message. Snippet from JVET-AG2027 below shows the part of the text that describe this association in PON SEI message.

--------------------- Start of snippet ---------------------

| processing_order_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| pon_num_po_ids_minus1 | u(8) |
| for( i = 0; i <= pon_num_po_ids_minus1; i++ ) |  |
| pon_target_po_id[ i ] | u(8) |
| pon_num_seis_minus1 | u(8) |
| for( i = 0; i <= pon_num_seis_minus1; i++ ) { |  |
| pon_processing_order[ i ] | u(8) |
| sei_message( ) |  |
| } |  |
| } |  |

…

For each associated SEI processing order SEI message there shall be at least one value of i in the range of 0 to pon_num_seis_minus1, inclusive, in the PON SEI message for which the associated SEI processing order SEI message has some entry k for which all of the following are true:

–	po_sei_processing_order[ k ] is equal to pon_processing_order[ i ]

–	po_sei_payload_type[ k ] is equal to the payloadType value of the i-th PON-nested SEI message.

–	When po_sei_prefix_flag[ k ] is equal to 1, po_sei_prefix_data_bit[ k ][ j ] for j in the range of 0 to po_num_bits_in_prefix_indication_minus1[ k ], inclusive, contain the same content as the po_num_bits_in_prefix_indication_minus1[ k ] plus 1 initial bits of the SEI message payload of the i-th PON-nested SEI message.

The i-th PON-nested SEI message should be applied as the k-th loop entry of the associated SEI processing order SEI message.

--------------------- End of snippet ---------------------

# Problem description

In our assertion, the tight association between SPO and PON SEI messages not only does not give any benefit, it also may cause confusion and problems. Such tight association does not give any benefit since by design it is already clear that SPO SEI message will be present earlier than PON SEI message in the bitstream, thus, there is no need to signal target po_id because if the SEI messages contained in the PON SEI message are included in SPO SEI message, that SPO SEI message would have been seen and parsed anyway.

The tight association between SPO and PON SEI messages may cause confusion because it is not clear what happens if one of these SPO or PON SEI messages is removed from the bitstream. Ideally, if a PON SEI message is removed from the bitstream, any SEI messages included in SPO message with its associated po_sei_wrapping_flag[ i ] equal to 1 should simply be treated as if those SEI messages are not present in the bitstream and decoder should be just fine. However, in the current draft text, it may not be the case as the semantics states that PON that contains those wrapped SEI messages is present. Likewise, if an associated SPO SEI message of a PON is removed from the bitstream, ideally, it should not be an issue as it can be considered such that the PON-nested SEI messages have no associated processing order. However, in the current draft text, the presence of the target SPO SEI message is needed.

In our assertion, the design of SPO SEI message and PON SEI message should allow removal of the SEI messages from the bitstream independently without causing any problem but the unnecessary tight association between the two prevents such independence.

# Proposal

To overcome the problems described above, we propose the following modifications:

- Remove the signalling that tightly associates SPO and PON SEI messages. The following syntax elements are removed from PON SEI message:

  - pon_num_po_ids_minus1

  - pon_target_po_id[ i ]

  - pon_processing_order[ i ]

- Add text in SPO SEI message semantics to describe that an SEI message with po_sei_wrapping_flag[ i ] equal to 1 may or may not present in the bitstream. When it is present, it is included in one or more PON SEI messages.

- Add text in PON SEI message semantics to describe that an SEI message contained in PON SEI message may or may not be associated with an SPO SEI message. When there is no associated SPO SEI message, the SEI message is treated as if it does not have known processing order.

# Text implementation

The text implementation of the above proposed points are as shown below:

Modification to SPO SEI message:

…

po_sei_wrapping_flag[ i ] equal to 1 specifies that when the i-th SEI message in this SPO SEI message is present in the current CVS, it shall be present in one or more processing order nesting SEI messages in the CVS.with both of the following constraints should be present:

–	pon_target_po_id[ j ] with any value of j is equal to po_id.

–	There is a k-th loop entry in the processing order nesting SEI message such that the payloadType of the k-th nested SEI message is equal to po_sei_payload_type[ i ] and pon_processing_order[ k ] is equal to po_sei_processing_order[ i ].

When po_sei_wrapping_flag[ i ] is equal to 0, an SEI message with payloadType equal to po_sei_payload_type[ i ] (and, when po_sei_prefix_flag[ i ] equal to 1, prefix data that matches the values of po_sei_prefix_data_bit[ i ][ j ]), if present, shall should be present outside of the processing order nesting SEI message.

NOTE 2 – po_sei_wrapping_flag[ i ] equal to 1 enables SEI messages to be carried within the processing order nesting SEI message to prevent such SEI messages from being incorrectly interpreted by decoders that do not process the SPO SEI message. Thus, po_sei_wrapping_flag[ i ] equal to 1 is intended to be used when po_sei_wrapping_flag[ i ] equal to 0 can lead to unintended results being produced by such decoders.

…

Modification to PON SEI message:

| processing_order_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| pon_num_po_ids_minus1 | u(8) |
| for( i = 0; i <= pon_num_po_ids_minus1; i++ ) |  |
| pon_target_po_id[ i ] | u(8) |
| pon_num_seis_minus1 | u(8) |
| for( i = 0; i <= pon_num_seis_minus1; i++ ) { |  |
| pon_processing_order[ i ] | u(8) |
| sei_message( ) |  |
| } |  |
| } |  |

…

The processing order nesting (PON) SEI message includes one or more SEI messages that may be applied only as parts of the processing chain identified by an associated SEI processing order SEI message and should not be applied in a manner that would contradict with the processing chain identified by the associated SEI processing order SEI message. [Move this to be after the following paragraph]

The SEI messages contained in a PON SEI message are referred to as PON-nested SEI messages.

NOTE – An encoder can include multiple PON SEI messages in the same access unit. For example, a first PON SEI message in an access unit can contain a PON-nested SEI message that applies to multiple processing chains and one or more other PON SEI messages in the same access unit that apply to a single processing chain only.

The processing order nesting (PON) SEI message includes one or more SEI messages that may should be applied only as parts of the processing chain identified by an associated SEI processing order SEI message and should not be applied in a manner that would contradict with the processing chain identified by the associated SEI processing order SEI message. When a PON-nested SEI message is not part of any processing chain (i.e., the PON-nested SEI message is not included in any SEI processing order SEI message), it is handled as if does not have associated processing order.

…

pon_num_po_ids_minus1 plus 1 specifies the number of the SEI processing order SEI messages SEI associated with this PON SEI message.

pon_target_po_id[ i ] indicates the po_id of the i-th associated SEI processing order SEI message.

pon_num_seis_minus1 plus 1 specifies the number of the PON-nested SEI messages that are included in this PON SEI message.

pon_processing_order[ i ] specifies the position of the i-th processing-order-nested SEI message within the processing order defined by the associated SEI processing order SEI message. When i is greater than 0, pon_processing_order[ i ] shall be greater than or equal to pon_processing_order[ i − 1 ].

For each associated SEI processing order SEI message there shall be at least one value of i in the range of 0 to pon_num_seis_minus1, inclusive, in the PON SEI message for which the associated SEI processing order SEI message has some entry k for which all of the following are true:

–	po_sei_processing_order[ k ] is equal to pon_processing_order[ i ]

–	po_sei_payload_type[ k ] is equal to the payloadType value of the i-th PON-nested SEI message.

–	When po_sei_prefix_flag[ k ] is equal to 1, po_sei_prefix_data_bit[ k ][ j ] for j in the range of 0 to po_num_bits_in_prefix_indication_minus1[ k ], inclusive, contain the same content as the po_num_bits_in_prefix_indication_minus1[ k ] plus 1 initial bits of the SEI message payload of the i-th PON-nested SEI message.

The i-th PON-nested SEI message should be applied as the k-th loop entry of the associated SEI processing order SEI message.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).