| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0161-v1 |
| --- | --- |

| Title: | AHG9: Region packing information SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela Honglei Zhang | Email: | {jill.boyce, miska.hannuksela, honglei.1.zhang} @nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

A region packing information SEI message is proposed for VSEI for the use case of packing rectangular regions of interest (ROIs) from a larger picture into a smaller resolution coded picture.  Use of a smaller coded picture size reduces bitrate and pixel rate and reduces complexity of video decoder and any post-processing operations. Optional reconstruction of a target picture from the specified regions of the decoded picture is enabled.

# Overview of the proposal

For some applications, specific regions of interest (ROIs) within a video are of greater interest than the rest of the video. This contribution enables an encoding system to pack selected regions of interest within a coded picture. Pixel rate complexity and bitrate can be reduced by coding smaller pictures containing only the specific regions of interest. The proposed SEI message enables interoperability for these applications by signalling the locations and sizes of the ROIs within the coded picture. To enable reconstruction of a target picture, optional signalling of corresponding region locations within a target picture is proposed.

Figure 1 shows an example with an original picture ROIs indicated. Figure 2(a) shows the coded picture for that example, and Figure 2(b) shows the reconstructed target picture.

Figure 1 – Original picture with selected regions shown

Figure 2(a) – Coded picture using proposed SEI message

Figure 2(b) – Reconstructed target picture

For certain applications, some regions of a video picture may require retaining full resolution while other regions may be downsampled. Selective resampling of regions is supported in the proposal. Figure 3(a) shows an example coded picture where one of two regions is downsampled. Figure 3(b) shows the reconstructed target picture, which is very similar to Figure 2(b), but with the second region having a lower effective resolution.

Figure 3(a) – Coded picture with region resampling

Figure 3(b) – Reconstructed target picture with region resampling

Background regions may optionally be included within the coded picture, with the region ID used to determine precedence between regions in a reconstructed target picture. Figure 4(a) shows an example coded picture with background resampling, and Figure 4(b) shows the reconstructed target picture. The reconstructed target picture in 4(b) appears similar to the reconstructed target picture in 2(b), but the background has a lower effective resolution. An encoding system may choose to reduce bitrate by filtering or replacing the areas of the downsampled background corresponding to selected regions also signalled in the coded picture, but this was not done in this example.

Figure 4(a) – Coded picture with background resampling

Figure 4(b) – Reconstructed target picture with background resampling

To reduce the bitrate required for signalling the region sizes and locations, a unit size is signalled, as a power of 2.

To enable efficient usage of the SEI message with reference picture resampling (RPR), a flag is used to indicate that the region sizes and locations signalled are with respect to the maximum picture size signalled in the SPS rather than the current picture size. This avoids the need to re-signal region sizes and locations for each picture when RPR is used.

Aspects of the proposed design are listed below.

- A region ID is optionally signalled for each region

- For each region, the top left (x,y) position and (width, height) in the coded picture are signalled in “units”, with unit size signalled as a power of 2

- A flag indicates whether the position and size of a region are with respect to the picture dimensions of the current cropped decoded picture signalled in the PPS, or with respect to the max cropped picture dimensions signalled in the SPS, e.g. when adaptive picture resolution is used

- The resolution of the target picture is optionally signalled, to enable optional reconstruction of the target picture

- The top left location of each region within the target picture is optionally signalled

- A list of resampling ratios as ratio of integers (numerator, denominator) is optionally signalled, with a selected resampling ratio signalled per region

- In the target picture reconstruction process, sample values are initialized to mid-gray

- Regions in the target picture may overlap. The region with the higher region ID has precedence in the reconstruction of the target picture.

- Persistence flag and cancel flags are signalled to specify persistence

# Proposed syntax & semantics

## VSEI

| packed_regions_info( payloadSize) | Descriptor |
| --- | --- |
| pri_cancel_flag | u(1) |
| if ( !pri_cancel_flag ) { |  |
| pri_persistence_flag | u(1) |
| pri_num_regions_minus1 | ue(v) |
| pri_use_max_dimensions_flag | u(1) |
| pri_log2_unit_size | u(4) |
| pri_region_size_len_minus1 | u(4) |
| pri_region_id_present_flag | u(1) |
| pri_target_pic_params_present_flag | u(1) |
| if( pri_target_pic_params_present_flag ) { |  |
| pri_target_pic_width_minus1 | u(16) |
| pri_target_pic_height_minus1 | u(16) |
| } |  |
| pri_num_resampling_ratios_minus1 | ue(v) |
| for( i = 1; i <= pri_num_resampling_ratios_minus1; i++ ) { |  |
| pri_resampling_width_num_minus1[ i ] | ue(v) |
| pri_resampling_width_denom_minus1[ i ] | ue(v) |
| pri_fixed_aspect_ratio_flag[ i ] | u(1) |
| if( !pri_fixed_aspect_ratio_flag[ i ] ) { |  |
| pri_resampling_height_num_minus1[ i ] | ue(v) |
| pri_resampling_height_denom_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| for( i = 0; i <= pri_num_regions_minus1; i++ ) { |  |
| if( pri_region_id_present_flag ) |  |
| pri_region_id[ i ] | ue(v) |
| pri_region_top_left_in_units_x[ i ] | u(v) |
| pri_region_top_left_in_units_y[ i ] | u(v) |
| pri_region_width_in_units_minus1[ i ] | u(v) |
| pri_region_height_in_units_minus1[ i ] | u(v) |
| if( pri_num_resampling_ratios_minus1 > 0 ) |  |
| pri_resampling_ratio_idx[ i ] | u(v) |
| if( pri_target_pic_params_present_flag ) |  |
| pri_target_region_top_left_x[ i ] | u(v) |
| pri_target_region_top_left_y[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

The packed regions info SEI message provides information regarding rectangular regions packed with the coded picture. This information may optionally be used to reconstruct a target picture from the samples of the cropped decoded picture corresponding to the regions described in this SEI message.

Use of this SEI message requires the definition of the following variables:

–	A picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples and PicHeightInLumaSamples, respectively.

–	A maximum picture width and maximum picture height in units of luma samples, denoted herein by MaxPicWidth and MaxPicHeight, respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3.

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY, and when ChromaFormatIdc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein by BitDepthC.

pri_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous packed regions information SEI message in output order that applies to the current layer. pri_cancel_flag equal to 0 indicates that packed regions information follows.

pri_persistence_flag specifies the persistence of the packed regions information SEI message for the current layer.

pri_persistence_flag equal to 0 specifies that the packed regions information applies to the current decoded picture only.

pri_persistence_flag equal to 1 specifies that the packed regions information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a packed regions information SEI message is output that follows the current picture in output order.

pri_num_regions_minus1 plus 1 specifies the number of regions for which information is signalled.

pri_use_max_dimensions_flag equal to 1 specifies that MaxPicWidth, MaxPicHeight, PicWidthInLumaSamples and PicHeightInLumaSamples will be used in variable calculations. pri_use_max_dimensions_flag equal to 0 specifies that MaxPicWidth, MaxPicHeight, PicWidthInLumaSamples and PicHeightInLumaSamples will not be used in variable calculations for the region parameters.

pri_log2_unit_size specifies a unit size used in variable calculations for the region parameters.

The variable priUnitSize is set equal to 1 << pri_log2_unit_size.

pri_region_size_len_minus1 plus 1 specifies the number of bits used to signal pri_region_top_left_in_units_x[ i ], pri_region_top_left_in_units_y[ i ], pri_region_width_in_units_minus1[ i ], pri_region_height_in_units_minus1[ i ],  pri_target_region_top_left_x[ i ], and pri_target_region_top_left_y[ i ].

pri_region_id_present_flag equal to 1 indicates the pri_region_id[ i ] syntax element is present. pri_region_id_present_flag equal to 0 indicates the pri_region_id[ i ] syntax element is not present.

pri_target_pic_params_present_flag equal to 1 indicates the pri_target_region_top_left_x[ i ], pri_target_region_top_left_y[ i ], pri_target_pic_width_minus1, and pri_target_pic_height_minus1 syntax elements are present. pri_target_pic_params_present_flag equal to 0 indicates the pri_target_region_top_left_x[ i ], pri_target_region_top_left_y[ i ], pri_target_pic_width_minus1, and pri_target_pic_height_minus1 syntax elements are not present.

pri_target_pic_width_minus1 plus 1 and pri_target_pic_height_minus1 plus 1, when present, indicate the width and height, respectively, in luma samples of the target picture that may be reconstructed from the samples of the cropped decoded picture corresponding to the regions described in this SEI message.

pri_num_resampling_ratios_minus1 specifies the number of resampling ratios that are signalled.

pri_resampling_width_num_minus1[ i ] plus 1 and pri_resampling_width_denom_minus1[ i ] plus 1 specify the numerator and denominator, respectively, for the width resampling of the i-th resampling ratio. Both pri_resampling_width_num_minus1[ i ] and pri_resampling_width_denom_minus1[ i ] shall be in the range of 0 to 65 535, inclusive.

The values of pri_resampling_ratio_width_num_minus1[ 0 ] and pri_resampling_ratio_width_denom_minus1[ 0 ] are inferred to be equal to 0.

pri_fixed_aspect_ratio_flag[ i ] equal to 1 specifies that the pri_resampling_height_num_minus1[ i ] and pri_resampling_height_denom_minus1[ i ] syntax elements are not present. pri_fixed_aspect_ratio_flag[ i ] equal to 0 specifies that the pri_resampling_height_num_minus1[ i ] and pri_resampling_height_denom_minus1[ i ] syntax elements are present.

pri_resampling_height_num_minus1[ i ] plus 1 and pri_resampling_height_denom_minus1[ i ] plus 1 specify the numerator and denominator, respectively, for the height resampling of the i-th resampling ratio. Both pri_resampling_height_num_minus1[ i ] and pri_resampling_height_denom_minus1[ i ] shall be in the range of 0 to 65 535, inclusive. When not present, the values of pri_resampling_height_num_minus1[ i ] and pri_resampling_height_denom_minus1[ i ] are inferred to be equal to the pri_resampling_width_num_minus1[ i ] and pri_resampling_width_denom_minus1[ i ], respectively.

pri_region_id[ i ] indicates the ID of the i-th region. When not present, the value of pri_region_id[ i ] is inferred to be equal to i.

pri_region_top_left_in_units_x[ i ] and pri_region_top_left_in_units_y[ i ] specify the horizontal and vertical positions, respectively, of the top left sample of the i-th region in units. The length of the syntax elements are pri_region_size_len_minus1 + 1.

The variables priRegionTopLeftX[ i ] and priRegionTopLeftY, representing the horizontal and vertical positions, respectively, in luma samples of the region in the cropped decoded picture, are derived as follows:

if( !pri_use_max_dimensions_flag ) {
	priRegionTopLeftX[ i ] =	 pri_region_top_left_in_units_x[ i ] * priUnitSize
	priRegionTopLeftY[ i ] =	 pri_region_top_left_in_units_y[ i ] * priUnitSize
} else {
	priRegionTopLeftX[ i ] =	( pri_region_top_left_in_units_x[ i ] * priUnitSize * 
			PicWidthInLumaSamples + MaxPicWidth /2 ) / MaxWidth
	priRegionTopLeftY[ i ] =	( pri_region_top_left_in_units_y[ i ] * priUnitSize* 
			PicHeightInLumaSamples + MaxPicHeight /2 ) / MaxHeight
}

pri_region_width_in_units_minus1[ i ] plus 1 and pri_region_height_in_units_minsu1[ i ] plus 1 specify the horizontal and vertical positions, respectively, of the width and height of the i-th region in units. The length of the syntax elements are pri_region_size_len_minus1 + 1.

The variables priRegionWidth[ i ] and priRegionHeight[ i ], representing the width and height, respectively, in luma samples of the i-th region in the cropped decoded picture are derived as follows:

if( !pri_use_max_dimensions_flag ) {
	priRegionWidth[ i ] =	 ( pri_region_width_in_units_minus1 [ i ] + 1) * priUnitSize 
	priRegionHeight[ i ] =	 ( pri_region_height_in_units_minus1 [ i ] + 1) * priUnitSize 
} else {
	priRegionWidth[ i ] =  ( ( pri_region_width_in_units_minus1 [ i ] + 1) * priUnitSize *
			PicWidthInLumaSamples + MaxPicWidth/2 ) / MaxWidth
	priRegionHeight[ i ] = ( ( pri_region_height_in_units_minus1 [ i ] + 1) * priUnitSize * 
			PicHeightInLumaSamples + MaxPicHeight/2 ) / MaxHeight
}

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

It is a requirement of bitstream conformance that priRegionWidth[ i ] % SubWidthC shall be equal to 0 and priRegionHeight[ i ] % SubHeightC shall be equal to 0.

pri_resampling_ratio_idx[ i ] specifies the index of the resampling ratio used for the i-th region. The length of the syntax element is Ceil( Log2( pri_num_resampling_ratios_minus1 + 1 ) ).

The variables priResampleWidthNum[ i ], priResampleWidthDenom[ i ], priResampleHeightNum[ i ]  and  priResampleHeightDenom[ i ]  are derived as follows.

priResampleWidthNum[ i ] = pri_resampling_width_num_minus1[ pri_rsampling_ratio_idx[ i ] ] + 1

priResampleWidthDenom[ i ] =

pri_resampling_width_denom_minus1[ pri_rsampling_ratio_idx[ i ] ]   + 1

priResampleHeightNum[ i ] =

pri_resampling_height_num_minus1[ pri_rsampling_ratio_idx[ i ] ] + 1

priResampleHeightDenom[ i ] =

pri_resampling_height_denom_minus1[ pri_rsampling_ratio_idx[ i ] ]   + 1

pri_target_region_top_left_x[ i ] and pri_target_region_top_left_y[ i ], when present, indicate the horizontal and vertical positions, respectively,  of the top left sample position in luma samples of the i-th region in the reconstructed target picture.

The variables priTargetRegionWidth and priTargetHeight, representing the width and height, respectively, in luma samples of the resampled region in the reconstructed target picture, are derived as follows:

priTargetRegionWidth = Round( ( priRegionWidth[ i ] * priResampleWidthNum[ i ] ) ÷

( priResampleWidthDenom[ i ]  * SubWidthC ) ) * SubWidthC

priTargetRegionHeight = Round( ( priRegionHeight[ i ] * priResampleHeightNum[ i ] ) ÷

( priResampleHeightDenom[ i ] * SubHeightC ) ) * SubHeightC

When reconstructing a target picture with luma sample array of size (pri_target_pic_width_minus1 + 1) × (pri_target_pic_height_minus1 + 1), all luma sample values are initialized to value 1  << ( BitDepthY – 1) and chroma samples, if present, to 1  << ( BitDepthC – 1).

If for any sample position (x,y) and regions j and k the following conditions are all met:

pri_region_id[ j ] > pri_region_id[ k ]
x is in (priRegionTopLeftX[ j ] .. priRegionTopLeftX[ j ] + priRegionWidth[ j ]) 
y is in (priRegionTopLeftY[ j ] .. priRegionTopLeftY[ j ] + priRegionHeight[ j ] 
x is in (priRegionTopLeftX[ k ] .. priRegionTopLeftX[ k ] + priRegionWidth[ k]) 
y is in (priRegionTopLeftY[ k ] .. priRegionTopLeftY[ k ] + priRegionHeight[ k ]

Then the reconstructed target picture sample at position (x, y) should be determined by the parameters signalled for the j-th region.

## VVC

D.12.13 Use of the packed regions information SEI message

For purposes of interpretation of the packed regions information SEI message, the following variables are specified:

–	PicWidthInLumaSamples is set equal to the value of pps_pic_width_in_luma_samples − ‌SubWidthC * ‌ ( pps_conf_win_left_offset + pps_conf_win_right_offset ).

–	PicHeightInLumaSamples is set equal to the value of pps_pic_height_in_luma_samples − ‌SubHeightC * ‌ ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ).

–	MaxPicWidth is set equal to the value of sps_pic_width_in_luma_samples − ‌SubWidthC * ‌ ( sps_conf_win_left_offset + sps_conf_win_right_offset ).

–	MaxPicHeight is set equal to the value of sps_pic_height_in_luma_samples − ‌SubHeightC * ‌ ( sps_conf_win_top_offset + sps_conf_win_bottom_offset ).

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).