| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0162-v1 |
| --- | --- |

| Title: | AHG9: Constituent rectangles SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela | Email: | {jill.boyce, miska.hannuksela} @nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

A constituent rectangles SEI message is proposed for VSEI to enable coded pictures formed from a composition of multiple constituent rectangles. The coded picture may contain constituent rectangles of different content types, such as texture, depth, alpha, or object mask, or may contain multiple constituent rectangles of the same type. Each constituent rectangle can optionally be identified with a rectangle ID and/or described by a text descriptor.

# Overview of the proposal

Some applications may benefit from having coded pictures that contain multiple constituent rectangles.  The rectangles may themselves be constituent pictures of different types, such as texture, depth, alpha, or object mask, or may contain multiple constituent pictures of the same type, such as multiple views.

Figure 1 shows an example coded picture containing both a (normal video) texture constituent picture and a depth constituent picture.

Figure 1. Example picture with texture and depth constituent rectangles

Figure 2 shows a multi-view example, with a coded picture containing 3 views.

Figure 2. Example picture with constituent rectangles for multi-view with 3 views

Although VVC can already support these use cases through the use of features such as multi-layer bitstreams and auxiliary pictures, some applications may prefer to use single layer bitstreams to simplify system timing and utilize existing VVC Main 10 profile encoders and decoders.

To reduce the signalling overhead, when subpictures are used, a flag may be used to indicate that the sub picture partitioning is used to identify the constituent rectangles, with the subpicture ID corresponding to the rectangle ID.

When all rectangles are the same size, the number of columns and rows can be signalled, to reduce overhead.

An empty rectangle type is defined and can be used with “same size” rectangle signalling to fill in unused area in the coded picture.

Figure 3. Example picture with constituent rectangles with 2x2 grid same size views and empty rectangle

Aspects of the proposed design are listed below.

- A rect type is optionally signalled per constituent rectangle

- Locations and sizes of the constituent rectangles can be signalled through the following options:

  - Subpic parameters signalled in the SPS may be used to identify the constituent rectangle locations and sizes

  - Optional signalling of number of columns and rows when all constituent rectangles are the same size, with derivation of constituent rectangle locations and sizes

  - Explicit signalling of constituent rectangle location and sizes, in any order

- Each sample location in the coded picture may be included in at most one constituent rectangle, e.g. rectangles in the coded picture do not overlap

- Samples in the coded picture are not required to be in a constituent rectangle

- An “empty” rect type can be signalled for a rectangular region, especially useful for vacant areas using “same size” signalling

- A rect ID is optionally signalled per constituent rectangle

- A rect description is optionally signalled per constituent rectangle

- SEI persists for an entire CLVS

- If adaptive resolution change is used, signalled constituent rectangle sizes and locations are for  max pic width/height signalled in SPS, and current picture sizes and locations are calculated

# Proposed syntax & semantics

## VSEI

| constituent_rectangles( payloadSize ) { | Descriptor |
| --- | --- |
| cr_num_rects_minus1 | u(12) |
| cr_rect_id_present_flag | u(1) |
| if ( cr_rect id_present_flag ) |  |
| cr_rect_id_len | u(4) |
| cr_rect_type_enabled_flag | u(1) |
| cr_rect_type_descriptions_enabled_flag | u(1) |
| cr_subpics_partitioning_flag | u(1) |
| if( !cr_subpics_partitioning_flag ) { | u(1) |
| cr_rect_same_size_flag | u(1) |
| if ( cr_rect_same_size_flag ) { |  |
| cr_num_cols_minus1 | ue(v) |
| cr_num_rows_minus1 | ue(v) |
| } else { |  |
| cr_log2_unit_size | u(4) |
| cr_rect_size_len_minus1 | u(4) |
| } |  |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) { |  |
| if ( cr_rect type_enabled_flag ) { |  |
| cr_rect_type_present_flag[ i ] | u(1) |
| if ( cr_rect_type_present_flag[ i ] ) |  |
| cr_rect_type_idc[ i ] | u(8) |
| } |  |
| if ( cr_rect type_idc[ i ] != 255 ) { |  |
| if ( cr_rect id_present_flag ) |  |
| cr_rect_id[ i ] | u(v) |
| if ( cr_rect type_description_enabled_flag ) |  |
| cr_rect_type_description_present_flag[ i ] | u(1) |
| } |  |
| if( !cr_subpics_partitioning_flag && !cr_rects_same_size_flag ) { |  |
| cr_rect_top_left_in_units_x[ i ] | u(v) |
| cr_rect_top_left_in_units_y[ i ] | u(v) |
| cr_rect_width_in_units_minus1[ i ] | u(v) |
| cr_rect_height_in_units minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( cr_rect_type_descriptions_enabled_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| cr_bit_equal_to_zero /* equal to 0 */ | f(1) |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) |  |
| if( rect_type_description_present_flag[ i ] ) |  |
| cr_rect_type_description[ i ] | st(v) |
| } |  |
| } |  |

The constituent rectangles SEI message enables composition of multiple rectangles within a coded picture and provides information about the rectangles, including ID, type, text description, location, and size.

Use of this SEI message requires the definition of the following variables:

–	A picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples and PicHeightInLumaSamples, respectively.

–	A maximum picture width and maximum picture height in units of luma samples, denoted herein by MaxPicWidth and MaxPicHeight, respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3.

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY, and when ChromaFormatIdc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein by BitDepthC.

–	A subpicture count, denoted by NumSubpics.

–	Arrays of the top left X and Y positions of the subpictures, denoted herein by SubPicTopLeftX[ i ] and SubPicTopLeftY[ i ], respectively.

–	Arrays of the width and height of the subpictures, denoted herein by SubPicTopLeftX[ i ] and SubPicTopLeftY[ i ], respectively.

If this SEI message is present in any picture unit that is not the first picture unit of a CLVS in decoding order, a constituent rectangles SEI message with the same payload content shall be present in the first picture unit of the CLVS in decoding order.

cr_num_rects_minus1 plus 1 specifies the number of constituent rectangles for which information is signalled in the SEI message.

cr_rect_id_present_flag equal to 1 specifies that the cr_rect_id[ i ] syntax element is present in the SEI message. cr_rect_id_present_flag equal to 0 specifies that the cr_rect_id[ i ] syntax element is not present in the SEI message.

cr_rect_id_len specifies the length of the cr_rect_id[ i ] syntax element.

cr_rect_type_enabled_flag equal to 1 specifies that the cr_rect_type_present_flag[ i ] syntax element is present in the SEI message. cr_rect_type_enabled_flag equal to 0 specifies that the cr_rect_type_ present_flag[ i ] syntax element is not present in the SEI message.

cr_rect_type_descriptions_enabled_flag equal to 1 specifies that the cr_rect_type_description_present_flag[ i ] syntax element is present in the SEI message. cr_rect_type_descriptions_enabled_flag equal to 0 specifies that the cr_rect_type_description_present_flag[ i ] syntax element is not present in the SEI message.

cr_subpics_partitioning_flag equal to 1 indicates that the subpic partitioning parameters in the SPS are used to determine the of constituent rectangle sizes and positions. cr_subpics_partitioning_flag equal to 0 indicates that determination of the constituent rectangle sizes and positions is not based on subpic partitioning parameters in the SPS.

When cr_subpics_partitioning_flag is equal to 1, it is a requirement of bitstream conformance that NumSubpics be greater than or equal to cr_num_rects_minus1 + 1.

cr_rect_same_size_flag equal to 1 indicates that all constituent rectangles have the same size and are arranged in a grid pattern. cr_rect_same_size_flag equal to 0 indicates that the sizes of the constituent rectangles may differ.

cr_num_cols_minus1 plus 1 and cr_num_rows_minus1 plus 1 specify the number of columns and rows, respectively, of the constituent rectangle grid when cr_rect_same_size_flag equal to 1.

The variable crNumCols is set equal to cr_num_cols_minus1 + 1.

The variable crNumRows is set equal to cr_num_rows_minus1 + 1.

cr_log2_unit_size specifies a unit size used in variable calculations for the constituent rectangle parameters.

The variable crUnitSize is set equal to 1 << cr_log2_unit_size.

cr_rect_size_len_minus1 plus 1 specifies the length of syntax elements cr_rect_top_left_in_units_x[ i ], cr_rect_top_left_in_units_y[ i ], cr_rect_width_in_units_minus1[ i ], and cr_rect_height_in_units minus1[ i ].

cr_rect_type_present_flag[ i ] equal to 1 specifies that the cr_rect_type_idc[ i ] syntax element is present in the SEI message. cr_rect_type_present_flag[ i ] equal to 0 specifies that the cr_rect_type_idc[ i ] syntax element is not present in the SEI message.

cr_rect_type_idc[ i ] indicates the constituent picture type of the i-th rectangle from Table X. When not present and i equal to 0, the value of cr_rect_type_idc[ i ] is inferred to be equal to 0. When not present and i greater than 0, cr_rect_type_idc[ i ] is inferred to be equal to cr_rect_type_idc[ i − 1 ].

Table X – Mapping of cr_rect_type_idc[ i ] to the type of constituent rectangle

| cr_rect_type_idc[ i ] | Name | Type of constituent rectangle |
| --- | --- | --- |
| 0 | CR_TEXTURE | Texture |
| 1 | CR_ALPHA | Alpha plane |
| 2 | CR_DEPTH | Depth picture |
| 3 | CR_OBJECT_MASK | Object mask |
| 4 .. 127 |  | Reserved |
| 128..254 |  | Unspecified |
| 255 | CR_EMPTY | Empty |

cr_rect_id[ i ] indicates the ID of the i-th rectangle. The length of the syntax element is cr_rect_id_len bits. When not present, the value of cr_rect_id[ i ] is inferred to be equal to i.

It is a requirement of bitstream conformance that when j not equal to k, cr_rect_id[ j ] shall not be equal to cr_rect_id[ k ].

cr_rect_type_description_present_flag[ i ] equal to 1 specifies that the cr_rect_type_description[ i ] syntax element is present in the SEI message. cr_rect_type_description_present_flag[ i ] equal to 0 specifies that the cr_rect_type_description[ i ] syntax element is not present in the SEI message. When not present, the value of cr_rect_type_description_present_flag[ i ] is inferred to be equal to 0.

cr_rect_top_left_in_units_x[ i ] and cr_rect_top_left_in_units_y[ i ], when present,  indicate the horizontal and vertical positions, respectively, of the top left position of the i-th constituent picture rectangle in units.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

cr_rect_width_in_units_minus1[ i ] plus 1 and cr_rect_height_in_units_minus1[ i ] plus 1, when present, indicate the width and height, respectively, of the i-th constituent rectangle in units.  The length of the syntax elements are cr_rect_size_len_minus1 + 1.

The variables crRectTopLeftX[ i ] and crRectTopLeftY[ i ], representing the x and y location, respectively, and variables crRectWidth[ i ] and crRectHeight[ i ], representing the width and height, respectively, of the i-th constituent rectangle are derived as follows.

If cr_subpics_partitioning_flag is equal to 0 and cr_rect_same_size_flag is equal to 0, the following applies:

–	The variable crRectTopLeftX[ i ] is set equal to cr_rect_top_left_in_units_x[ i ] * crUnitSize

–	The variable crRectTopLeftY[ i ] is set equal to cr_rect_top_left_in_units_y[ i ] * crUnitSize

–	The variable crRectWidth[ i ] is set equal to ( cr_rect_width_in_units minus1 + 1 ) * crUnitSize

–	The variable crRectHeight[ i ] is set equal to ( cr_rect_height_in_units minus1 + 1 ) * crUnitSize

Otherwise, if cr_subpics_partitioning_flag is equal to 1, the following applies:

–	The variable crRectTopLeftX[ i ] is set equal to SubPicTopLeftX[ i ]

–	The variable crRectTopLeftY[ i ] is set equal to SubPicTopLeftY[ i ]

–	The variable crRectWidth[ i ] is set equal to SubPicWidth[ i ]

–	The variable crRectHeight[ i ]  is set to equal to  SubPicHeight[ i ]

Otherwise (cr_rect_same_size_flag is equal to 1), the following applies:

–	The variable crRectTopLeftX[ i ] is set equal to (i % crNumCols ) * MaxPicWidth / crNumCols

–	The variable crRectTopLeftY[ i ] is set equal to (i / crNumCols ) * MaxPicHeight / crNumRows

–	The variable crRectWidth [ i ] is set equal to MaxPicWidth / crNumCols

–	The variable crRectHeight [ i ] is set to equal to MaxPicHeight / crNumRows

When PicWidthInLumaSamples is not equal to MaxPicWidth, the following applies:

–	crRectTopLeftX[ i ] is set equal to (crRectTopLeftX[ i ] * PicWidthInLumaSamples + MaxPicWidth / 2 ) / MaxPicWidth

–	crRectWidth[ i ] is set equal to (crRectWidth[ i ] * PicWidthInLumaSamples + MaxPicWidth / 2 ) / MaxPicWidth

When PicHeightInLumaSamples is not equal to MaxPicHeight, the following applies:

–	crRectTopLeftY[ i ] is set equal to (crRectTopLeftY[ i ] * PicWidthInLumaSamples + MaxPicHeight / 2 ) / MaxPicHeight

–	crRectHeight[ i ] is set equal to (crRectHeight[ i ] * PicWidthInLumaSamples + MaxPicHeight / 2 ) / MaxPicHeight

It is a requirement of bitstream conformance that for each sample position (x, y) in the coded picture there shall be at most one rectangle, j, for which both of the following conditions apply:

–	x is in (crRectTopLeftX[ j ] .. crRectTopLeftX[ j ] + crRectWidth[ j ] - 1)

–	y is in (crRectTopLeftY[ j ] .. crRectTopLeftY[ j ] + crRectHeight[ i ] - 1)

It is a requirement of bitstream conformance that crRectTopLeftX[ i ]% SubWidthC shall be equal to 0, crRectTopLeftX [ i ] % SubHeightC shall be equal to 0, crRectWidth[ i ] ]% SubWidthC shall be equal to 0, and crRectHeight[ i ] ]% SubHeightC shall be equal to 0.

cr_bit_equal_to_zero shall be equal to 0.

cr_rect_type_description[ i ] specifies a text description of the constituent rectangle. The length of the syntax element shall be less than or equal to 4097 bytes, not including the null termination byte.

## VVC

D.12.13 Use of the constituent rectangles SEI message

For purposes of interpretation of the constituent rectangles SEI message, the following variables are specified:

–	PicWidthInLumaSamples and PicHeightInLumaSamples are set equal to pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples, respectively.

–	MaxPicWidth and MaxPicHeight are set equal to sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples, respectively.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

–	NumSubpics is set equal to sps_num_subpics_minus1 + 1

–	SubPicTopLeftX[ i ] is set equal to sps_subpic_ctu_top_left_x[ i ] * CtbSizeY

–	SubPicTopLeftY[ i ] is set equal to sps_subpic_ctu_top_left_y[ i ] * CtbSizeY

–	SubPicWidth[ i ] is set equal to (sps_subpic_width_minus1[ i ] + 1 ) * CtbSizeY − 1

–	SubPicHeight[ i ] is set equal to (sps_subpic_height_minus1[ i ] + 1 ) * CtbSizeY – 1

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).