| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0163-v2 |
| --- | --- |

| Title: | AHG12: ALF syntax modification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Franck Galpin, Guillaume Boisson, Philippe Bordes, Frédéric Lefebvre, Ismail Marzuki | Email: | franck.galpin@interdigital.com guillaume.boisson@interdigital.com philippe.bordes@interdigital.com frederic.lefebvre@interdigital.com ismail.marzuki@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

Adaptive loop filter (ALF) syntax is twofold: ALF filters are signalled in Adaptation Parameter Sets (APSs), while ALF control syntax elements are signalled at CTU level. In this contribution, modifications of both ALF filter coding and ALF control syntax are proposed. In addition, ALF RDO criterion is relaxed so that more ALF filters are signalled.

On top of ECM-12.0 with common test conditions, simulation results of the proposed methods are reported as follows:

|  | Y-BDR | U-BDR | V-BDR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| All Intra | -0.01% | 0.02% | 0.02% | 99.3% | 96.0% |
| Random Access | -0.08% | 0.04% | 0.04% | 99.1% | 99.9% |
| Low Delay B | -0.02% | 0.00% | -0.02% | 100.8% | 102.0% |

# Introduction

In ECM-12.0, ALF Luma and Chroma filters are signalled in ALF Adaptation Parameter Sets (APSs). Filter coefficients are encoded with exponential-Golomb coding for their absolute value, plus a sign bit.

Signalled Luma filters, in addition, rely on a 25-class classification. They are therefore gathered per filter set, that is: up to 25 filters, together with a mapping list that specifies which filter shall be used for each class if the filter set contains more than one filter.

An ALF APS can contain up to 4 Luma filter sets and up to 8 Chroma filters. Up to eight ALF APSs can be referenced at the same time.

At CTU level, flags signal whether signalled Luma and Chroma filters are used or not. Further indices signal which APS is used among referenced APSs, the index of the Luma filter set, and the index of the Chroma filter.

At encoder side, the decision to transmit a new ALF APS is taken accordingly with a RDO criterion at slice-level, based on the number of CTBs that would use a new filter or filter set.

# Proposed method

In this contribution, ALF syntax is modified both at APS and CTU levels.

In ALF APSs, filter indices of Luma filter sets are encoded predictively instead of independently. Also the way filter coefficients are signalled is modified with the introduction of exp-Golomb orders look-up tables (LUTs) that make the binarization more compact.

At CTU level, lists of most probable APSs, Luma filter sets and Chroma filters are maintained based on indices from neighbours.

In addition, at encoder side, ALF RDO criterion D+λR is relaxed so that more ALF filters are signalled in the bitstream.

# Simulation results

The proposed method was implemented on top of ECM-12.0 and tested following common test conditions described in JVET-AF2017.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.06% | 0.00% | 99.0% | 92.6% | 99.7% | 101.1% |
| Class A2 | -0.01% | 0.00% | 0.03% | 100.8% | 91.2% | 100.9% | 101.6% |
| Class B | -0.01% | 0.04% | 0.06% | 97.2% | 98.1% | 99.8% | 100.8% |
| Class C | -0.01% | 0.00% | 0.00% | 102.4% | 101.7% | 100.4% | 100.2% |
| Class E | -0.01% | 0.00% | 0.02% | 97.5% | 93.6% | 98.8% | 100.6% |
| Overall | -0.01% | 0.02% | 0.02% | 99.3% | 96.0% | 99.9% | 100.8% |
| Class D | -0.01% | -0.04% | -0.05% | 99.6% | 98.3% | 100.0% | 100.0% |
| Class F | -0.01% | 0.00% | 0.01% | 97.2% | 94.9% | 102.4% | 100.3% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.11% | -0.12% | 0.07% | 100.8% | 100.6% | 100.0% | 100.0% |
| Class A2 | -0.07% | 0.11% | 0.03% | 101.0% | 99.9% | 99.9% | 100.0% |
| Class B | -0.10% | 0.11% | 0.03% | 98.7% | 99.9% | 100.1% | 100.1% |
| Class C | -0.05% | 0.02% | 0.02% | 97.1% | 99.5% | 100.0% | 99.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.08% | 0.04% | 0.04% | 99.1% | 99.9% | 100.0% | 100.0% |
| Class D | -0.11% | -0.08% | 0.15% | 95.3% | 97.7% | 100.4% | 100.0% |
| Class F | -0.10% | 0.00% | -0.09% | 92.0% | 87.3% | 99.9% | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.09% | -0.03% | 0.10% | 102.3% | 101.0% | 99.5% | 99.9% |
| Class C | 0.01% | 0.26% | 0.14% | 98.4% | 102.9% | 100.0% | 100.3% |
| Class E | 0.04% | -0.29% | -0.45% | 101.7% | 102.5% | 100.5% | 99.7% |
| Overall | -0.02% | 0.00% | -0.02% | 100.8% | 102.0% | 99.9% | 100.0% |
| Class D | -0.16% | 0.51% | -0.31% | 95.9% | 102.5% | 100.0% | 100.1% |
| Class F | 0.06% | -0.30% | -0.20% | 98.8% | 101.4% | 98.3% | 100.7% |

# Conclusion

This contribution proposes modifications in ALF syntax. It is proposed to include this method in next round of EE.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).