| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0164-v1 |
| --- | --- |

| Title: | AHG9: Quality metric SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela Francesco Cricri | Email: | {jill.boyce, miska.hannuksela, francesco.cricri}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to expand the functionality of the post-processing filter gain SEI message in the TuC and rename it the quality metric SEI message. The existing post-processing filter gain SEI message signals quality gain achieved using post-processing filters, while the proposed quality metric SEI adds signalling of absolute quality metrics per CLVS and/or individual output picture and supports additional quality metrics, including user-specified metrics.

# Overview of the Proposal

A quality metric SEI message is proposed, as an expansion of the post-processing filter gain SEI message currently in the TuC.  The current post-processing filter gain SEI provides signalling of the quality gain achieved by applying an indicated post-processing filter (PPF) or an indicated PPF group. For comparison purposes, the syntax table of the existing post-processing filter gain SEI message from the TuC in JVET-AF2032-v4 is provided in section 3 of this contribution.

The proposed SEI message adds optional signaling of absolute quality metrics for the CLVS and/or individual output pictures, with a flag to indicate whether a signalled quality metric represents gain or an absolute value. Use cases for signaling absolute quality metrics include the following:

- A high complexity post-processing operation, e.g., an AI task such as facial recognition, may select the highest quality frames of a video to operate on.

- A decoder may choose not to display pictures with quality below a target quality level.

The proposed SEI message normatively references quality references metrics defined in ISO/IEC 23001-10 Carriage of timed metadata metrics of media in ISO base media file format or ISO/IEC 23001-11 Energy-Efficient Media Consumption (Green Metadata). It is noted that MPEG Green Metadata provides the capability to signal absolute quality metrics for each coded picture or subpicture but does not provide the full set of capabilities provided in the proposed SEI message.

The proposed SEI message also supports use of user-specified metrics. For user-specified metrics, signalling includes a text description of the metric, a flag to indicate whether higher or lower values of the metric correspond to increased quality, and a flag to indicate if the quality metric is a full reference metric. Providing capability for user-specified quality metrics enables signaling of proprietary or newly developed metrics, while providing useful information about them. Some image quality metrics measure error rather than quality, for which lower values indicate increased quality. Non full reference quality metrics may represent non-reference quality metrics, or may measure other characteristics, such as measuring the degree of realism of generative AI images or video, rather than measuring the impact of video compression.

When the SEI message is used to indicate post-processing gain, an additional flag indicates whether the gain is with respect to the cropped decoded picture or relative to the input of the current post-processing filter of the post-processing filter group.

The existing post-processing filter gain SEI assists receiver/decoders to decide whether or not to apply post-processing filtering to a CLVS. Expanding the SEI to include signaling of per picture metrics assists receiver/decoders to decide whether or not to apply post-processing filtering to individual pictures.

The proposed functionality is summarized below.

- Signal quality metrics per CLVS and/or cropped decoded picture

- Signal quality metrics as absolute or as relative gains for PPFs or SPO SEI processing chains (indicating PPFGs), relative to the cropped decoded picture or relative to the input of the current PPF or an indicated processing stage

- User-specified quality metrics with

  - Text description of the metric

  - Flag to indicate whether higher or lower values of the metric correspond to increased quality

  - Flag to indicate if the quality metric is a full reference metric

  - Signalling of number of bytes signalled for the metric values

- Quality metrics can be signaled for a single colour component, or for 3 components individually, or for a combination of all colour components.

# Proposed syntax & semantics

| quality_metric( payloadSize ) { | Descriptor |
| --- | --- |
| qm_metric_definitions_present_flag | u(1) |
| qm_clvs_values_present_flag | u(1) |
| qm_pic_values_present_flag | u(1) |
| qm_num_metrics_minus1 | u(4) |
| if ( qm_metric_definitions_present_flag ) { |  |
| qm_gain_enabled_flag | u(1) |
| for( i = 0; i <= qm_num_metrics_minus1; i++ ) { |  |
| if ( qm_gain_enabled_flag ) |  |
| qm_gain_flag[ i ] | u(1) |
| if ( qm_gain_flag[ i ] ) |  |
| qm_gain_reference_flag[ i ] | u(1) |
| qm_metric_type[ i ] | u(8) |
| qm_three_component_flag[ i ] | u(1) |
| if ( qm_metric_type[ i ] = = 0 ¦ ¦ qm_metric_type[ i ] >= 128) { |  |
| qm_metric_increasing_flag[ i ] | u(1) |
| qm_full_reference_flag[ i ] | u(1) |
| qm_value_len_minus1_in_bytes[ i ] | u(2) |
| qm_metric_description_present_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| qm_bit_equal_to_zero /* equal to 0 */ | f(1) |
| for( i = 0; i <= qm_num_metrics_minus1; i++ ) { |  |
| if ( qm_metric_description_present_flag[ i ] ) |  |
| qm_metric_description[ i ] | st(v) |
| if( qm_clvs_values_present_flag ) |  |
| for( i = 0; i <= qm_num_metrics_minus1; i++ ) |  |
| for( c = 0; c < ( qm_three_component_flag ? 3 : 1 ); c++ ) |  |
| qm_clvs_metric_value[ i ][ c ] | u(v) |
| if( qm_pic_values_present_flag ) |  |
| for( i = 0; i <= qm_num_metrics_minus1; i++ ) |  |
| for( c = 0; c < ( qm_three_component_flag ? 3 : 1 ); c++ ) |  |
| qm_pic_metric_value[ i ][ c ] | u(v) |
| } |  |

The quality metrics SEI message includes quality metric values indicating any of the following:

–	The quality of a single picture.

–	The mean quality of all the pictures corresponding to a CLVS.

–	The quality gain of a single picture, which is difference between the quality of a single picture relative to the quality of a gain reference picture.

–	The mean quality gain of all the pictures corresponding to a CLVS.

Use of this SEI message requires the definition of the following variables:

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	A count of pictures NumPics.

–	Lists of picture widths and heights in units of luma samples, denoted herein by PicWidth[ i ] and PicHeight[ i ], respectively, where i is in the range of 0 to NumPics − 1, inclusive.

–	A list of pictures TestPicList[ i ] where i is in the range of 0 to NumPics − 1, inclusive.

–	When any qm_gain_flag[ i ] is equal to 1, a list of gain reference pictures GainRefPicList[ i ] for i is in the range of 0 to NumPics − 1, inclusive.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

TestPicList[ i ][ cIdx ] and ReferencePicList[ i ][ cIdx ] denote the cIdx-th sample array of the i-th picture among TestPicList and ReferencePicList, respectively.

TestPicList[ i ][ cIdx ][ x ][ y ] and ReferencePicList[ i ][ cIdx ][ x ][ y ] denote the sample at location ( x, y ) within the cIdx-th sample array of the i-th picture among TestPicList and ReferencePicList, respectively, where x is in the range of 0 to ( ( cIdx = = 0 ) ? PicWidth[ i ] : PicWidth[ i ] / SubWidthC ) − 1, inclusive and y is in the range of 0 to ( ( cIdx = = 0 ) ? PicHeight[ i ] : PicHeight[ i ] / SubHeightC ) − 1, inclusive.

Let currPicIdx have a value such that the output time of TestPicList[ currPicIdx ] is equal to the output time of the current picture.

qm_metric_definitions_present_flag equal to 1 specifies that information defining the quality metrics is present. qm_metric_definitions_present_flag equal to 0 specifies that information defining the quality metrics is not present.

When this SEI message is the first quality metrics SEI message in a CLVS, in decoding order,  qm_metric_definitions_present_flag shall be equal to 1.

Otherwise, (this SEI message is not the first quality metrics SEI message in a CLVS, in decoding order,)  it is a requirement of bitstream conformance that at least one of the two following conditions shall be met:

–	qm_metric_definitions_present_flag is equal to 0

–	the values of the qm_metric_type[i], qm_three_component_flag[ i ],  qm_gain_flag[ i ], qm_gain_reference_flag[ i ], qm_metric_increasing_flag[ i ], qm_full_reference_flag[ i ], qm_value_len_minus1_in_bytes[ i ], qm_metric_description_present_flag[ i ], and qm_metric_description[ i ] syntax elements, when present,  shall be equal to the respective syntax elements in the first quality metric SEI message in the CLVS

qm_clvs_values_present_flag equal to 1 specifies that qm_clvs_metric_value[ i ][ c ] syntax elements are present. qm_clvs_values_present_flag equal to 0 specifies qm_clvs_metric_value[ i ][ c ] syntax elements are not present.

qm_pic_values_present_flag equal to 1 specifies that qm_pic_metric_value[ i ][ c ] syntax elements are present. qm_pic_values_present_flag equal to 0 specifies that qm_pic_metric_value[ i ][ c ] syntax elements are not present.

qm_num_metrics_minus1 plus 1 specifies the number of quality metric entries signaled.

qm_gain_enabled_flag equal to 1 specifies that the qm_gain_flag[ i ] syntax element is present. qm_gain_enabled_flag equal to 0 specifies that the qm_gain_flag[ i ] syntax element is not present.

qm_gain_flag[ i ] and qm_gain_reference_flag[ i ], when present, indicate the interpretation of the values of the qm_clvs_metric_value[ i ][ c ] and qm_pic_metric_value[ i ][ c ] syntax elements in clause X-X. When qm_gain_flag[ i ] and qm_gain_reference_flag[ i ] are not present, they are inferred to be equal to 0.

qm_metric_type[ i ] specifies the type of the quality metric of the i-th entry, as specified in Table X. The value of qm_metric_type[ i ] shall be in the range of 0..8, inclusive, or in the range of 128..255, inclusive, in bitstreams conforming to this version of the Specification. Values in the range of of 9..127, inclusive, for qm_metric_type[ i ] are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification.

When the value of qm_metric_type [ i ] is in the range of 9 .. 127, decoders conforming to this version of this Specification shall ignore all the syntax elements for the i-th entry in this syntax structure.

When the value of qm_metric_type[ i ] is in the range of 128 .. 255, the quality metric type is unspecified or specified by other means not specified in this Specification.

Table X. Interpretation of qm_metric_type[ i ]

| t | Description | IncreasingFlag[t] | FullReferenceFlag[t] | NumBytes[t] |
| --- | --- | --- | --- | --- |
| 0 | User-defined |  |  |  |
| 1 | PSNR [2] | 1 | 1 | 2 |
| 2 | PSNR-YUV [2] | 1 | 1 | 2 |
| 3 | SSIM [2] | 1 | 1 | 2 |
| 4 | MS-SSIM [1] | 1 | 1 | 2 |
| 5 | MOS [1] | 1 | 0 | 1 |
| 6 | wPSNR [2] | 1 | 1 | 2 |
| 7 | WS-PSNR [2] | 1 | 1 | 2 |
| 8 | MSE | 0 | 1 | 2 |
| 9..127 | Reserved |  |  |  |
| 128..255 | Unspecified |  |  |  |

qm_three_component_flag[ i ] equal to 1 indicates that 3 component values will be present for the i-th metric. qm_three_component_flag[ i ] equal to 0 indicates that a single value will be present for the i-th metric. It is a requirement of bitstream conformance that when ChromaFormatIdc is equal to 0, qm_three_component_flag[ i ] shall be equal to 0.

qm_metric_increasing_flag[ i ] equal to 1 indicates that a higher value of the i-th metric value represents an improvement in quality. qm_metric_increasing_flag[ i ] equal to 0 indicates that a lower value of the i-th metric value represents an improvement in quality. When not present, the value of qm_metric_increasing_flag[ i ] is inferred to be equal to IncreasingFlag[qm_metric_type[ i ]] in Table X.

qm_full_reference_flag[ i ] equal to 1 indicates that the quality metric is a full reference quality metric, calculated by comparison of the pictures in TestPicList with the respective quality reference pictures. qm_full_reference_flag[ i ] equal to 0 indicates that the quality metric may or may not include comparison of the pictures in TestPicList with the respective quality reference picture. When not present, the value of qm_full_reference_flag[ i ] is inferred to be equal to FullReferenceFlag[qm_metric_type[ i ]]  in Table X.

qm_value_len_minus1_in_bytes[ i ] plus 1 specifies the length in bytes of the qm_pic_metric_value[ i ][ c ] syntax element. When not present, the qm_value_len_minus1_in_bytes[ i ] is inferred to be equal to NumBytes[ [qm_metric_type[ i ] ] – 1.

qm_metric_description_present_flag[ i ] equal to 1 specifies that qm_metric_description[ i ] is present. qm_metric_description_present_flag[ i ]  equal to 0 specifies that qm_metric_description[ i ] is not present.

qm_bit_equal_to_zero shall be equal to 0.

qm_metric_description[ i ] specifies a text description of the i-th quality metric. The length of the syntax element shall be less than or equal to 4097 bytes, not including the null termination byte.

qm_clvs_metric_value[ i ][ c ] specifies the mean value of the i-th quality metric for the c-th component of the CLVS. The length of the syntax element is 8 * (qm_value_len_minus1_in_bytes[ i ] + 1) bits.

qm_pic_metric_value[ i ][ c ] specifies the value of the i-th quality metric for the c-th component of the current picture. The length of the syntax element is 8 * (qm_value_len_minus1_in_bytes[ i ] + 1) bits.

The semantics of the quality metric value are determined by the values of qm_metric_type[ i ], qm_gain_flag[ i ], and qm_gain_reference_flag[ i ].

When qm_pic_values_present_flag is equal to 1, qm_pic_metric_value[ i ][ c ] indicates the picture metric value, picMetricValue[ i ][ c ], of type qm_metric_type[ i ] as described in Table X, and the following applies:

–	If qm_gain_flag[ i ] is equal to 0, qm_pic_metric_value[ i ][ c ] has value derived by the process specified in clause X.X with testPic, picWidth, and picHeight assigned to be TestPicList[ currPicIdx ], PicWidth[ currPicIdx ], and PicHeight[ currPicIdx ], respectively.

–	Otherwise (qm_gain_flag[ i ] is equal to 1), the following applies:

–	picMetricValueTest[ i ][ c ] is set equal to picMetricValue[ i ][ c ] derived by the process specified in clause X.X with testPic, picWidth, and picHeight assigned to be TestPicList[ currPicIdx ], PicWidth[ currPicIdx ], and PicHeight[ currPicIdx ], respectively.

–	picMetricValueGainRef[ i ][ c ] is set equal to picMetricValue[ i ][ c ] derived by the process specified in clause X.X with testPic, picWidth, and picHeight assigned to be GainRefPicList[ currPicIdx ], PicWidth[ currPicIdx ], and PicHeight[ currPicIdx ], respectively.

–	qm_pic_metric_value[ i ][ c ] has the value picMetricValueTest[ i ][ c ] − picMetricValueGainRef[ i ][ c ].

When qm_clvs_values_present_flag is equal to 1, qm_clvs_metric_value[ i ][ c ] indicates the mean value of the picture metric values, listPicMetricValue[ j ][ i ][ c ], calculated over all pictures in TestPicList, of type qm_metric_type[ i ] as described in Table X, where each listPicMetricValue[ j ][ i ][ c ] is derived as follows for each value of j in the range of 0 to NumPics − 1, inclusive:

–	If qm_gain_flag[ i ] is equal to 0, listPicMetricValue[ j ][ i ][ c ] is equal to the picMetrictValue[ i ][ c ] derived by the process specified in clause X.X is performed with testPic, picWidth, and picHeight assigned to be TestPicList[ j ], PicWidth[ j ], and PicHeight[ j ], respectively.

–	Otherwise (qm_gain_flag[ i ] is equal to 1), the following applies:

–	listPicMetricValueTest[ j ][ i ][ c ] is the set equal to picMetricValue[ i ][ c ] derived by the process specified in clause X.X with testPic, picWidth, and picHeight assigned to be TestPicList[ currPicIdx ], PicWidth[ currPicIdx ], and PicHeight[ currPicIdx ], respectively.

–	picMetricValueGainRef[ j ][ i ][ c ] is the set qual to picMetricValue[ i ][ c ] derived by the process specified in clause X.X with testPic, picWidth, and picHeight assigned to be GainRefPicList[ currPicIdx ], PicWidth[ currPicIdx ], and PicHeight[ currPicIdx ], respectively.

–	qm_pic_metric_value[ j ][ i ][ c ] has the value picMetricValueTest[ j ][ i ][ c ] − picMetricValueGainRef[ j ][ i ][ c ].

## X.X Process for derivation of picMetricValue[ i ][ c ]

Inputs to this process are a tested picture testPic, a picture width picWidth in luma samples, and a picture height picHeight in luma samples.

Let a quality reference picture referencePic be the original picture that was given as input to the encoding system and has the output time equal to the output time of testPic.

testPic[ cIdx ] and referencePic[ cIdx ] denote the cIdx-th sample array of the testPic and referencePic, respectively.

testPic[ cIdx ][ x ][ y ] and referencePic[ cIdx ][ x ][ y ] denote the sample at location ( x, y ) within the cIdx-th sample array of testPic and referencePic, respectively.

The pictures quality metric, picMetricValue[ i ][ c ] is derived as follows:

When qm_metric_type[ i ] is equal to 0,

When qm_metric_increasing_flag[ i ] is equal 1, a higher value of picMetricValue[ i ][ c ] indicates testPic is of better quality than for a picture with a lower value of picMetricValue[ i ][ c ].

When qm_full_reference_flag[ i ] is equal to 1, of picMetricValue[ i ][ c ] indicates a quality metric value calculated from a comparison of testPic with referencePic.

qm_metric_description[ i ] provides a text description of the quality metric indicated by picMetricValue[ i ][ c ].

Additional interpretation of picMetricValue[ i ][ c ] is determined by external means not specified in this Specification.

When qm_metric_type[ i ] is equal to 1,

picMetricValue[ i ][ 0 ] is set equal to the PSNR value calculated using clauses 9.4.2 and D.2 of ISO/IEC 23001-11 [2] for the luma components of testPic and referencePic, with bit depth OrigBitDepth, width picWidth, and height picHeight.

When qm_three_component_flag[ i ] is equal to 1,

picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ]  are set equal to the PSNR values calculated using clauses 9.4.2 and D.2 of ISO/IEC 23001-11 [2] for the Cb and Cr components, respectively, of testPic and referencePic, with bit depth OrigBitDepth width picWidth/SubWidthC, and height picHeight/SubHeightC.

When qm_metric_type[ i ] is equal to 2,

picMetricValue[ i ][ 0 ]  is set equal to the value of the variable psnrYUV calculated as follows:

The variable psnrY is set equal to the PSNR value calculated using clauses 9.4.2 and D.2 of ISO/IEC 23001-11 [2]  for the luma components of testPic and referencePic, with bit depth OrigBitDepth width picWidth, and height picHeight.

The variables psnrU and psnrV are set equal to the PSNR values calculated using clauses 9.4.2 and D.2 of ISO/IEC 23001-11 [2] for the Cb and Cr components, respectively, of testPic and referencePic, with bit depth OrigBitDepth, width picWidth/SubWidthC, and height picHeight/SubHeightC.

psnrYUV =  (10*psnrY + psnrU + psnrV ) ÷ 12

When qm_metric_type[ i ] is equal to 3,

picMetricValue[ i ][ 0 ]  is set equal to the value of SSIM calculated from clauses 9.4.2 and D.5 of ISO/IEC 23001-11 [2] for the luma components of testPic and referencePic, with bit depth OrigBitDepth width picWidth, and height picHeight.

When qm_three_component_flag[ i ] is equal to 1,

picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ]  are set equal to the SSIM values calculated from clauses 9.4.2 and D.5 of ISO/IEC 23001-11 [2] for the Cb and Cr components, respectively, of testPic and referencePic, with bit depth OrigBitDepth, width picWidth/SubWidthC, and height picHeight/SubHeightC.

When qm_metric_type[ i ] is equal to 4,

picMetricValue[ i ][ 0 ]  is set equal to the value of MS-SSIM calculated from clause 4.3.3 of ISO/IEC 23001-10 [1]  for the luma components of testPic and referencePic, with bit depth OrigBitDepth.

When qm_three_component_flag[ i ] is equal to 1,

picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ]  are set equal to the MS-SSIM values calculated from clause 4.3.3 of ISO/IEC 23001-10 [1]  for the Cb and Cr components, respectively, of testPic and referencePic, with bit depth OrigBitDepth.

When qm_metric_type[ i ] is equal to 5,

picMetricValue[ i ][ 0 ]  is set equal to the value of MOS specified in clause 4.3.6 of ISO/IEC 23001-10 [1].

When qm_metric_type[ i ] is equal to 6,

picMetricValue[ i ][ 0 ]  is set equal to the value of wPSNR calculated from clauses 9.4.2 and D.3 of ISO/IEC 23001-11 [2]  for the luma components of testPic and referencePic, with bit depth OrigBitDepth width picWidth, and height picHeight.

When qm_three_component_flag[ i ] is equal to 1,

picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ]  are set equal to the wPSNR values calculated from clauses 9.4.2 and D.3 of ISO/IEC 23001-11 [2] for the Cb and Cr components, respectively, of testPic and referencePic, with bit depth OrigBitDepth, width picWidth/SubWidthC, and height picHeight/SubHeightC.

When qm_metric_type[ i ] is equal to 7,

picMetricValue[ i ][ 0 ]  is set equal to the value of WS-PSNR calculated from clauses 9.4.2 and D.4 of ISO/IEC 23001-11 [2] for the luma components of testPic and referencePic, with bit depth OrigBitDepth, width picWidth, and height picHeight.

When qm_three_component_flag[ i ] is equal to 1,

picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ]  are set equal to the WS-PSNR values calculated from clauses 9.4.2 and D.4 of ISO/IEC 23001-11 [2]for the Cb and Cr components, respectively, of testPic and referencePic, with bit depth OrigBitDepth, width picWidth/SubWidthC, and height picHeight/SubHeightC.

When qm_metric_type[ i ] is equal to 8,

picMetricValue[ i ][ 0 ]  is set equal to the value of lumaMse, interpreted as a floating-point value, derived as follows:

lumaSse = 0

for( y = 0; y < picHeight; y++)
			for( x = 0; x < picWidth; x++ ) 
				lumaSse += (testPic[ 0 ][ x ][ y ] – referencePic[ 0 ][ x ][ y ])2

lumaMse = ( lumaSse ÷ (CroppedHeight * CroppedWidth) )/ 100

When qm_three_component_flag[ i ] is equal to 1, picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ]  are set equal to the values of CbMse and CrMse, respectively, interpreted as floating-point values, derived as follows:

CbSse = 0

CrSse = 0

for( y = 0; y < picHeight / SubWidthC; y++)
			for( x = 0; x < picWidth / SubWidthC; x++ ) {
				CbSse += (testPic[1][y][x] – referencePic[1][y][x])2

CbSse += (testPic[1][y][x] – referencePic[1][y][x])2

}

CbMse = ( CbSse ÷ ( picHeight * CroppedWidth / (SubWidthC * SubWidthC) ) / 100

CrMse = ( CrSse ÷ ( picHeight * CroppedWidth / (SubWidthC * SubWidthC) ) / 100

References:

[1] ISO/IEC 23001-10 Carriage of timed metadata metrics of media in ISO base media file format

[2] ISO/IEC  23001-11 Energy-Efficient Media Consumption (Green Metadata)

Text to be included in VVC

D.12.xx Use of the quality metric SEI message

For purposes of interpretation of the quality metric SEI message, the derivation of the variables ChromaFormatIdc, NumPics, TestPicList, PicWidth, PicHeight, and GainRefPicList is specified below.

Let TestPicList initially consist of the cropped decoded pictures of the current CLVS in output order.

When the quality metric SEI message is included as a type of an SEI message in an SEI processing order SEI message, any quality metric SEI message that is associated with the SEI processing order SEI message shall be included in a processing order nesting SEI message.

When the quality metric SEI message is included as the i-th type of an SEI message in an SEI processing order SEI message, the following applies:

–	It is a requirement of bitstream conformance that an SEI message seiB that implies post-processing to be performed shall be present as the j-th type of an SEI message in the same SEI processing order SEI message and po_sei_processing_order[ j ] shall be equal to po_sei_processing_order[ i ].

–	The quality metric SEI message indicates the picture quality resulting from the post-processing implied by seiB.

–	TestPicList is updated as follows for each post-processing stage with po_sei_processing_order[ j ] less than or equal to po_sei_processing_order[ i ] in a non-decreasing order of j.

–	When the post-processing stage results into a picture picA with an output time that is equal to the output time of a picture picB in TestPicList, picB in TestPicList is replaced by picA.

–	When the post-processing stage results into a picture picA with an output time that is not equal to the output time of any picture in TestPicList, picA is inserted in TestPicList in a manner that pictures in TestPicList remain in output order.

NumPics is set equal to the count of pictures in TestPicList.

PicWidth[ i ] and PicHeight[ i ] are set equal to the width and height of TestPicList[ i ], respectively, in luma samples.

When a quality metric SEI message is included as the k-th SEI message in a processing order nesting SEI message and qm_gain_flag[ i ] is equal to 1 for any value of i, the following applies:

–	If qm_gain_reference_flag[ i ] is equal to 0, the i-th metric value in the quality metric SEI message represents a gain of the post-processing stage with po_sei_processing_order[ j ] equal to pon_processing_order[ k ] relative to the picture or pictures used as input to that post-processing stage and GainRefPicList is set equal to TestPicList derived for the processing stages up to but not including the processing stage with po_sei_processing_order[ j ].

–	Otherwise (qm_gain_reference_flag[ i ] is equal to 1), the i-th metric value in the quality metric SEI message represents a cumulative gain of all the post-processing stages with po_sei_processing_order[ j ] less than or equal to pon_processing_order[ k ] relative to the cropped decoded picture or pictures and GainRefPicList consists of the cropped decoded pictures of the current CLVS in output order.

–	It is a requirement of bitstream conformance that the count of pictures in GainRefPicList shall be equal to NumPics and the width and height of GainRefPicList[ i ] in luma samples shall be equal to PicWidth[ i ] and PicHeight[ i ], respectively.

The value of ChromaFormatIdc is derived as follows:

–	If the quality metric SEI message is not included in a processing order nesting SEI message, ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	Otherwise (the quality metric SEI message is included in a processing order nesting SEI message), ChromaFormatIdc is set to a value that matches the chroma format of the pictures in TestPicList and it is a requirement of bitstream conformance that the chroma format of all the pictures in TestPicList and GainRefPicList, when present, shall be identical.

When a quality metric SEI message qmSeiA is present in a picture unit that is not the first picture unit of a CLVS in decoding order and at least one value of qm_clvs_metric_value[ i ][ c ] is present, the following applies:

–	If qmSeiA is not included in a processing order nesting SEI message, each value of qm_clvs_metric_value[ i ][ c ] shall be equal to the value of qm_clvs_metric_value[ i ][ c ] in the quality metric SEI message that is present in the first picture unit of the CLVS and is not is included in a processing order nesting SEI message.

–	Otherwise (qmSeiA is included in a processing order nesting SEI message with a certain set of pon_target_po_id[ i ] values), the following applies:

–	It is a requirement of bitstream conformance that there shall be quality metric SEI message qmSeiB that is present in the first picture unit of the CLVS and is included in a processing order nesting SEI message with the same set of pon_target_po_id[ i ] values.

–	Each value of qm_clvs_metric_value[ i ][ c ] in qmSeiA shall be equal to the value of qm_clvs_metric_value[ i ][ c ] in qmSeiB.

# Syntax of post-processing filter gain SEI

For comparison purposes, the syntax of the post-processing filter gain SEI in JVET-AF2032-v4 is provided below.

| post_processing_filter_gain( payloadSize ) { | Descriptor |
| --- | --- |
| pf_gain_id | ue(v) |
| pf_gain_num_entries_minus1 | ue(v) |
| for( i = 0; i <= pf_gain_num_entries_minus1; i++ ) |  |
| pf_gain_metric_idc[ i ] | ue(v) |
| for( i = 0; i <= pf_gain_num_entries_minus1; i++ ) { |  |
| pf_gain_per_component_flag[ i ] | u(1) |
| if( pf_gain_per_component_flag[ i ] ) { |  |
| pf_gain_y_flag[ i ] | u(1) |
| pf_gain_cb_flag[ i ] | u(1) |
| if( pf_gain_y_flag[ i ] + pf_gain_cb_flag[ i ] > 0 ) |  |
| pf_gain_cr_flag[ i ] | u(1) |
| } |  |
| pf_gain_word_cnt_minus1[ i ] | ue(v) |
| for( j = 0; j < pfGainNumValuesPerEntry[ i ]; j++ ) |  |
| pf_gain_value[ i ][ j ] | u(v) |
| } |  |
| } |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).