| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17-24 April 2024 | Document: JVET-AH0166-v1 |
| --- | --- |

| Title: | AHG9: Improvements to the film grain regions characteristics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shaowei Xie Ping Wu Ying Gao Yaxian Bai Menghu Jia Weihong Niu Cheng Huang | Email: | {xie.shaowei, ping.wu, huang.cheng5}@zte.com.cn |
| Source: | ZTE |  |  |

_____________________________

- Abstract

In last meeting, the film grain regions characteristics SEI message was adopted into the TuC for future extensions of VSEI (version 3) [1]. In order to improve the adaptability of this SEI message, two comments about region adaptive film grain model application are proposed, wherein:

- Allowing separate “fgr_model_id” to be applied in different regions, instead of restricting only one single “fgr_model_id” in the whole SEI message.

- Allowing separate “fgr_blending_mode_id” to be applied in different regions, instead of restricting only one single “fgr_blending_mode_id” in the whole SEI message.

Introduction

In the 33rd JVET meeting, the unified contribution JVET-AG0328 [2] proposed a region-dependent film grain characteristics SEI message, that includes the existing FGC SEI message syntax and adds elements to describe a list of picture regions, where each region is associated with a set of intensity intervals. As recorded in the meeting report [3], it was agreed to document this proposed SEI message in the TuC.

However, in the new film grain regions characteristics SEI message, only one single “fgr_model_id” and one single “fgr_blending_mode_id” could be used for different film grain regions in the whole SEI message. It can be mentioned that the document JVET-AC0199 [4] provides the film grain synthesis evaluation results of both the frequency-based method and the auto-regressive based method. The observations in JVET-AC0199 were summarized into a few bullet points. Particularly, it commented: the frequency based FGS (film grain synthesis) method and auto-regressive (AR) based method show different results under different circumstances; how to select the proper method for a film grain test video is important. With current two models in use, there might be a need to even switch the model inside the same picture.

In order to improve the adaptability of the film grain regions characteristics SEI message, two comments about region adaptive film grain model application are proposed in this proposal.

Proposed Solution

It is proposed to extend the functionality of the film grain regions characteristics SEI message, through the following two improvement approaches:

- Allowing separate “fgr_model_id” to be applied in different regions, instead of restricting only one single “fgr_model_id” in the whole SEI message.

- Allowing separate “fgr_blending_mode_id” to be applied in different regions, instead of restricting only one single “fgr_blending_mode_id” in the whole SEI message.

Proposal

On the basis of the TuC for future extensions of VSEI (version 3) [1], we propose to modify the syntax structure of the film grain regions characteristics SEI message in clause 8.38, as follows. It should be noted that, the separate proposals could be applied individually or jointly.

Proposal #1: Allowing separate “fgr_model_id”

8.38 Film grain regions characteristics SEI message

8.38.1 Film grain regions characteristics SEI message syntax

| film_grain_regions_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| fgr_cancel_flag | u(1) |
| if( !fgr_cancel_flag ) { |  |
| fgr_single_model_flag | u(1) |
| if (fgr_single_model_flag) { |  |
| fgr_model_id | u(2) |
| } |  |
| fgr_separate_colour_description_present_flag | u(1) |
| if( fgr_separate_colour_description_present_flag ) { |  |
| fgr_bit_depth_luma_minus8 | u(3) |
| fgr_bit_depth_chroma_minus8 | u(3) |
| fgr_full_range_flag | u(1) |
| fgr_colour_primaries | u(8) |
| fgr_transfer_characteristics | u(8) |
| fgr_matrix_coeffs | u(8) |
| } |  |
| fgr_blending_mode_id | u(2) |
| fgr_log2_scale_factor | u(4) |
| for( c = 0; c < 3; c++ ) |  |
| fgr_comp_model_present_flag[ c ] | u(1) |
| for( c = 0; c < 3; c++ ) |  |
| if( fgr_comp_model_present_flag[ c ] ) { |  |
| fgr_num_intensity_intervals_minus1[ c ] | u(8) |
| fgr_num_model_values_minus1[ c ] | u(3) |
| for( i = 0; i <= fgr_num_intensity_intervals_minus1[ c ]; i++ ) { |  |
| fgr_intensity_interval_lower_bound[ c ][ i ] | u(8) |
| fgr_intensity_interval_upper_bound[ c ][ i ] | u(8) |
| for( j = 0; j <= fgr_num_model_values_minus1[ c ]; j++ ) |  |
| fgr_comp_model_value[ c ][ i ][ j ] | se(v) |
| } |  |
| } |  |
| fgr_region_information_present_flag | u(1) |
| if( fgr_region_information_present_flag ) { |  |
| fgr_region_based_adaptation_flag | u(1) |
| if( fgr_region_based_adaptation_flag ) { |  |
| fgr_active_regions_number_minus1 | u(8) |
| for( i = 0; i <= fgr_active_regions_number_minus1; i++ ) { |  |
| fgr_region_top[ i ] | u(16) |
| fgr_region_left[ i ] | u(16) |
| fgr_region_width[ i ] | u(16) |
| fgr_region_height[ i ] | u(16) |
| if (!fgr_single_model_flag) { |  |
| fgr_model_id[ i ] | u(2) |
| } |  |
| fgr_film_grain_enabled_flag[ i ] | u(1) |
| if ( fgr_film_grain_enabled_flag[ i ] ) { |  |
| for ( c = 0; c < 3; c++ ) |  |
| if ( fgr_comp_model_present_flag[ c ] ) { |  |
| fgr_region_interval_min[ i ][ c ] | u(8) |
| fgr_region_interval_max[ i ][ c ] | u(8) |
| } |  |
| } |  |
| } |  |
| } |  |
| else |  |
| fgr_alpha_channel_adaptation_flag | u(1) |
| } |  |
| fgr _persistence_flag | u(1) |
| } |  |
| } |  |

8.38.2 Film grain regions characteristics SEI message semantics

fgr_single_model_flag equal to 1 indicates that the film grain simulation models used in current picture have the same fgr_model_id. fgr_single_model_flag equal to 0 indicates that the film grain simulation models used in different regions of the current picture may have different fgr_model_id.

fgr_model_id identifies the film grain simulation model as specified in Table 5. The value of fgr_model_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for fgr_model_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain regions characteristics SEI messages with fgr_model_id equal to 2 or 3.

Table 5 – fgr_model_id values

| Value | Description |
| --- | --- |
| 0 | Frequency filtering |
| 1 | Auto-regression |

Proposal #2: Allowing separate “fgr_blending_mode_id”

8.38 Film grain regions characteristics SEI message

8.38.1 Film grain regions characteristics SEI message syntax

| film_grain_regions_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| fgr_cancel_flag | u(1) |
| if( !fgr_cancel_flag ) { |  |
| fgr_model_id | u(2) |
| fgr_separate_colour_description_present_flag | u(1) |
| if( fgr_separate_colour_description_present_flag ) { |  |
| fgr_bit_depth_luma_minus8 | u(3) |
| fgr_bit_depth_chroma_minus8 | u(3) |
| fgr_full_range_flag | u(1) |
| fgr_colour_primaries | u(8) |
| fgr_transfer_characteristics | u(8) |
| fgr_matrix_coeffs | u(8) |
| } |  |
| fgr_single_blending_mode_flag | u(1) |
| if (fgr_single_blending_mode_flag) { |  |
| fgr_blending_mode_id | u(2) |
| } |  |
| fgr_log2_scale_factor | u(4) |
| for( c = 0; c < 3; c++ ) |  |
| fgr_comp_model_present_flag[ c ] | u(1) |
| for( c = 0; c < 3; c++ ) |  |
| if( fgr_comp_model_present_flag[ c ] ) { |  |
| fgr_num_intensity_intervals_minus1[ c ] | u(8) |
| fgr_num_model_values_minus1[ c ] | u(3) |
| for( i = 0; i <= fgr_num_intensity_intervals_minus1[ c ]; i++ ) { |  |
| fgr_intensity_interval_lower_bound[ c ][ i ] | u(8) |
| fgr_intensity_interval_upper_bound[ c ][ i ] | u(8) |
| for( j = 0; j <= fgr_num_model_values_minus1[ c ]; j++ ) |  |
| fgr_comp_model_value[ c ][ i ][ j ] | se(v) |
| } |  |
| } |  |
| fgr_region_information_present_flag | u(1) |
| if( fgr_region_information_present_flag ) { |  |
| fgr_region_based_adaptation_flag | u(1) |
| if( fgr_region_based_adaptation_flag ) { |  |
| fgr_active_regions_number_minus1 | u(8) |
| for( i = 0; i <= fgr_active_regions_number_minus1; i++ ) { |  |
| fgr_region_top[ i ] | u(16) |
| fgr_region_left[ i ] | u(16) |
| fgr_region_width[ i ] | u(16) |
| fgr_region_height[ i ] | u(16) |
| if (!fgr_single_blending_mode_flag) { |  |
| fgr_blending_mode_id[ i ] | u(2) |
| } |  |
| fgr_film_grain_enabled_flag[ i ] | u(1) |
| if ( fgr_film_grain_enabled_flag[ i ] ) { |  |
| for ( c = 0; c < 3; c++ ) |  |
| if ( fgr_comp_model_present_flag[ c ] ) { |  |
| fgr_region_interval_min[ i ][ c ] | u(8) |
| fgr_region_interval_max[ i ][ c ] | u(8) |
| } |  |
| } |  |
| } |  |
| } |  |
| else |  |
| fgr_alpha_channel_adaptation_flag | u(1) |
| } |  |
| fgr _persistence_flag | u(1) |
| } |  |
| } |  |

8.38.2 Film grain regions characteristics SEI message semantics

fgr_single_blending_mode_flag equal to 1 indicates that the blending modes used to blend the simulated film grains with the input image regions have the same fgr_blending_mode_id. fgr_single_blending_mode_flag equal to 0 indicates that the blending modes used to blend the simulated film grains with the input image regions may have different fgr_blending_mode_id.

fgr_blending_mode_id identifies the blending mode used to blend the simulated film grain with the input images as specified in Table 6. fgr_blending_mode_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for fgr_blending_mode_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with fgr_blending_mode_id equal to 2 or 3.

Table 6 – fgr_blending_mode_id values

| Value | Description |
| --- | --- |
| 0 | Additive |
| 1 | Multiplicative |

Reference

- Sean McCarthy, Jie Chen, Sachin Deshpande, Miska M. Hannuksela Hendry, Technologies under consideration for future extensions of VSEI (version 3), JVET-AG2032, Mar. 2024.

- Stephan Wenger, Arianne Hinds, Gilles Teniou (Tencent), P. de Lagrange, E. François, D. Doyen, M. Le Pendu, C. Salmon-Legagneur (Interdigital), Ying Gao, Shaowei Xie, Yaxian Bai, Menghu Jia, Cheng Huang, Ping Wu (ZTE), AHG9/AHG13: unified proposal on film grain regions characteristics SEI message, JVET-AG0328, Jan. 2024.

- Jens-Rainer Ohm, Meeting Report of the 33rd JVET Meeting, AG1000, Feb. 2024.

- Xuewei Meng, Wenhao Zhang, Scott Labrozzi, AHG13: On Film Grain Synthesis Subjective Evaluation, JVET-AC0199, Jan. 2023.

Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).