| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0167-v1 |
| --- | --- |

| Title: | AHG9: On payload size signalling for SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Mitra Damghanian Rickard Sjöberg | Email: | martin.m.pettersson@ericsson.commitra.damghanian@ericsson.com rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to conditionally signal the payload size for SEI messages. Three options are proposed where option 1 proposes modifications to the SEI signalling in the TuC, option 2 claims to be a backwards compatible VVC modification to sei_message(), and option 3 targets the next generation video codec:

- Add a vsei_payload_size_flag to a claimed harmonized solution of the large SEI message and the versatile RBSP SEI solutions from the TuC. When vsei_payload_size_flag is equal to 1 the payload size is signalled with a 2-byte syntax element vsei_payload_size_16bits for all SEI messages in the NAL unit. When vsei_payload_size_flag is equal to 0, the payload size is not signalled and the SEI message shall be the only SEI message in the NAL unit.

- Assign an additional payload type for each SEI message for which the payload size of the SEI message is not signalled in the SEI RBSP. Compared to the versatile SEI RBSP solution in the TuC no new NAL unit type is needed. In the proposed syntax, an additional payload type for which payload size syntax elements are not present has been added for all existing SEI messages in VVC, but it can be discussed for which SEI messages an additional payload type should be added, e.g., only for large SEI messages or only for SEI messages that are new to VVC v4.

- Add a payload_size_flag syntax element in sei_message() that specifies whether the payload size of the SEI message is signalled in the RBSP or not. In this option, the first syntax element for the payload type is signalled with 7 bits.

It is proposed to include one or more of the three options to the TuC for future extensions of VSEI.

# Introduction

The TuC for future extensions of VSEI (JVET-AG2032) includes two solutions for reducing the payload size signalling for SEI messages, the large SEI message proposed in JVET-AF0148, and the versatile SEI RBSP proposed in JVET-AG0051.

The large SEI message solution uses 16 bits for the payload size to be used for large SEI messages instead of the mechanism used in the sei_message( ) structure in VVC that uses one byte for signalling the payload size for each 255 bytes of payload. The large SEI message in the TuC has the following syntax:

| lsei_message( ) { | Descriptor |
| --- | --- |
| lsei_position | u(2) |
| lsei_relevance | u(2) |
| lsei_reserved | u(4) |
| lsei_payload_type_byte | u(8) |
| lsei_payload_size_16bits | u(16) |
| lsei_payload( lseiPayloadType, lseiPayloadSize ) |  |
| } |  |

The versatile SEI RBSP solution omits the signalling of the payload size in the RBSP, with the motivation that all other RBSPs of VVC do not signal the payload size prior to the payload. The versatile SEI RBSP may be carried in a NAL unit. For existing codecs, a new NAL unit type may be added for each of prefix and suffix SEI messages. The versatile SEI RBSP solution in the TuC has the following syntax:

| vsei_rbsp( ) { | Descriptor |
| --- | --- |
| vsei_importance | u(1) |
| vsei_reserved | u(7) |
| versatile_sei_message( ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| versatile_sei_message( ) { | Descriptor |
| --- | --- |
| vsei_payload_type_byte | u(8) |
| vsei_payload( vseiPayloadType ) |  |
| } |  |

# Discussion

The large SEI message in the TuC targets reducing the payload size signalling for large SEI messages. However, the overall compression of the bitstream would also benefit from not having to signal the payload size for any SEI message when not needed.

In VVC the payload size is needed for SEI messages carried in the scalable nesting SEI message in order to properly parse the SEI messages. The scalable nesting SEI message is not only useful for reducing the bitrate by not having to send the NAL unit header for each nested SEI message, but is also intended for grouping SEI messages and provides a mechanism to associate SEI messages with specific OLSs, specific layers, or specific sets of subpictures. Versatile SEI messages in the versatile SEI RBSP solution in the TuC would not be supported by the scalable nesting SEI message since the payload size is not signaled.

# Proposal

It is proposed to conditionally signal the payload size for SEI messages. By allowing the payload size to be signalled, the SEI message may be carried in a scalable nesting SEI message. Three options are proposed where option 1 proposes modifications to the SEI signalling in the TuC, option 2 claims to be a backwards compatible VVC modification to sei_message(), and option 3 targets the next generation video codec:

- Add a vsei_payload_size_flag to a claimed harmonized solution of the large SEI message and the versatile RBSP SEI solutions from the TuC. When vsei_payload_size_flag is equal to 1 the payload size is signalled with a 2-byte syntax element vsei_payload_size_16bits for all SEI messages in the NAL unit. When vsei_payload_size_flag is equal to 0, the payload size is not signalled and the SEI message shall be the only SEI message in the NAL unit.

- Assign an additional payload type for each SEI message for which the payload size of the SEI message is not signalled in the SEI RBSP. Compared to the versatile SEI RBSP solution in the TuC no new NAL unit type is needed. In the proposed syntax, an additional payload type for which payload size syntax elements are not present has been added for all existing SEI messages in VVC, with new payload type values in the range between 70 and 112,  but it can be discussed for which SEI messages an additional payload type should be added, e.g. only for large SEI messages or only for SEI messages that are new to VVC v4.

- Add a payload_size_flag syntax element in sei_message() that specifies whether the payload size of the SEI message is signalled in the RBSP or not. In this option, the first syntax element for the payload type is signalled with 7 bits.

It is proposed to include one or more of the three options to the TuC for future extensions of VSEI.

## Option 1: Harmonization of large SEI message and versatile SEI RBSP

In this first option, lsei_message() and versatile_sei_message() from the TuC are merged into a single syntax structure where vsei_payload_size_flag equal to 1 supports the current lsei_message() features and vsei_payload_size_flag equal to 0 supports versatile_sei_message().

Syntax and semantics for option 1 are shown below with additions on top of the TuC text in JVET-AG2032 marked in yellow and removed text marked with red strikethrough.

| vsei_rbsp( ) { | Descriptor |
| --- | --- |
| vsei_position | u(2) |
| vsei_importance | u(1) |
| vsei_relevance | u(2) |
| vsei_payload_size_flag | u(1) |
| vsei_reserved | u(73) |
| do |  |
| versatile_sei_message( ) |  |
| while( vsei_payload_size_flag && more_rbsp_data( ) ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

lvsei_position indicates if the SEI message corresponds to the PREFIX_SEI_NUT and SUFFIX_SEI_NUT.  lvsei_position equal 0 indicates that the SEI message is treated as PREFIX_SEI_NUT. lvsei_position equal 1 indicates that the SEI message is treated as SUFFIX_SEI_NUT. Values 3 and 4 of lvsei_position are reserved for future use and shall be ignored.

lvsei_relevance indicates the relevance of the SEI message for the target application. lvsei_relevance ranges from 0 to 3, with 0 being the least relevant and 3 being the most relevant. vsei_relevance equal to 3 indicates that the versatile SEI message may be a required versatile SEI message.

vsei_payload_size_flag equal to 0 specifies that no vsei_payload_size_16bits syntax element is present in the versatile SEI message. vsei_payload_size_flag equal to 1 specifies that one or more vsei_payload_size_16bits syntax elements are present in the SEI message. When vsei_payload_size_flag is equal to 0, the SEI message shall be the only SEI message in the NAL unit.

| versatile_sei_message( ) { | Descriptor |
| --- | --- |
| vsei_payload_type_byte | u(8) |
| vsei_payload( vseiPayloadType ) |  |
| } |  |

| lseiversatile_sei_message( ) { | Descriptor |
| --- | --- |
| lsei_position | u(2) |
| lsei_relevance | u(2) |
| lsei_reserved | u(4) |
| lvsei_payload_type_byte | u(8) |
| if( vsei_payload_size_flag ) |  |
| vsei_payload_size_16bits | u(16) |
| lvsei_payload(lseiPayloadType, lseiPayloadSize, vsei_payload_type_byte, vsei_payload_size_16bits ) |  |
| } |  |

vsei_payload_size_16bits isspecifies the payload size in bitsbytes of a largeversatile SEI message. payloadSize = payload_size_16bits. When vsei_payload_size_16bits is not present it is inferred to be equal to NumBytesInNalUnit – 4.

## Option 2: New payload types for not signalling payload size

Syntax and semantics for option 2 are shown below with additions on top of VVC v3 marked in yellow.

| sei_message( ) { | Descriptor |
| --- | --- |
| payloadType = 0 |  |
| do { |  |
| payload_type_byte | u(8) |
| payloadType += payload_type_byte |  |
| } while( payload_type_byte = = 0xFF ) |  |
| if ( payloadType < 70 ¦¦ payloadType > 112 ) { |  |
| payloadSize = 0 |  |
| do { |  |
| payload_size_byte | u(8) |
| payloadSize += payload_size_byte |  |
| } while( payload_size_byte = = 0xFF ) |  |
| } else |  |
| payloadSize = NumBytesInNalUnit – 3 |  |
| } |  |
| sei_payload( payloadType, payloadSize ) |  |
| } |  |

payload_size_byte is a byte of the payload size of an SEI message. When no payload_size_byte syntax element is present in the SEI message, the SEI message shall be the only SEI message in the NAL unit.

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| SeiExtensionBitsPresentFlag = 0 |  |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ¦¦ payloadType = = 70 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ¦¦ payloadType = = 71 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 3 ¦¦ payloadType = = 72) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ¦¦ payloadType = = 73) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ¦¦ payloadType = = 74) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 19 ¦¦ payloadType = = 75) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| film_grain_characteristics( payloadSize ) |  |
| … // and so on |  |
| } |  |
| } |  |

## Option 3: For next generation codec

Syntax and semantics for option 3 are shown below with additions on top of VVC v3 marked in yellow and removed text marked with red strikethrough.

| sei_message( ) { | Descriptor |
| --- | --- |
| payloadType = 0 |  |
| payload_size_flag | u(1) |
| payload_type_7_bits | u(7) |
| payloadType = payload_type_7_bits |  |
| if ( payload_type_7_bits = = 0x7F ) { |  |
| do { |  |
| payload_type_byte | u(8) |
| payloadType += payload_type_byte |  |
| } while( payload_type_byte = = 0xFF ) |  |
| } |  |
| payloadSize = 0 |  |
| if (payload_size_flag) { |  |
| do { |  |
| payload_size_byte | u(8) |
| payloadSize += payload_size_byte |  |
| } while( payload_size_byte = = 0xFF ) |  |
| } else |  |
| payloadSize = NumBytesInNalUnit – 3 |  |
| sei_payload( payloadType, payloadSize ) |  |
| } |  |

payload_size_flag equal to 0 specifies that no payload_size_byte syntax element is present in the SEI message. payload_size_flag equal to 1 specifies that one or more payload_size_byte syntax elements are present in the SEI message. When payload_size_flag is equal to 0, the SEI message shall be the only SEI message in the NAL unit.

payload_type_7_bits is a value of the payload type of an SEI message.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).