| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0171-v2 |
| --- | --- |

| Title: | AHG10/AHG12 GOP-based RPR encoder control results on UHD test data |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Ström, P. Wennersten, W. Ahmad, C. Hollmann | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

Encoding in reduced resolution has been shown to be effective especially for higher resolution video and coding at challenging bitrates. A GOP-based RPR encoder control is present in both VTM and ECM.

This document reports objective performance results when deploying GOP-based RPR on VTM-23.0 compared to VTM-23.0 without that enabled on a set of sequences from JVET-AH0041 for random access:

BDR 4K (PSNR):    -2.97%(Y),  -6.46%(U),   -1.64%(V)

BDR 4K (MSSIM): -6.61%(Y), -14.25%(U), -10.67%(V)

It is also reported performance when deploying GOP-based RPR on ECM-12.0 (commit 6da105bb) compared to ECM-12.0 without that enabled on a set of sequences from JVET-AH0041 for random access:

BDR 4K (PSNR):    -0.15%(Y),  5.43%(U),   11.73%(V)

BDR 4K (MSSIM): -3,35%(Y), -4,91%(U), -3,51%(V)

It is also asserted that subjective quality can be improved.

It is suggested that GOP-based RPR is included in next version of the VTM11_ANC to be aligned with corresponding functionality in ECM and VTM.

# Introduction

A GOP-based RPR encoder control is present in both VTM and ECM but is missing in VTM11_ANC. It selects to encode a segment of the video in reduced resolution based on rescaling performance.

# Proposal

Add the GOP based RPR functionality from VTM to VTM11_ANC.

# Results

## VTM-23.0

GOP-based RPR is enabled in VTM-23.0 by setting the configuration parameter GOPBasedRPR to 1.

### PSNR

|  | Y | U | V |
| --- | --- | --- | --- |
| Procession_3840x2160_60p_420_10 | -5,43% | -9,50% | -12,44% |
| BeachMountain2_3840x2160_30fps_ | -3,72% | -28,48% | -21,92% |
| CrossRoad2s500_3840x2160_50fps_ | -0,09% | 23,79% | 26,62% |
| DrivingPOV3_3840x2160_60fps_10b | -2,62% | -11,67% | 1,17% |
| Average | -2,97% | -6,46% | -1,64% |

### MSSSIM

|  | Y | U | V |
| --- | --- | --- | --- |
| Procession_3840x2160_60p_420_10 | -7,79% | -9,51% | -11,30% |
| BeachMountain2_3840x2160_30fps_ | -4,57% | -26,62% | -22,63% |
| CrossRoad2s500_3840x2160_50fps_ | -8,27% | -6,61% | -4,49% |
| DrivingPOV3_3840x2160_60fps_10b | -5,83% | -14,25% | -4,25% |
| Average | -6,61% | -14,25% | -10,67% |

## ECM

Here GOP-based RPR is enabled (GOPBasedRPR=1) on commit 6da105bb since it was some bugs that caused troubles for RPR in random access. The anchor is still ECM-12.0.

### PSNR

|  | Y | U | V |
| --- | --- | --- | --- |
| Procession_3840x2160_60p_420_10 | -2,32% | -5,72% | -5,39% |
| BeachMountain2_3840x2160_30fps_ | -1,42% | -7,62% | -7,67% |
| CrossRoad2s500_3840x2160_50fps_ | 2,66% | 37,56% | 47,14% |
| DrivingPOV3_3840x2160_60fps_10b | 0,48% | -2,50% | 12,84% |
| Average | -0,15% | 5,43% | 11,73% |

### MSSSIM

|  | Y | U | V |
| --- | --- | --- | --- |
| Procession_3840x2160_60p_420_10 | -4,49% | -8,57% | -6,88% |
| BeachMountain2_3840x2160_30fps_ | -1,85% | -4,78% | -9,16% |
| CrossRoad2s500_3840x2160_50fps_ | -5,69% | -2,16% | 0,34% |
| DrivingPOV3_3840x2160_60fps_10b | -1,37% | -4,12% | 1,68% |
| Average | -3,35% | -4,91% | -3,51% |

# Conclusions

It has been shown that GOP-based RPR encoder control can provide significant objective gains on VTM-23.0 when applied for random access on a set of UHD test sequences in JVET-AH0041. Some gains are also achieved on ECM but to a lesser extent.

It is suggested to include GOP-based RPR to the VTM11_ANC to be aligned with ECM.

# References

[1] M. Wien, “AHG4: AHG meeting report on ECM expert viewing preparations”, JVET-AH0041, Rennes France, April 2024.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).