| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0173-v1 |
| --- | --- |

| Title: | AHG9: SEI message for International Color Consortium (ICC) profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arianne T. Hinds Stephan Wenger Gilles Teniou | Tel: Email: | { ahinds, swenger, teniou } @global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Contribution JVET-AF0141, which was adopted at the 32nd JVET meeting, proposed that 3 commonly-used image metadata formats be supported as SEI messages. One of the remaining commonly-used image metadata formats that could supported, also in an SEI message, is the profile of the International Color Constortium (ICC). ICC profiles have been codified as ISO standards (see ISO 15076-1:2010, ISO 15076-2:202X, and ISO 20677:2019). This contribution proposes that an SEI message be added for the carriage of ICC profile metadata, which is commonly provided as metadata by digital cameras (including mobile devices).

# Introduction

The International Color Consortium (ICC) was established in 1993 by a group of vendors with the mission to develop an open, vendor-neutral, cross-platform color management system. Such a system endeavored to enable the exchange of imagery across multiple platforms, applications, and vendors with the ability to preserve color fidelity. The results of the ICC have been (to date) 3 generations of ICC profiles: “v2”, which is published as ISO 15076-1:2005, “v4” which is published as ISO 15076-1:2010 and revised in ISO 15076-2:2024, and “iccMAX”, which is published as ISO 20677:2019.  Each of these profile generations are self-describing in a well-defined header structure so that they may be distinguished, one from another.

A number of white papers on the topics of color management workflows, color fundamentals, differences between the profile generations, different types of rendering ,are available from the ICC website. However, in the general context of digital video, an ICC profile describes precise color attributes of the capture system, i.e., the digital camera, and likewise, an ICC profile describes the precise color attributes of the display. In the architecture of the ICC color management system, a Profile Connection Space (PCS) is defined to enable the unambiguous translation to and from ICC profiles. Thus the use of the PCS enables unambiguous translation between ICC profiles. A more complete description of the fundamentals of ICC profiles is available at: https://www.color.org/iccprofile.xalter.

# Proposed approach

## General

As with the case of other image metadata from JVET-AF0141, the proposed approach is to create an SEI that would carry the payload of an ICC profile, i.e., to deliver the profile of the capture device to rendering applications at the receiver. The usage of both cancel and persistence flags enables the management of the duration of the SEI. Finally, the option to store the profile data at a URI is supported.

## ICC Profile SEI Syntax

| ICC_profile ( payloadSize ) { | Descriptor |
| --- | --- |
| iccp_cancel_flag | u(1) |
| if (!iccp_cancel_flag) { |  |
| iccp_persistence_flag | u(1) |
| iccp_mode_id | u(8) |
| if(iccp_mode_id == 0) { |  |
| for( i = 1; i < payloadSize; i++ ) |  |
| iccp_data_payload_byte | b(8) |
| } |  |
| else if(iccp_mode_id == 1) { |  |
| iccp_data_URI | st(v) |
| } |  |
| } |  |
| } |  |

## ICC profile SEI message semantics

The ICC profile SEI message indicates the ICC profile metadata, as specified by any of the ISO standards in which the ICC profile is specified. Currently, the relevant ISO standards include ISO 15076-1, ISO 15076-2, and ISO 20677.

iccp_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous ICC profile SEI message in output order. iccp_cancel_flag equal to 0 indicates that ICC profile information follows.

iccp_persistence_flag specifies the persistence of the ICC profile SEI message for the current layer.

iccp_persistence_flag equal to 0 specifies that the ICC profile SEI message applies to the current decoded picture only.

iccp_persistence_flag equal to 1 specifies that the ICC profile SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an ICC profile SEI message is output that follows the current picture in output order.

iccp_mode_id specifies the mode by which the ICC profile is obtained.

–	If iccp_mode_id is equal to 0, the ICC profile is obtained directly from the payload of the SEI message.

–	Otherwise if iccp_mode_id is equal to 1, the ICC profile is obtained from a URI with syntax and semantics as specified in IETF Internet Standard 66.

All other values of iccp_mode_id are reserved.

iccp_data_payload_byte shall be a byte containing data having syntax and semantics as specified by any of the ISO standards that specify an ICC profile.

iccp_data_URI shall contain a URI with syntax and semantics as specified in IETF Internet Standard 66 identifying the ICC profile.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).