| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0174-v0 |
| --- | --- |

| Title: | [AHG9] Spatial extrapolation as a new NNPF purpose and some bug fixes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang 8910 University Center Ln, San Diego, CA92122, USA | Email: | xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

The following changes are proposed:

- A new bit of nnpfc_purpose is used to support a NNPF purpose of spatial extrapolation. Accordingly, the output width and height are adjusted and the preferred display method is indicated.

(The following changes are proposed to address some asserted issues in the semantics of nnpfc_num_metadata_extension_bits and nnpfc_reserved_metadata_extension):

- Specifies the following: When nnpfc_num_metadata_extension_bits is greater than 0, let numSpecifiedMetadataExtensionBits be the number of bits representing all syntax elements between nnpfc_num_metadata_extension_bits and nnpfc_reserved_metadata_extension. nnpfc_num_metadata_extension_bits greater than 0 specifies the sum of numSpecifiedMetadataExtensionBits and the length, in bits, of nnpfc_reserved_metadata_extension.

- Specifies the following: The value of nnpfc_num_metadata_extension_bits shall be in the range of numSpecifiedMetadataExtensionBits to 2 048, inclusive. Values in the range of numSpecifiedMetadataExtensionBits + 1 to 2 048, inclusive, for nnpfc_num_metadata_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of nnpfc_num_metadata_extension_bits in the range of numSpecifiedMetadataExtensionBits + 1 to 2 048, inclusive.

- In the NNPFC SEI message syntax, a closing curly bracket, '}', is added immediately after the syntax element nnpfc_reserved_metadata_extension.

- The specification for the length, in bit, of nnpfc_reserved_metadata_extension syntax element is simplified as follows: When present, the length, in bits, of nnpfc_reserved_metadata_extension is equal to nnpfc_num_metadata_extension_bits − numSpecifiedMetadataExtensionBits.

# Spatial extrapolation

At the 33rd JVET meeting, AI image expander was proposed to be used as an NNPF. Figure 1 shows an example of AI image expander, which is a kind of spatial extrapolation.

Figure 1. Illustration of AI image expander. The blue area denotes the original image. The image is expanded horizontally with newly generated content.

It was suggested by experts that one reasonable extension is to consider spatial extrapolation as a new NNPF purpose, similar as temporal extrapolation.

To support spatial extrapolation as a new NNPF purpose, similar as temporal extrapolation, a new bit of nnpfc_purpose is used. When the bit is set, left offset, right offset, top offset and bottom offset related to the cropped decode picture are signalled to indicate the newly generated area. The output picture resolution is modified accordingly. Spatial extrapolation can also be combined with resolution resampling, which means that image resizing and expansion happen simultaneously. The preferred scan type is signalled after image expansion.

# Some issues related to NNPFC metadata extension

The following issues related to NNPFC metadata extension have been observed:

- The semantics of nnpfc_num_metadata_extension_bits is incorrect, because per the semantics in VSEI v3, all bits of the syntax elements between nnpfc_num_metadata_extension_bits and nnpfc_reserved_metadata_extension also need to be taken into consideration, but that is not the case. Therefore, the current semantics is not backwards compatible.

- The value nnpfc_num_metadata_extension_bits is allowed to be greater than 2048; this is another aspect that is not backwards compatible with VSEI v3.

- In the NNPFC SEI message syntax, a closing curly bracket, '}' is missing immediately after the syntax element nnpfc_reserved_metadata_extension.

- In the semantics of nnpfc_reserved_metadata_extension, for specifying the length of the syntax element in bits, the phrase "Length of (nnpfc_application_purpose_tag_uri) – 1" is used. This aspect needs an editorial improvement. Furthermore, the length specification is somewhat complicated (with three conditional branches and in each branch the condition is not simple) and not scalable (i.e., it will become more and more complicated while more specified extension data elements are added).

To resolve the above issues, the changes 2 to 5 summarized in the abstract are proposed.

# Proposed text changes

Most relevant parts that have been added or modified are highlighted in turquoise, and some of the deleted parts are highlighted in yellow strikethrough in red fonts. There may be some other changes that are editorial in nature and thus not highlighted. The text changes are based on JVET-AG2034.

8.28.2.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( ResolutionResamplingFlag ) { |  |
| nnpfc_pic_width_num_minus1 | ue(v) |
| nnpfc_pic_width_denom_minus1 | ue(v) |
| nnpfc_pic_height_num_minus1 | ue(v) |
| nnpfc_pic_height_denom_minus1 | ue(v) |
| } |  |
| ... |  |
| nnpfc_num_metadata_extension_bits | ue(v) |
| if( nnpfc_num_metadata_extension_bits > 0 ) { |  |
| if( nnpfc_purpose = = 0 ) { |  |
| nnpfc_application_purpose_tag_uri_present_flag | u(1) |
| if( nnpfc_application_purpose_tag_uri_present_flag ) |  |
| nnpfc_application_purpose_tag_uri | st(v) |
| } |  |
| if( SpatialExtrapolationFlag ) { |  |
| nnpfc_scan_type_idc | u(2) |
| nnpfc_spatial_extrapolation_left_offset | ue(v) |
| nnpfc_spatial_extrapolation_right_offset | ue(v) |
| nnpfc_spatial_extrapolation_top_offset | ue(v) |
| nnpfc_spatial_extrapolation_bottom_offset | ue(v) |
| } |  |
| nnpfc_reserved_metadata_extension | u(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

8.28.2.2 Neural-network post-filter characteristics SEI message semantics

...

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20, where ( nnpfc_purpose & bitMask ) not equal to 0 indicates that the NNPF has the purpose associated with the bitMask value in Table 20. When nnpfc_purpose is greater than 0 and ( nnpfc_purpose & bitMask ) is equal to 0, the purpose associated with the bitMask value is not applicable to the NNPF. When nnpfc_pupose is equal to 0, the NNPF may be used as determined by the application and as specified by the nnpfc_application_purpose_tag_uri.

All NNPFC SEI messages with a particular value of nnpfc_id within a CLVS shall have the same value of nnpfc_purpose.

The value of nnpfc_purpose shall be in the range of 0 to 127255, inclusive, in bitstreams conforming to this version of this Specification. Values of 128256 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 128256 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| bitMask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x40 | Temporal extrapolation (i.e., generating one or more future pictures) |
| 0x80 | Spatial extrapolation (i.e., generating content outside of the spatial area of the input pictures) |

The variables ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, ColourizationFlag, and TemporalExtrapolationFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, colourization, and temporal extrapolation, respectively, are derived as follows:

ChromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
ResolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
PictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(75)
BitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
ColourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
TemporalExtrapolationFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : 0
SpatialExtrapolationFlag = ( ( nnpfc_purpose & 0x80 ) > 0 ) ? 1 : 0

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value or any one of a set of values including that value.

When ChromaFormatIdc is equal to 3, ChromaUpsamplingFlag shall be equal to 0.

When ChromaUpsamplingFlag is equal to 1, ColourizationFlag shall be equal to 0.

When PictureRateUpsamplingFlag or TemporalExtrapolationFlag is equal to 1 and the input picture with index 0 is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, all input pictures are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

When TemporalExtrapolationFlag is equal to 1, the extrapolated pictures generated by the NNPF follow all input pictures of the NNPF in output order. When TemporalExtrapolationFlag is equal to 1 and there is a decoded output picture that follows, in output order, the current picture for which the NNPF is activated, the extrapolated pictures generated by the NNPF precede that decoded output picture in output order.

...

nnpfc_pic_width_num_minus1 plus 1 and nnpfc_pic_width_denom_minus1 plus 1 specify the numerator and denominator, respectively, for the resampling ratio of the width of the NNPF-generated pictures relative to CroppedWidth. Both nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 shall be in the range of 0 to 65 535, inclusive.

The value of ( nnpfc_pic_width_num_minus1 + 1 ) ÷ ( nnpfc_pic_width_denom_minus1 + 1 ) shall be in the range of 1 ÷ 16 to 16, inclusive. When nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 are not present, the values of nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 are both inferred to be equal to 0.

The variable nnpfcOutputPicWidth, representing the width of the luma sample arrays of the NNPF-generated pictures, is derived as follows:

nnpfcOutputPicWidth = Ceil( CroppedWidth *		(77)
		( nnpfc_pic_width_num_minus1 + 1 ) ÷ ( nnpfc_pic_width_denom_minus1 + 1 ) )

When SpatialExtrapolation is equal to 1, nnpfcOutputPicWidth is updated as follows:

nnpfcOutputPicWidth += outSubWidthC * ( nnpfc_spatial_extrapolation_left_offset + nnpfc_spatial_extrapolation_right_offset )

It is a requirement of bitstream conformance that the value of nnpfcOutputPicWidth % outSubWidthC shall be equal to 0.

nnpfc_pic_height_num_minus1 plus 1 and nnpfc_pic_height_denom_minus1 plus 1 specify the numerator and denominator, respectively, for the resampling ratio of the height of the NNPF-generated pictures relative to CroppedHeight. Both nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 shall be in the range of 0 to 65 535, inclusive.

The value of ( nnpfc_pic_height_num_minus1 + 1 ) ÷ ( nnpfc_pic_height_denom_minus1 + 1 ) shall be in the range of 1 ÷ 16 to 16, inclusive. When nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 are not present, the values of nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 are both inferred to be equal to 0.

The variable nnpfcOutputPicHeight, representing the height of the luma sample arrays of the NNPF-generated pictures, is derived as follows:

nnpfcOutputPicHeight = Ceil( CroppedHeight *		(78)
		( nnpfc_pic_height_num_minus1 + 1 ) ÷ ( nnpfc_pic_height_denom_minus1 + 1 ) )

When SpatialExtrapolation is equal to 1, nnpfcOutputPicHeight is updated as follows:

nnpfcOutputPicHeight += outSubWidthC * ( nnpfc_spatial_extrapolation_top_offset + nnpfc_spatial_extrapolation_bottom_offset )

It is a requirement of bitstream conformance that the value of nnpfcOutputPicHeight % outSubHeightC shall be equal to 0.

When ResolutionResamplingFlag is equal to 1, at least one the following conditions shall be true:

–	The value of nnpfcOutputPicWidth is not equal to CroppedWidth.

–	The value of nnpfcOutputPicHeight is not equal to CroppedHeight.

–	SpatialExtrapolationFlag is equal to 1.

...

nnpfc_num_metadata_extension_bits equal to 0 specifies that nnpfc_reserved_metadata_extension is not present. When nnpfc_num_metadata_extension_bits is greater than 0, let the variable numSpecifiedMetadataExtensionBits be the number of bits representing all syntax elements between nnpfc_num_metadata_extension_bits and nnpfc_reserved_metadata_extension. nnpfc_num_metadata_extension_bits greater than 0 specifies the sum of numSpecifiedMetadataExtensionBits and the length, in bits, of nnpfc_reserved_metadata_extension.

The value of nnpfc_num_metadata_extension_bits shall be in the range of numSpecifiedMetadataExtensionBits 0 to 2 048, inclusive in this edition of this document when nnpfc_purpose is not equal to 0 and in the range of 1 to 2048 when nnpfc_prupose is equal to 0. Values in the range of numSpecifiedMetadataExtensionBits + 1 to 2 048 2049 to 4096, inclusive, for nnpfc_num_metadata_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of nnpfc_num_metadata_extension_bits in the range of numSpecifiedMetadataExtensionBits + 1 to 2 048 0 to 4096, inclusive.

nnpfc_application_purpose_tag_uri_present_flag equal to 1 indicates that the nnpfc_application_purpose_tag_uri syntax element is present in this NNPFC SEI message. nnpfc_application_purpose_tag_uri_present_flag equal to 0 indicates that the nnpfc_application_purpose_tag_uri syntax element is not present in this NNPFC SEI message. When not present nnpfc_application_purpose_tag_uri_present_flag is inferred to be equal to 0.

nnpfc_application_purpose_tag_uri specifies a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the application determined purpose of the NNPF, when nnpfc_purpose is equal to 0.

NOTE 4 – nnpfc_application_purpose_tag_uri enables uniquely identifying the application determined purpose of NNPF without needing a central registration authority.

nnpfc_scan_type_idc equal to 0 indicates that the preferred display method for the pictures output by the NNPF is unknown or unspecified or specified by external means. nnpfc_scan_type_idc equal to 1 indicates that the pictures output by the NNPF are suitable for display using overscan. nnpfc_scan_type_idc equal to 2 indicates that the pictures output by the NNPF contain visually important information in the entire region out to the edges of the picture, such that the pictures output by the NNPF should not be displayed using overscan. Instead, they should be displayed using either an exact match between the display area and the edges, or using underscan. As used in this paragraph, the term "overscan" refers to display processes in which some parts near the borders of the pictures are not visible in the display area. The term "underscan" describes display processes in which the entire pictures are visible in the display area, but they do not cover the entire display area. For display processes that neither use overscan nor underscan, the display area exactly matches the area of the pictures. The value of nnpfc_scan_type_idc shall not be equal to 2. When not present, the value of nnpfc_scan_type_idc is inferred to be equal to 0.

nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset specify the spatial extrapolation area. The luma samples with horizontal picture coordinates from outSubWidthC * nnpfc_spatial_extrapolation_left_offset to nnpfcOutputPicWidth − ( outSubWidthC * nnpfc_spatial_extrapolation_right_offset ) and vertical picture coordinates from outSubWidthC * nnpfc_spatial_extrapolation_top_offset to nnpfcOutputPicWidth − ( outSubWidthC * nnpfc_spatial_extrapolation_bottom_offset ) correspond to the spatial area of the input picture. The value of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset and nnpfc_spatial_extrapolation_bottom_offset shall be in the range of 0 to 65536, inclusive. At least one of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset and nnpfc_spatial_extrapolation_bottom_offset shall be greater than 0.

nnpfc_reserved_metadata_extension shall not be present in bitstreams conforming to this version of this Specification. However, decoders conforming to this version of this Specification shall ignore the presence and value of nnpfc_reserved_metadata_extension. When present, and when nnpfc_purpose is equal to 0 and nnpfc_application_purpose_tag_uri_present_flag is equal to 1 the length, in bits, of nnpfc_reserved_metadata_extension is equal to nnpfc_num_metadata_extension_bits – Length of (nnpfc_application_purpose_tag_uri) – 1. When present and when nnpfc_purpose is equal to 0 and nnpfc_application_purpose_tag_uri_present_flag is equal to 0 the length, in bits, of nnpfc_reserved_metadata_extension is equal to nnpfc_num_metadata_extension_bits – 1 bits. When present, and when nnpfc_purpose is not equal to 0 the length, in bits, of nnpfc_reserved_metadata_extension is equal to nnpfc_num_metadata_extension_bits − numSpecifiedMetadataExtensionBits.

nnpfc_alignment_zero_bit_b shall be equal to 0.

nnpfc_payload_byte[ i ] contains the i-th byte of a bitstream conforming to ISO/IEC 15938-17. The byte sequence nnpfc_payload_byte[ i ] for all present values of i shall be a complete bitstream that conforms to ISO/IEC 15938-17.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).