| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0175-v0 |
| --- | --- |

| Title: | [AHG9] On constraints and reference of MatrixCoefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang 8910 University Center Ln, San Diego, CA92122, USA | Email: | xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

The following 5 items are discussed in this contribution regarding MatrixCoefficients in CICP and its reference:

Item 1: it is asserted that constraints of referencing MatrixCoefficient in CICP are missing for the syntax element fg_matrix_coeffs in the VSEI text when its value is equal to 0 or 8.

Item 2: when CICP 3rd edition is applied, new constraints on referencing YCgCo-Re and YCgCo-Ro should be added.

Item 3: when there is no constraints on luma bit depth value, a non-positive RGB bit depth may be derived.

Item 4: left shifting by a negative integer number, which is undefined, may be applied in some equations to interpret MatrixCoefficients.

Item 5: chroma sample value in YCgCo may be out of range in the current specification.

# Problem statement and solution

Item 1:

In a film grain characteristics SEI message, the luma and chroma bit depths of the film grain model may be signalled. So does fg_matrix_coeffs, which specifies the matrix coefficients of the film grain characteristics. It is asserted that for some values of fg_matrix_coeffs shall be disallowed according to the luma and chroma bit depths of the film grain model, similar to the cases of vui_matrix_coeffs and nnpfc_matrix_coeffs. Specifically, the following two constraints shall be added:

- matrix_coeffs shall not be equal to 0 unless the luma bit depth is equal to the chroma bit depth.

- matrix_coeffs shall not be equal to 8 unless the luma bit depth is equal to the chroma bit depth, or the luma bit depth plus 1 is equal to the chroma bit depth.

Item 2:

In CICP 3rd edition, two new MatrixCoefficients identifiers, YCgCo-Re (value 16) and YCgCo-Ro (value 17) are added. For YCgCo-Re and YCgCo-Ro, they have the same equations as YCgCo-R (value 8 and BitDepthC is equal to BitDepthY + 1). Similar constraints for YCgCo-R shall be applied to YCgCo-Re and YCgCo-Ro. Specially, the following constraints shall be added for all syntax elements in VSEI text that referencing MatrixCoefficients, which include vui_matrix_coeffs in VUI parameters, fg_matrix_coeffs in the film grain characteristics SEI message, colour_transform_matrix_coefficients in the colour transform information SEI message, nnpfc_matrix_coeffs in the neural-network post-filter characteristics SEI message:

For YCgCo-Re:

MatrixCoefficients shall not be equal to 16 unless both of the following conditions are true:

–	the luma bit depth is equal to the chroma bit depth,

–	the signal format is equal to 3 (the 4:4:4 chroma format).

For YCgCo-Ro:

MatrixCoefficients shall not be equal to 17 unless both of the following conditions are true:

–	the luma bit depth is equal to the chroma bit depth,

–	the signal format is equal to 3 (the 4:4:4 chroma format).

Considering that those constraints for all syntax elements with the value being 0, 8, 16 and 17 are essentially the same, the other solution is to move all MatrixCoefficeints constraints to the CICP text, which is more reasonable.

Item 3:

When MatrixCoefficients is equal to 16 or 17, the RGB bit depth is equal to the luma bit depth minus an offset, which is 1 or 2. Thus, when the luma bit depth is less than or equal to 2, the RGB bit depth may be non-positive. We suggest restricting luma bit depth to avoid such a case.

Item 4:

In the equations to derive R, G, B or Y, Cb, Cr values, a left shifting, i.e. 1 << ( BitDepth − 8 ), may be undefined. One solution is to restrict that all bit depth values shall be greater than or equal to 8, which is the case for the setting in various video coding standards. However, considering that for RGB image or video, bit depth less than 8 is still widely used, e.g. in BMP file format [1] and Microsoft DirectShow [2], RGB555(5-bit for R, G and B channels) images and videos are supported, only specifying case for RGB bit depth >= 8 may limit the CICP usage.

The other solution is to implement the 1 << ( BitDepth − 8 ) by ( 1 << BitDepth ) ÷ ( 1 << 8 ) , which is a pure editorial change when BitDepth >= 8 and make the equations valid for all BitDepth values.

Item 5:

In the current CICP text, if MatrixCoefficients is equal to 8 and BitDepthC is equal to BitDepthY, the following operations are applied:

Cb = Round( 0.5 * G − 0.25 * ( R + B ) ) + (1 << ( BitDepthC − 1 ) )

Cr = Round( 0.5 * (R − B ) ) + (1 << ( BitDepthC − 1 ) )

Let's asumme that BitDepthY is equal to 8, which means that BitDepthRGB is also equal to 8. In the above operations, for example, when G value is 255, R and B values are both 0, Cb is derived to be 256, which is out of the 8-bit range. A clipping operation shall be applied to Cb. Cr is of the same situation.

# Proposed text changes

Most relevant parts that have been added or modified are highlighted in turquoise, and some of the deleted parts are highlighted in yellow strikethrough in red fonts. There may be some other changes that are editorial in nature and thus not highlighted.

Item 1:

The following changes are based the latest VSEI text:

8.5.2 Film grain characteristics SEI message semantics

...

fg_matrix_coeffs has the same semantics as specified in subclause 7.3 for the vui_matrix_coeffs syntax element, except as follows:

—	fg_matrix_coeffs specifies the matrix coefficients of the film grain characteristics specified in the SEI message, rather than the matrix coefficients used for the CLVS.

—	When fg_matrix_coeffs is not present in the film grain characteristics SEI message, the value of fg_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

—	The values allowed for fg_matrix_coeffs are not constrained by the chroma format of the decoded video pictures that is indicated by the value of ChromaFormatIdc for the semantics of the VUI parameters.

fg_matrix_coeffs shall not be equal to 0 unless fg_bit_depth_luma_minus8 is equal to fg_bit_depth_chroma_minus8.

fg_matrix_coeffs shall not be equal to 8 unless one of the following conditions is true:

–	fg_bit_depth_luma_minus8 is equal to fg_bit_depth_chroma_minus8.

–	fg_bit_depth_chroma_minus8 is equal to fg_bit_depth_luma_minus8 + 1.

...

Item 2 (solution 2):

The following changes are based the latest CICP text:

8.3 MatrixCoefficients

Type: Unsigned integer, enumeration
Range: 0–255, plus associated flag

MatrixCoefficients describes the matrix coefficients used in deriving luma and chroma signals from the green, blue and red or X, Y and Z primaries, as specified in Table 4 and equations 11 to 87.

The value of MatrixCoefficients shall not be equal to 0, 16 or 17 unless both the following conditions are true:

–	BitDepthY is equal to BitDepthC.

–	the picture chroma format is 4:4:4.

The specification of the use of MatrixCoefficients with the value equal to 0, 16 or 17 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

The value of MatrixCoefficients shall not be equal to 8 unless one of the following conditions are true:

–	BitDepthY is equal to BitDepthC.

–	BitDepthC is equal to BitDepthY + 1 and the picture chroma format is 4:4:4.

The specification of the use of MatrixCoefficients with the value equal to 8 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

Item 3:

8.3 MatrixCoefficients

...

The specification of the use of BitDepthY with the value less than 3 is reserved for future use by ITU-T | ISO/IEC.

...

Item 4 (solution 2):

8.3 MatrixCoefficients

...

The interpretation of MatrixCoefficients is specified as follows.

| — | If VideoFullRangeFlag is equal to 0, the following applies: |  |
| --- | --- | --- |
|  | — | If MatrixCoefficients is equal to 0, 8, 16, or 17, pseudocode operations (27) to (29) apply: |

R = Clip3( 0, MaxValRGB, ( 1 << ( BitDepthRGB − 8 ) ) ÷ ( 1 << 8 ) * ( 219 * E′R + 16 ) )	(27)

G = Clip3( 0, MaxValRGB, ( 1 << ( BitDepthRGB − 8 ) ) ÷ ( 1 << 8 ) * ( 219 * E′G + 16 ) )	(28)

B = Clip3( 0, MaxValRGB, ( 1 << ( BitDepthRGB − 8 ) ) ÷ ( 1 << 8 ) * ( 219 * E′B + 16 ) )	(29)

|  | — | Otherwise, if MatrixCoefficients is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, 13, 14, or 15, pseudocode operations (30) to (32) apply: |
| --- | --- | --- |

Y = Clip1Y ( Round( ( 1 << (BitDepthY − 8 ) ) ÷ ( 1 << 8 ) * ( 219 * E′Y + 16 ) ) )	(30)

Cb = Clip1C ( Round( ( 1 << (BitDepthC − 8 ) ) ÷ ( 1 << 8 ) * ( 224 * E′PB + 128 ) ) )	(31)

Cr = Clip1C ( Round( ( 1 << (BitDepthC − 8 ) ) ÷ ( 1 << 8 ) * ( 224 * E′PR + 128 ) ) )	(32)

Item 5:

8.3 MatrixCoefficients

...

| — | Otherwise, if MatrixCoefficients is equal to 8 and BitDepthC is equal to BitDepthY, pseudocode operations (51) to (53) apply: |
| --- | --- |

Y = Round( 0.5 * G + 0.25 * ( R + B ) )		(51)

Cb = Clip1C( Round( 0.5 * G − 0.25 * ( R + B ) ) + (1 << ( BitDepthC − 1 ) ) )	(52)

Cr = Clip1C( Round( 0.5 * (R − B ) ) + (1 << ( BitDepthC − 1 ) ) )	(53)

# References

[1] https://en.wikipedia.org/wiki/BMP_file_format

[2] https://learn.microsoft.com/en-us/windows/win32/directshow/uncompressed-rgb-video-subtypes

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).