| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0176-v1 |
| --- | --- |

| Title: | EE2-6.1: Context initialization for B-slice |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Yan Ye Jie Chen Xinwei Li | Tel: Email: | {ruling.lrl, yan.ye, jiechen.cj, sid.lxw}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 Test 6.1 which studies adding a new set of context model parameters for B slices. A fourth set of context model parameters is added for low delay B slices in addition to the existing three sets of context model parameters for B-/P-/I-slice types already supported in ECM. The context model of B slices can be initialized using the new set or the original set of context model parameters according to whether the low delay condition is satisfied. It is reported that on top of ECM-12.0, the overall coding performance for {Y, U, V, EncT, DecT} is:

- Switching of context initialization for B-slice at slice level:

{-0.01%, 0.06%, 0.02%, 100.1%, 99.9%} for RA and {-0.06%, -0.23%, -0.29%, 100.0%, 100.2%} for LB.

- Switching of context initialization for B-slice at SPS level:

{0.00%, 0.00%, 0.00%, 100.2%, 99.9%} for RA and {-0.06%, -0.23%, -0.29%, 99.6%, 99.4%} for LB.

# Introduction

In the EE2 test 6.1, it is proposed to add a new set of context model parameters for B-slices. The initial probabilities, window sizes, adaptive weights and two adjustment weights in this new set are trained using the bitstream generated by ECM-12.0 under low delay B condition. Two tests are evaluated. In the first test, each B slice initializes its context model according to the low delay condition. The new set of context model is used for low delay B slices, whereas the original set of context model is used for non-low delay B slices. In the second test, an SPS flag is signaled to indicate whether the original set or the new set of context model parameters are used to initialize the context model of B slices. At the encoder, the SPS flag is configurated according to the random access or low delay B configurations.

# Simulation results

The proposed methods are implemented on top of ECM-12.0 and are tested under CTC for enhanced compression tool testing. Table 1 and Table 2 show the simulation results of switching of context initialization for B-slice at slice level or SPS level, respectively.

Table 1. Simulation results of switching of context initialization for B-slice at slice level

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.01% | -0.01% | 100.3% | 100.4% | 99.9% | 99.7% |
| Class A2 | 0.00% | 0.00% | 0.04% | 100.3% | 100.4% | 99.6% | 100.0% |
| Class B | -0.02% | 0.16% | 0.03% | 100.0% | 99.5% | 99.0% | 99.8% |
| Class C | 0.00% | 0.01% | 0.04% | 100.0% | 99.7% | 100.8% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | 0.06% | 0.02% | 100.1% | 99.9% | 99.8% | 99.9% |
| Class D | 0.04% | -0.22% | -0.04% | 99.5% | 98.7% | 99.8% | 100.0% |
| Class F | 0.00% | 0.06% | 0.02% | 99.8% | 98.6% | 99.1% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.07% | 0.04% | -0.38% | 99.4% | 99.6% | 100.0% | 98.8% |
| Class C | 0.03% | -0.28% | 0.04% | 100.6% | 100.1% | 99.0% | 99.8% |
| Class E | -0.16% | -0.60% | -0.57% | 100.2% | 101.5% | 98.9% | 100.0% |
| Overall | -0.06% | -0.23% | -0.29% | 100.0% | 100.2% | 99.4% | 99.4% |
| Class D | -0.12% | -0.18% | -0.78% | 101.5% | 100.5% | 100.0% | 100.0% |
| Class F | 0.11% | -0.27% | -0.70% | 99.5% | 98.8% | 99.4% | 100.4% |

Table 2. Simulation results of switching of context initialization for B-slice at SPS level

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | 0.00% | 100.5% | 100.2% | 100.2% | 100.1% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100.2% | 100.2% | 99.5% | 100.1% |
| Class B | 0.00% | 0.00% | 0.00% | 100.0% | 99.6% | 99.3% | 99.9% |
| Class C | 0.00% | 0.00% | 0.00% | 100.1% | 99.7% | 100.4% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100.2% | 99.9% | 99.8% | 100.0% |
| Class D | 0.00% | 0.00% | 0.00% | 99.8% | 98.8% | 99.8% | 100.0% |
| Class F | 0.00% | 0.00% | 0.00% | 99.7% | 97.6% | 99.8% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.07% | 0.04% | -0.38% | 99.6% | 98.9% | 99.8% | 98.8% |
| Class C | 0.03% | -0.28% | 0.04% | 99.9% | 99.5% | 99.0% | 99.8% |
| Class E | -0.16% | -0.60% | -0.57% | 99.3% | 100.4% | 100.0% | 100.0% |
| Overall | -0.06% | -0.23% | -0.29% | 99.6% | 99.4% | 99.6% | 99.4% |
| Class D | -0.12% | -0.18% | -0.78% | 101.4% | 101.2% | 100.2% | 100.0% |
| Class F | 0.11% | -0.27% | -0.70% | 99.9% | 98.6% | 100.5% | 100.4% |

# Conclusion

The contribution reports the results of adding a new set of context model parameters for B slices to differentiate between low-delay B and non-low delay B slices. Two tests are evaluated. Switching context model initialization at slice level achieves 0.01% and 0.06% gain for RA and LB, respectively. Switching context model initialization at SPS level has no impact on RA and achieves 0.06% gain for LB. There is no impact on encoder and decoder runtime. It is recommended to adopt one of the proposed tests into the next version of ECM.

# Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).