| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0178-v1 |
| --- | --- |

| Title: | Non-EE2: High Accuracy Sample Based Bi-Directional Optical Flow |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar, Yuwen He, Kai Zhang, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Email: | mehdi.salehifar@bytedance.com yuwen.he@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

High accuracy sample adjustment for sample based Bi-Directional Optical Flow (BDOF) is proposed.

In this method, high accuracy parameter derivation, which is used for decoder side motion vector refinement (DMVR), i.e. BDOF DMVR, is reused to derive accurate sample adjustment.

Simulation results on top of the ECM-12.0 are reported as:

RA:  -0.11%, -0.09%, -0.09%, EncT: 101.5%, DecT: 101.2%

# Introduction

A high-precision equation to derive the BDOF MV refinement parameters was proposed in JVET-AC0182 [1] and adopted into ECM as JVET-AD0195 [2] based on high accuracy formula.

Later an iterative BDOF was proposed in JVET-AD0196 and JVET-AD0176 [3-4], then it was adopted into ECM as JVET-AE0065 [5], adding another BDOF-based motion refinement pass as the 4th pass of the multi-pass DMVR.

Finally, improvements for DMVR were proposed in JVET-AF0160 [6] and adopted into ECM as JVET-AG0067 [7], using the mean removed equations to derive the BDOF DMVR parameters as well as adding the 16×16 BDOF DMVR layer.

# Proposed method

In this contribution, it is proposed to reuse the high accuracy formula used for BDOF DMVR for sample based BDOF too. Specifically:

(Gx.Gx+R1) * vx + Gx.Gy * vy = dI . Gx - dM . Gx

Gx.Gy * vx + (Gy.Gy+R1) * vy = dI . Gy - dM . Gy

where Gx/Gy are the summation of the two horizontal/vertical gradients derived for each reference block.

Summations () are weighted sums.

# Experimental results

The proposed method has been implemented on top of the ECM-12.0 [8]. Testing results under RA configuration is summarized in Tables I. The simulations were performed following the common test conditions [9]. More detailed results can be found in the attached spreadsheet.

Table I:Performance of the proposed method

# Conclusions

In this contribution, improvement for sample based BDOF is proposed. With a negligible impact on encoder and decoder complexity, it provides a reasonable coding gain. It is recommended to study this new method in EE2.

# References

[1] M. Salehifar, Y. He, K. Zhang, L. Zhang, “Non-EE2: High-Precision MV Refinement for BDOF,” JVET-AC0182, Jan. 2023.

[2] M. Salehifar, Y. He, K. Zhang, L. Zhang, “EE2-2.6: High-Precision MV Refinement for BDOF,” JVET-AD0195, Apr. 2023.

[3] M. Salehifar, Y. He, K. Zhang, H. Liu, L. Zhang, “EE2-related: On Iterative BDOF and EE2-2.6 Improvement” JVET-AD0196, Apr. 2023.

[4] J. Chen, R. -L. Liao, X. Li, Y. Ye, “Non-EE2: Improvements on multi-pass DMVR” JVET-AD0176, Apr. 2023.

[5] M. Salehifar, Y. He, K. Zhang, H. Liu, L. Zhang, J. Chen, R.-L. Liao, X. Li, Y. Ye, “EE2-3.5: Iterative BDOF pass in multi-pass DMVR”, JVET-AE0065, July 2023.

[6] M. Salehifar, Y. He, K. Zhang, L. Zhang, “Non-EE2: On DMVR Extensions,” JVET-AF0160, Oct. 2023.

[7] M. Salehifar, Y. He, K. Zhang, L. Zhang, “EE2-2.8: On DMVR Extensions,” JVET-AG0067, Jan. 2024.

[8] ECM-12.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0

[9] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).